EuropeAid/129522/C/SER/Multi Contract number 2010/232-231

Air Quality Governance in the ENPI East Countries

Capacity building programme for decision makers 18 June 2014

| Version |  | Date | Description | Prepared by | Reviewed by |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 18 June 2014 | Report for Task 1.6.6 Capacity building for decision makers, including guidelines, publications, drafting of legal acts, trainings, etc. | Short-term experts of Component 2 | Aiga Kala, Key Expert 2 – Air Quality Assessment and Management Vladimir Morozov, Key Expert 3 – Industry/IPPC |  |

SUMMARY

Project Title:	Air Quality Governance in ENPI East Countries

Contract Number:	2010/232-231

Countries:	Armenia, Azerbaijan, Belarus, Georgia, the Republic of Moldova, the Russian Federation, and Ukraine

Lead Contractor

Name	MWH

Address	Nysdam Office Park
Avenue Reine Astrid, 92
B-1310 La Hulpe
Belgium

Tel. number	+32 2 655 2230

Fax number	+32 2 655 2280

Contact person	Aïda Yassine, Project Manager

Signature

Date of report:	18 June 2014

Experts:

Michael Begak, International Non-Key IPPC Expert

Monika Přibylová, International Non-Key IPPC Expert

Oleg Bely, Local Non-Key IPPC Expert, Belarus

Viktar Khodzin, Local Non-Key IPPC Expert, Belarus

Tatiana Guseva, Local Non-Key IPPC Expert, the Russian Federation

Dmitry Laznenko, Local Non-Key IPPC Expert, Ukraine

Content

List of abbreviations	5

Background	6

1	Status Review	7

1.1	Legal and regulatory analysis for partner countries	7

1.2	Example of Belarus	8

1.3	Environmental regulation of non-Annex I installations in EU	8

2	Needs and recommendations for improvement of environmental permitting procedures and authorizations	10

3	Improvement of emission factors database	12

4	Identification of economic activities/installations for development/improvement of ELVs and other conditions setting for non-Annex I installations	13

5	Development of recommendations for ELVs and other conditions setting for selected installations	21

5.1	Recommendations for petrol storage and handling	21

5.2	Recommendations for small combustion installations	22

6	Overall conclusions and recommendations for policy makers	24

6.1	Overall regulatory tools framework	24

6.2	Overall conclusions and recommendations for policy makers	25

Annex 1 – Air pollution regulation for small businesses in Belarus	27

Annex 2 – Tool for facility register	47

Annex 3 – Content of official collection of emission factors in Ukraine	49

Annex 4 - Approaches for ELVs establishing for the small combustion installations	54

- List of abbreviations

BAT – Best Available Techniques

BREF – BAT Reference document

ELV – Emission Limit Value

ENPI – The European Neighbourhood and Partnership Instrument
that supports the European Neighbourhood Policy (ENP)

EU – European Union

IED – Industrial Emissions Directive 2010/75/EU

IPPC – Integrated Pollution Prevention and Control

SMEs – small and medium enterprises

- Background

This document is prepared in the frame of Air Quality Governance project under its Component 1, namely Task 1.6. Development of a mechanism for accounting for local conditions and technological capacity in the course of setting emission limit values, Subtask 1.6.6 Capacity building for decision makers, including guidelines, publications, drafting of legal acts, trainings, etc. (Output O 1.6 e Capacity building programme for decision makers).

It aims to support a pending introduction of integrated permitting in the ENPI-East region when dealing with small and medium size enterprises (SMEs) for their better environmental performance with less administrative load and providing guidelines for effective regulation of installations with smaller capacity.

Considering that SMEs are normally defined by number of employees and turnover while this report deals with environmental pollution, in fact it is more exactly about low and medium polluting installations and activities. Some of them are anyway regulated through integrated permitting in case being auxiliary for operation of so called Annex I installations as defined by Industrial Emissions Directive 2010/75/EU (IED).

Due to great number of such small businesses and their common presence in residential areas their general impact should be considered equally important to regulation of major polluters.

Fulfilment of the Subtask 1.6.6 is based on outputs of previous activities, presented in earlier reports O 1.6 b Review of best EU practices in regulation of non-Annex I installations based on GBRs, O 1.6 c List of non-Annex I installations for drafting of recommendations, and O 1.6 d Recommendations for ELVs and other conditions setting for selected activities, and for harmonisation of legislation for the registration and permitting systems.

Beside other, proposed solutions were specially discussed at workshops of IPPC and Permitting training programme in Kiev, Ukraine in October 2013 and in Batumi, Georgia in April 2014. It helped to receive a necessary feedback and define priority measures for related reforming of environmental permitting.

As it was demonstrated in the Legal and regulatory analysis for the project partner countries (Subtask 2.1.2.2) capacity building in the area requires significant efforts already in initial assessment of environmental impact of small businesses and improvement of tools first of all for air emissions inventory.

Best European practices and positive practical examples of the ENPI-East countries serve as a reliable guide for resolute implementation of effective regulation measures described in the presented paper. Special attention is paid to current situation in Belarus where ongoing reform stipulates full scale introduction of integrated environmental permits before 2016.

Status Review

Legal and regulatory analysis for partner countries

Regulation of non-Annex I installations was of special interest at the Legal and regulatory analysis for the project partner countries (Subtask 2.1.2.2). Country review is available in the corresponding report, below is an extract from its summary table with related questions. For benchmark comparison this table is provided with additional column for Czech Republic.

| Armenia | Azerbaijan | Belarus | Georgia | Republic of Moldova | Russian Federation | Ukraine | Czech Republic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| What methodology/criteria for identification of low polluting installations are proposed? |  |  |  |  |  |  |  |
| Criteria for air emissions | Criteria for air emissions | Criteria for emissions | Criteria for air emissions, fuel and electricity use | Criteria for air emissions to define 4 categories of installations | Criteria for emissions to define 4 categories of installations | Criteria for emissions | Three categories for air pollution sources. Similarly conditions for operators that need waste permit or water permit. |
| Are there technically based ELVs for economic sectors/installations? |  |  |  |  |  |  |  |
| No | No | Very limited (for some boilers, gas-turbine units, waste incineration) | No | No | No | Yes National ELVs for dozens of air pollutants and sectoral ELVs for 8 economic activities | Yes. Except numerous BAT recommendations in BREFs sectoral ELVs for LCP etc |
| Are there other environmental GBRs for economic sectors/installations? |  |  |  |  |  |  |  |
| No | No | No | No | No | No | No | Yes. |
| Are there additional to ELVs operational conditions, compliance with technological parameters applicable for permits? |  |  |  |  |  |  |  |
| No | No | Yes As exception | No | No | No | Yes As exception | Yes Usual widespread practice |
| Are national EFs (emission factors) for installations/sources of emissions specified? |  |  |  |  |  |  |  |
| No | No (GHG only) | No | Yes From 2003 | No | No (GHG only) | Yes From 2004 | Yes For such activities as farms, petrol stations, etc. |
| Is there official list of calculation methods for estimation of emissions? |  |  |  |  |  |  |  |
| No | No | Yes From 2004 | Yes From 2003 | Yes Approved in 2000 | No | No | Yes |
| Applied reference methods/instruments for emissions inventory and control. Is there official list for them? |  |  |  |  |  |  |  |
| No | Yes Require updating | Yes From 2004 | Yes From 2003 | No | No | Yes Require updating | Yes |
| Do petrol stations require air permit, with dispersion modelling? |  |  |  |  |  |  |  |
| Yes | No | Yes | No | Yes | Yes | Yes | Yes They are middle source of air pollution |

Table 1. Specific issues of regulation of non-Annex I installations in partner countries and the Czech Republic

Table 1 demonstrates principal gaps that are typical for all seven project countries, absence of reliable tools for assessment and regulation of emissions of industrial facilities, utilities and services. Both major polluters and small businesses require practical sectoral approach with consideration of their specific features.

Detailed review of situation for each country of the project is presented in consolidated report for Subtasks 2.1.2.2 Legal and regulatory analysis and 2.2.2.1 Overview of the IPPC/BAT elements in existing legal system of the partner countries.

Example of Belarus

Without doubt Belarus can be considered in the ENPI-East region as a leader of environmental regulation providing practical example for its neighbours. Its air pollution regulation for small businesses is described in Annex 1.

Beside other it defines ELVs and order of their development depending of type of air pollution source and their use in permitting procedures. Air permit is required for plants with air emissions more than 0.1 tonne per year or more than 0.1 kg per year of most hazardous substances of 1 class defined by a national classification.

There are special lists of 242 priority substances for development of ELVs for all sources of air pollution and 66 economic activities without significant impact on air quality exempted from ELVs development.

Environmental regulation of non-Annex I installations in EU

The overview of good EU practice in environmental regulation of non-Annex I installations and examples of non-Annex I installations specifications for drafting of recommendations for countries participating in the Air-Q-Gov project was done within Subtasks 1.6.2 Review of practice of ELVs and other conditions setting in EU member countries for non-Annex I installations, related to Directive 2010/75/EU and 1.6.3 Identification of economic activities/installations for development/improvement of ELVs and other conditions setting for non-Annex I installations, with corresponding outputs O 1.6 b Review of best EU practices in regulation of non-Annex I installations based on GBRs and O 1.6 c List of non-Annex I installations for drafting of recommendations.

It is commonly acknowledged that 60–70% of the environmental impact originates from SMEs. Though there is little evidence on the actual impact from SMEs, and it would be too complex and burdensome for companies and public authorities to determine the detailed contribution made by the 20 million SMEs to pollution (e.g. air pollution) in terms of the environmental load from different types of pollutants (e.g. SOx, NOx, VOC, etc.).

Table below illustrates the variations across countries where the impact from SMEs can be as low as 51% in some countries, such as in the United Kingdom (with a relatively low share of persons employed in SMEs), or above 70% or even 80%, such as in Cyprus (with a relatively large share of persons employed in SMEs) where the estimated impact share is 84%.

The table shows the calculated average of employees in SMEs per country across sectors. Across sectors the variations can be even bigger.

The average environmental impact from SMEs in the EU27 – per country (equals the average share of employees in the SME size companies per country).

Source: Structural Business Statistics of Eurostat and estimations for the EU27 aggregated at level 3 data

Needs and recommendations for improvement of environmental permitting procedures and authorizations

Simplification of the permission procedures for SMEs already for decades remains an insistent recommendation of numerous technical assistance projects in the region. Unfortunately financial interests are dominating in permitting process and provoke excessive demands both from governmental bodies and consultants. Even with valid legal recommendations to use simplified approach any exceptional option may become a common rule if it is supported by extra fees.

So, specific sectoral guidelines may be also a solution not only for grounded technical requirements but serve as a firewall for uncontrolled regulation. Area of responsibility of any SME should be clear-cut and concrete.

It starts from inventory issues and goes to monitoring obligation, where for example imposing of requirements of direct measurements shall generally not be a question at all.

Comprehensive sectoral guidelines will enhance staff competences both in permitting units and in SMEs, it helps to organise necessary exchanges of information.

In many cases SMEs find it more difficult to comply with environmental legislation than their large counterparts. Lack of capacity in SMEs, which means that they seldom have an overview of sources of legal and technical support, and how these can be used.

Typically SMEs are incurring unnecessary significant extra costs to hire external environmental consultants.

Major recommendations of OECD for small businesses are as follows:

- Regulatory simplification

- Sectoral approach to promotional activities

- Adaptation of tools to SME needs

- Using business arguments and business partners

- Packages of information-based tools

- Importance of market signals

- Need for initial public funding

- Development of performance indicators.

Creation of one-stop shops for the provision of information and tools for the fulfilment of administrative requirements by SMEs, streamlining of their environmental reporting is nowadays greatly encouraged by existing network technologies. In fact they require only a small fraction of IT services that their own accountant normally uses.

Advanced efforts to create so say ‘electronic government’ services also for environmental regulation are now in several countries of the region, for example in the Republic of Moldova.  Special permitting web-pages are under development at official site of the Ministry of Natural Resources and Environmental Protection of the Republic of Belarus. The current introductory website for environmental permitting is presented on the following picture.

Application of market or financial signals

The practical impact of using financial tools for increasing environmental performance of SME can be seen in the UK, where the regulator (the Environment agency) imposes yearly fee for large and medium polluting installations. The fee is bases on the environmental performance and potential environmental risk of the regulated operator. The fee is lower for those operators which operate in compliance with the valid permit or GBRs, does not use significant amounts of hazardous substance and has good history from the environmental performance’ point of view.

Although this approach is not directly applicable in the project countries, the cost, benefits and incentives of new regulatory tools such as simplified permitting procedure or one-stop shops hall be always considered prior it implementation.

Improvement of emission factors database

As a next in the step capacity building Belarus side proposed to develop recommendations for simplified assessment of air emissions less than 25 tonnes per year. Corresponding terms of reference are in Annex 2.

Such document should be used for other project countries too. Ukrainian experience is of special interest when dealing with emission factors database. Ukraine introduced such national database in 2004, with updates it’s valid till now.

It was prepared as a compilation in one document of reliable calculation methods and sectoral emission factors with consideration of EMEP/CORINAIR Atmospheric emission inventory guidebook (which is called now EMEP/EEA air pollutant emission inventory guidebook).

Table of content of official Ukrainian collection of emission factors is presented in Annex 3. It comprises a vast area of economic sectors from the heavy industry to food processing and auxiliary operations like welding. Most of data in fact relates to all partner countries though for example coke production chapter is interesting for Ukraine and Russia with their highly developed iron making, and subchapter for open hearth furnaces is nowadays is only Ukrainian matter of concern as that is probably only country in the world where this obsolete technology is still widely used.

Another typical problem of the region is absence of realistic prioritisation of methods of air emissions assessment and ungrounded requirements for obligatory direct measurements.

It is necessary to legalise widespread use of sources of emission factors and their priority list, as it is for example in practice of Latvia.

In the Latvian Cabinet of Ministers Regulations No. 182, Adopted on 2 April 2013 "Provisions for development of emission limits project for fixed pollution sources" it is said "10. ... emissions factors, used for the quantitative determination of the emissions, may be derived from:

10.1. the instrumental measurements at identical or similar emission source;

10.2. the calculations using the material balance sheet;

10.3. from the European Environment Agency air pollutant emission inventory guidebook CORINAIR (Tier 3) or, if it is not available, from the United States Environmental Protection Agency Compilation of Air Pollutant Emission Factors AP-42. If the European Environment Agency or the United States Environmental Protection Agency emission factors are not available for the polluting activity, emission factors derived from other emission factors database (methodology) shall be used."

Certainly for countries like Ukraine with officially developed national databases, they will top the list.

Identification of economic activities/installations for development/improvement of ELVs and other conditions setting
for non-Annex I installations

The assessment of environmental regulation of non-Annex I installations in EU has resulted into a conclusion that the practice of regulation of non-Annex I installations most capable of being transferred to the partner countries’ system is the framework currently in force in the Republic of Latvia. Yet, taking into account that also Czech and UK systems have valuable elements, it is suggested to develop a framework that would be mainly based on Latvia’s experience, but will be tailored to the needs of partner countries at issue.

Identification of economic activities/installations for development/improvement of ELVs and other conditions setting for non-Annex I installations was elaborated under Subtask 1.6.3 with its output O 1.6 c List of non-Annex I installations for drafting of recommendations.

The general framework of proposed regulation for the partner countries is derived from the Latvian example, which is highly suitable firstly due to the similarities in the background history, which has a considerable impact on industrial activities carried out in the countries.

It is proposed to use a two-layered system for regulation of non-Annex I activities. Its first group encompasses installations and activities which, though not included in Annex I of the IED, still can have a considerable negative impact on the environment, therefore require stricter, permit-based regulation. The activities and installation at issue are provided in Table 2 below.

Sector and type of activity

1. Energy industry:

1.1. combustion installations with rated thermal input:

1.1.1. from 5 to 50 megawatts, if biomass (also wood and peat) or gaseous fuels are used in the combustion installation

1.1.2. from 0.5 to 50 megawatts, if liquid fuels are used in the combustion installation, except fuel oil (heavy fuel oil)

1.1.3. from 5 to 50 megawatts, if liquid fuel or fuel oil, is utilised in the combustion installation that is used in a grain dryer

1.1.4. from 0.2 to 50 megawatts, if coal is utilised in the combustion installation

1.2. combustion installations in which fuel oil (heavy fuel oil) is utilised

1.3. oil depots and terminals with 5,000 or more tonnes of fuel per year (the largest total amount of fuel pumped per year during the last three years)

1.4. petrol stations with 2,000 or more cubic metres of fuel per year (the total largest amount of fuel pumped during the last three years)

1.5. gas storage installations with a capacity of 100 m3 or more and underground storage sites of natural gas

1.6. coal and brown-coal briquetting equipment

1.7. the production of charcoal

2. Production and processing of metals:

2.1. installations for the production of pig iron or steel, also for continuous casting, with a capacity not exceeding 2.5 tonnes per hour

2.2. installations for the processing of ferrous metals;

2.2.1. hot-rolling mills which process less than 20 tonnes of crude steel per hour

2.2.2. installations for the application of protective fused metal coats, which treat less than 2 tonnes of crude steel per hour

2.3. ferrous metal foundries with a production capacity of up to 20 tonnes per day

2.4. installations for smelting, also fusion, of non-ferrous metals, including metals to be used for recycling, the melting capacity of which does not exceed four tonnes of molten lead or cadmium per day or 20 tonnes of other metals per day, except installations that are used in crafts and sculpture, including for the processing of gold and silver

2.5. installations in which electrolysis or chemical processes are used for surface treatment of metals and plastic materials and the total volume of the treatment vats of which does not exceed 30 m3

2.6. installations for surface treatment during the operation of which dust is created, including the polishing of iron, steel or other metallic objects, cleaning by sand blasting and powder painting, if the total discharge of the installation is 10,000 or more cubic metres per hour

2.7. floating docks and dry docks of a steel shipyard

2.8. other installations for industrial processing of iron, steel or other metals with a production area of 1,000 m2 or more

2.9. installations for the production of cables

2.10. installations for the production of accumulators and batteries

2.11. electro-technical equipment for the production of transformer and printed circuits

3. Production of mineral products:

3.1. installations for the production of cement clinker in rotary kilns the production capacity of which does not exceed 500 tonnes per day or installations for the production of lime in rotary kilns with a production capacity that does not exceed 50 tonnes per day, or in other furnaces with a production capacity that does not exceed 50 tonnes per day

3.2. installations for the manufacture of glass, including glass fibre, with a melting capacity that does not exceed 20 tonnes per day, except craftsmanship

3.3. installations for melting mineral substances, including the production of mineral wool, with a melting capacity that does not exceed 20 tonnes per day

3.4. installations for the manufacture of ceramic products by firing, including roofing tiles, bricks, refractory bricks, tiles, stove tiles, pottery, faience or porcelain, in which up to 75 tonnes of finished products may be manufactured per day, except for craftsmanship

3.5. cement production units with a production capacity of 20,000 or more tonnes per year and installations for the production of concrete and concrete products with a capacity of 20,000 or more cubic metres per year

3.6. installations for the production of plaster products, except craftsmanship

4. Chemical industry and activities with chemical substances and chemical products:

4.1. installations for the production of organic and inorganic substances, products or intermediary products, including enzymes, in which physical production processes (for example, dilution and mixing) are utilised

4.2. installations for the storage of unpacked organic or inorganic chemical substances, chemical products or intermediary products, if one tonne or more is stored, for the storage of enzymes – 20 tonnes or more

4.3. installations for the production of pharmaceutical products, in which physical processes (for example, dilution and mixing) are utilised

4.4. installations for the production of explosives, in which physical production processes (for example, mixing) are utilised

4.5. installations for the production of munitions

4.6. installations for industrial production of colorants, additives and ancillary substances (also usable in food industry), in which physical processes are utilised (for example, dilution and mixing), except retail trade

4.7. installations for the production of chemical substances and chemical products and also for the production of plant protection products and biocides using physical methods (for example, dilution and mixing), packing and filling

4.8. installations for the production of soaps, detergents and cleaning agents with a production capacity of one tonne or more tonnes per year

4.9. installations for the production of paints, varnishes or glue

4.10. installations for the production of goods with teflon thermo-coating, thermoplastic materials moulded by extrusion or by performing recycling of fibrous thermoplastic composite materials, if 100 or more kilograms of plastic are used per day

4.11. installations for the production of plastic goods, using injection moulding from alloy, the extrusion process, including calendering or thermal moulding, if five or more tonnes of plastic are used per day Installations for the production of plastic goods from expanded polystyrene, if five or more tonnes of plastic are used per day

4.12. installations for the production of goods of rubber with a production capacity above 500 tonnes per year

4.13. installations for the production of regenerated pulp

4.14. installations for the production of gelatine and glue from the skin and bones of animals

4.15. installations for the production of organic chemical products via chemical, biological or physical process (that are not subject to regulation in accordance with Annex I of IED

4.16. installations for the production of asphalt and road surfacing materials

4.17. installations for the production of roof covering, using tar and bitumen

4.18. installations for the distillation of tar

4.19. gas and coke plants

4.20. weaveries, spinneries and knitwear production units, if the production capacity is 100 or more kilograms per day

4.21. dry-cleaners

4.22. laundries with a capacity exceeding 1,000 kilograms per day

5. Waste management:

5.1. installations for the disposal or processing of hazardous waste, including petroleum product waste, the capacity of which does not exceed 10 tonnes per day

5.2. installations for the incineration or co-incineration of municipal waste and other waste that may not be classified as hazardous waste, if the capacity of the installation does not exceed three tonnes per hour

5.3. installations for the incineration or co-incineration of hazardous waste, the capacity of which does not exceed 10 tonnes per day

5.4. installations for biological or physico-chemical treatment of municipal waste, the capacity of which does not exceed 50 tonnes per day, except composting installations with an intake capacity not exceeding 100 tonnes per year and composting installations for animal manure

5.5. installations for the treatment of municipal waste for purposes of disposal in which the biological or physico-chemical treatment method is not utilised

5.6. landfills for the processing of municipal waste with capacity not exceeding 75 tonnes per day

5.7. landfills that can receive up to 10 tonnes of waste per day or with a total capacity not exceeding 25,000 tonnes, excluding landfills of inert waste

5.8. landfills of inert waste

5.9. places for the disposal, storage or composting of wastewater sludge and waste that may not be classified as hazardous waste in accordance with legislation

5.10. installations for the processing of discarded vehicles with capacity not exceeding 75 tonnes per day and for the recycling and storage of ship wrecks

5.11. installations for the sorting or temporary storage of municipal waste, including reloading stations with a receiving capacity of 30 or more tonnes of waste per day

5.12. installations for the storage of, recycling or treatment of waste of animal origin, also installations for composting and bio-gas installations with a receiving capacity of waste of animal or vegetable origin, including animal droppings and waste from slaughterhouses, of 30 or more tonnes per day

5.13. installations for the storage of hazardous waste (including at the places of creation) for more than one year

5.14. installations for temporary (not more than one year) storage of hazardous waste with a total capacity not exceeding 50 tonnes, for example, reloading stations and container warehouses, excluding storage of waste for such a short period of time or in such an insignificant amount that the waste does not cause a risk to human health or the environment

5.15. installations for the recycling of electric and electronic waste with capacity not exceeding 75 tonnes per day

6. Agriculture, forestry and wood processing:

6.1. slaughterhouses with a carcass production capacity from 5 to 50 tonnes per day

6.2. installations for the disposal or recycling of animal carcasses and waste of animal origin, the capacity of which is from 1 to 10 tonnes per day

6.3. slaughterhouses with a production capacity of products of poultry origin of 5,000 or more tonnes per year

6.4. the production of matches

6.5. the production of oriented plywood panels, plywood panels or fibre plywood panels (separate types of panels or different types of panels together) with a production capacity not exceeding 600 m3 per day

6.6. the production of furniture, if the production area is 1,000 m2 or more

7. Food industry:

7.1. installations for the collection, pre-treatment and processing of milk, in which the quantity of milk received is from 10 to 200 tonnes per day (average value on an annual basis)

7.2. installations for the production of food products, in which products of animal origin (other than milk) are treated and processed and which produce from 1 to 75 tonnes of finished product per day, or which treat and process vegetable products and produce from 10 to 300 tonnes of finished product per day (average value on a quarterly basis), including:

7.2.1. the production of oils and fats of vegetable and animal origin

7.2.2. the production of beer and malt

7.2.3. the production and bottling of non-alcoholic beverages

7.2.4. installations for industrial production of starch and potato starch

7.2.5. fish meal and fish oil production units

7.2.6. sugar production units

7.2.7. the production of coffee, tea and food additives

7.2.8. grain processing

7.2.9. the production of yeast

7.2.10. the production and bottling of alcohol and alcoholic beverages

7.2.11. the conservation, filling and packaging of products of animal and vegetable origin

7.2.12. other food product production installations in which vegetables are treated and processed

7.2.13. installations for the production of fish and crustacean products, including for the production of canned, smoked and frozen products

7.3. facilities for the production of meat meal, including bone meal, blood meal, blood plasma and feather meal production units

7.4. the production of protein and pectin

7.5. installations for the production of tobacco products

8. Other sectors:

8.1. in manufacturing:

8.1.1. installations for the production of paper and cardboard with a production capacity not exceeding 20 tonnes per day

8.1.2. installations for the pre-treatment of fibres and fabric (washing, bleaching, mercerisation) or dyeing, the treatment capacity of which is from 0.5 to 10 tonnes per day

8.1.3. installations for the tanning of hides and skins, in which less than 12 tonnes of finished products are produced per day

8.1.4. volatile organic compounds emitting installations listed in Annex I, part 1, where the use of organic solvents in the installation emitting volatile organic compounds exceeds thresholds prescribed in Annex I, part 2

8.2. crematoria

8.3. airports and airfields

8.4. railway depot and stations that perform the functions of a freight station, marshalling station or district station

8.5. berths of ports for the loading and unloading of cargoes into ships with gross tonnage not less than 450 tonnes

8.6. hospitals with the number of beds above 100

8.7. washing installations intended for the cleaning of storage and transportation receptacles and containers of chemical substances

8.8. waste water treatment plants with capacity of 20 or more cubic metres per day that drain the treated waste water in the environment

Table 2. SME’ activities and installations requiring permit-based regulation

The second group of non-Annex I activities partly consists of activities and installations of the same nature or functions, though falling below the minimum threshold (expressed as capacity, produced units etc.) set for the activities requiring permit-based regulation (as proposed in the table 2 above). In addition, the group also encompasses other activities and installations that have a potential of having a negative impact on the environment, though not as unequivocal as in the case of proposed permit-regulated activities. The second group of activities and installations is proposed to be regulated by general binding regulations (GBRs).

As noted by OECD’s Environment Policy Committee, the following principal criteria are commonly applied when the use of GBRs or other rules-based regimes is considered for a segment of the regulated community:

- Rules must cover a sufficiently large number of regulated entities in a particular sector to make this regulatory regime effective

- The state of technology and techniques in that sector must not be fast moving, as rules cannot be updated frequently; and

- The facilities must have a similar, low-risk environmental impact.

While the simplified permitting procedure always involves a formal application from the operator, which is approved by the regulatory authority (although authority in many countries increasingly use straightforward online application forms tailored to individual sectors), rules-based regimes may or may not require notification of, or registration with, the competent authority.

Sector and type of activity

1. Power industry:

1.1. combustion installations with rated thermal input of more than 0.2 megawatts (if a permit is not required for the combustion installation in accordance with regulations on Annex I activities or non-Annex I activities requiring permit-based regulation)

1.2. wind power stations or power station parks with the total capacity of more than 125 kilowatts

1.3. petrol stations with fuel amount of up to 2000 m3 per year (the total largest amount of fuel pumped during the last three years)

1.4. gas filling stations

1.5. oil depots with fuel amount of less than 5000 tonnes per year

1.6. installations for the production of heating fuel from the remains of timber

1.7. installations for the production of heating fuel from peat

2. Production and processing of metals:

2.1. installations for surface treatment that create dust during operation, including the polishing of iron, steel or other metallic objects, sand blasting (cleaning by sand blasting) and powder painting, if the total emission of the installation is from 300 to 10,000 m3/hour

2.2. other installations for the processing of iron, steel or other metals with a production area from 100 m2 to 1,000 m2

2.3. foundries usable in craftsmanship, also for the casting of gold and silver

2.4. production facilities for electro-technical products, except installations for the production of transformers or printed circuits

2.5. bonding of plastic products

3. Manufacturing of mineral products (processing of mineral substances):

3.1. cement production units with a production capacity from 2 to 20,000 tonnes per year and installations for the production of concrete and concrete products with a capacity from 2 to 20,000 cubic metres per year

3.2. installations for the production and mixing of gravel or lime mortar and installations for the crushing of stones, which are not installed at the places where the stones are obtained

3.3. stationary installations for the production of aerated concrete, coal dust or lime-and-sand bricks

4. Agriculture, forestry and wood processing:

4.1. animal housings in which 10 or more animal units are bred for commercial purposes (including the storage, collection and drainage of solid manure, liquid manure, slurry and silage juice); animal housings are located in a highly sensitive territory and in which five or more animal units are bred for commercial purposes (if the animal housing does not require permit in accordance with regulations on Annex I activities)

4.2. sawmills and wood-processing installations in which timber-cutting machinery is utilised and which process 2,000 m3 or more round wood and timber per year; installations in which industrial chemical treatment of timber is performed, also pressure impregnation (high-pressure impregnation), vacuum impregnation (low-pressure impregnation) and protection of timber against blue stain and mould

4.3. fish farms

5. Food industry:

5.1. installations for the collection, pre-treatment and processing of milk, in which the quantity of milk received is from one to ten tonnes per day (average value on an annual basis)

5.2. installations for the production of food, in which products of animal origin (excluding milk) are processed and which produce from 0.1 to 1 tonne of finished products per day, and in which products of vegetable origin are processed and from 0.5 to 10 tonnes of finished products are produced per day (average value on a quarterly basis)

5.3. installations for the production of fish and crustacean products, including for the production of canned, smoked and frozen products, in which less than one tonne of finished products is produced per day

5.4. smoke-houses, meat and gastronomy production units (also in shops) in which 500 or more kilograms of food products are produced per day

5.5. installations for the baking of bread and industrial production of confectionery products with a production capacity that exceeds two tonnes per day

5.6. slaughterhouses with a production capacity of carcasses less than five tonnes per day

6. Other sectors

6.1. repair and maintenance shops for mechanical land vehicles of all categories (L, M, N, O), mobile agricultural machinery and mobile non-road machinery, and other movable aggregates, where the following activities are performed: - diagnostics, maintenance and repair of motor; - maintenance and repair of power system; - installation, diagnostics and repair of electric devices and alarm systems; - diagnostics, maintenance and repair of transmission and elements; - diagnostics, maintenance and repair of suspension and steering equipment; - diagnostics, maintenance and repair of brake system; - assembly, adjustment and repair of tires and wheels; - body diagnostics, geometry reconstruction and repair; - anti-corrosion treatment of body; - preparation for painting and painting; - car wash and body maintenance.

6.2. chemical and biological laboratories (except study laboratories)

6.3. waste water treatment plants with a capacity from 5 to 20 cubic metres per day, if waste water is drained in the environment

6.4. installations and photographic laboratories in which 1000 or more square metres of photographic films are processed per year

6.5. installations for the storage of salt and salt mixtures, if one tonne or more of salt or salt mixtures is stored, installations for obtaining salt and salt mixtures (with a capacity of one tonne or more per day) for scattering on roads during winter conditions

6.6. installations for the production of casement windows and doors

6.7. installations for the storage of packed organic and inorganic chemical substances, chemical products or intermediary products, if more than 10 tonnes of chemical substances, chemical products or intermediary products are stored

Table 3. Activities and installations requiring GBRs

Development of recommendations for ELVs and other conditions setting for selected installations

To support practical implementation of new approaches to regulation of small businesses in the partner countries there were developed recommendations for ELVs, other conditions setting for selected installations.

They were developed in the framework of Sub-task 1.6.4 for two previously chosen activities:

- Petrol storage and petrol handling with emphasis on petrol filling stations ELVs and other permit conditions

- Combustion of fuels in installations with a total rated thermal input below 50 MW.

Recommendations for petrol storage and handling

The petrol and diesel fuels contain volatile organic compounds (VOCs) that evaporate in storage tanks. When unloading of petrol to an underground storage tank of the petrol filling station, petrol vapour in the tank will be displaced by the incoming petrol. Unless controlled, the petrol vapour will dissipate into the atmosphere.

The emissions of VOCs during the operations with petrol in EU are regulated by two Directives, namely:

- European Parliament and Council Directive 94/63/EC of 20 December 1994 on the control of volatile organic compound (VOC) emissions resulting from the storage of petrol and its distribution from terminals to service stations

- Directive 2009/126/EC of the European Parliament and of the Council of 21 October 2009 on Stage II petrol vapour recovery during refuelling of motor vehicles at service stations

The petrol filling stations are mainly the sources of emissions into ambient air. The leaks of fuel from the station equipment can cause the pollution of underground and surface waters.

The experience of EU countries, USA and Canada confirms the effectiveness of petrol vapour recovery systems for storages loading (stage I) and motor vehicles refuelling (stage II). Installation of such systems at petrol filling stations with annual petrol throughput 500 m3 or more can be recommended for Project countries as necessary requirement for operation permit granting.

The efficiency of fuel vapour recovery has to be not less than 85%.

For ELV value 35 g/Nm3 as 1 hourly average for total carbon can be recommended.

All other environmental risks have to be estimated and excluded with the help of environmental risk list.

Detailed recommendations are presented in the report for Subtask 1.6.4 Development of recommendations for ELVs and other conditions setting for selected installations.

Recommendations for small combustion installations

Main pollutants related to fossil fuels or biomass combustion are as follows: nitrogen oxides (NO and NO2, represented as NOX), sulphur dioxide (SO2), dust or particulate matter (PM), and carbon monoxide (CO). The key combustion related greenhouse gas is carbon dioxide (CO2). The most important substances and compounds from the point of public health protection are: NOX, SO2 and dust.

Attention to the environmental impact at EU level was caused by high requirements for ambient air quality. Air quality Directive foresees step by step rising of environmental standards for such pollutants as NOX, PM, SO2, and ground level ozone. Current situation analysis has shown that the measures of impact reduction of “big polluters” (covered by IED) and transport are not enough to meet ambient air standards in 2025 and 2030.

This circumstance has drawn environmentalist’s attention to medium and small combustion plants as noticeable sources of ambient air pollution. The shares of medium and small combustion plants in, for example, PM2.5 emissions was as much as 76% in Latvia, 74% in Poland, 57% in Denmark, 38% in Estonia. But for some countries this share is rather small: Germany – 16%, Sweden – 12% and UK – 9%.

So, the general recommendation for Project Countries is at first estimate the environmental impact of medium and small combustion plants and its role in ambient air pollution. If this role will be significant for such pollutants as NOX and PM, then the abatement measures gave to be planned.

It will be reasonable to adopt EU classification of medium and small combustion plants: medium plants are those with a total rated thermal input from 5 MW to 50 MW, small plants are those with a total rated thermal input below 5 MW.

There are four options discussing now at EU level and in Member States regarding the “permitting regime” for medium combustion plants:

- Integrated permit similar to the Industrial Emissions Directive regime (covering air, water, soil, waste, etc.)

- Air emissions permit: permit covering emissions to air of SO2, NOX and PM

- Registration on the basis of notification (no permit)

- General binding rules without permit, notification or registration.

The choice of an option depends on the role of medium combustion plants in overall environmental impact. It can be recommended to set up an integrated permitting regime or air emissions permit for 5 MW to 50 MW plants in densely populated countries or regions, where medium combustion plant emissions plays significant role. For other project countries such installations can operate on the base of registration or general binding rules.

It is recommended annual measurement of ELVs for plants in the 20-50 MWth and triennial measurement of ELVs for plants in the 1 -20 MWth.

For small combustion plants with a total rated thermal input below 1 MW it is recommended to use instruments similar to EU Ecodesign Directive, ecolabelling, and some provisions of amended Gothenburg Protocol.

For wood stoves it is recommended to use North countries ecolabelling experience (Swan label). Stoves labelled with the Nordic Swan must have total emissions below 5 g/kg wood.

The following primary measures to reduce emissions from existing residential combustion stoves and boilers can be recommended:

- Use mass-media advertising for provision of new models of stoves and boilers, paying attention on the level of emissions in connection with fuel economy and energy efficiency and about proper kinds of fuel.

- Establishing a programme to promote the replacement of the oldest existing boilers and stoves by modern appliances.

- Establishing an obligation to exchange or retrofit old appliances.

It is reasonable to set ELVs for dust (PM) only, because the abatement NOX measures for small scales are ineffective, and the reduction of SO2 emissions for small combustion plants it is the question of proper choice of fuel.

Approaches for ELVs establishing for the small combustion installations are presented in Annex 4 to the document.

Detailed recommendations are presented in the report for Subtask 1.6.4 Development of recommendations for ELVs and other conditions setting for selected installations.

Overall conclusions and recommendations for policy makers

Overall regulatory tools framework

In summary, it is evident from the EU experience, experience of partner countries and literature review that a single isolated regulation, regulatory tool or intervention is unlikely to engage all types of SMEs in environmental improvement. More specifically, the experience and review suggests that commitment from various stakeholders (regulators, regulated operators,…) and a coordinated mixture of regulatory tools is required to encourage all types of SMEs to make environmental improvements. Table 4 summarises the key points presented in the “A review of interventions to encourage SMEs to make environmental improvements” written by Dr Craig M. Parker in 2009.

The table shows what features of each intervention/regulatory tool are needed to improve their effectiveness, and which type of SME each intervention is likely to be most effective with. Thus, from this table it is possible to devise a mix of regulatory tools that are likely to be highly effective in engaging a particular type of SME in environmental improvement. It is assumed that SMEs can have the following types of motivations for applying regulatory tools: Environment-driven, Advantage-driven, Compliance-driven, and Profit-driven.

| Regulatory tool | Effective when ... | Ineffective when ... | Most effective for |
| --- | --- | --- | --- |
| Voluntary regulations and standards | - driven by personal ethics - external demand for compliance | - it is the only intervention - no awareness of regulations - barriers are greater than benefits - no demand for compliance | Environment-driven Advantage-driven |
| Compulsory regulations | - it is clearly communicated - provides equity for all firms - combined with financial penalties - financial support is provided | - it is the only intervention - no awareness of regulations - perceived as a business threat - not monitored or enforced | Compliance-driven Advantage-driven |
| Financial penalties | - linked to regulatory framework - makes bad practice unviable | - it is the only intervention - penalty too small to be noticed - no viable penalty possible | Compliance-driven Profit-driven |
| Financial support | - promoted clearly/directly to SMEs - simple to apply for - offset regulation compliance costs | - it is the only intervention - too difficult to apply for - criteria are too restrictive - only a temporary measure | Environment-driven Advantage-driven |
| Self-directed and facilitated education | - linked to regulatory framework - tailored to individual firm needs - it has a specific problem focus - run by trusted/credible parties - promotes a change in attitude - encourages learner interaction - it uses business language - helps firms gauge their progress - helps to identify opportunities - learning is actionable immediately - provided conveniently to firms - includes real examples/cases | - it is the only intervention - providers do not know SMEs - does not address specific needs - providers are not credible/trusted - too many information sources - lack of knowledge integration - it uses sustainability language - it is too expensive to attend | Environment-driven Advantage-driven It is challenging but possible to provide education for: Compliance-driven Profit-driven |

Table 4. A mixed regulatory tools framework to engage all SMEs in environmental improvement

Overall conclusions and recommendations for policy makers

The environmental impact of SMEs is generally believed to be considerable and yet efforts to encourage SMEs to engage in environmental improvement have encountered considerable resistance and scepticism. Research on specific interventions that encourage SME owner-managers to engage in environmental improvement has produced conflicting results concerning their effectiveness. This appears to be due to the fact that SMEs are extremely diverse, operate under widely differing business conditions in terms of perceived pressures and drivers for environmental improvement and therefore adopt quite different business models and levels of environmental commitment. Owner-managers of SMEs also exhibit widely differing views, understanding and aspirations where environmental issues are concerned. Some owner-managers believe that these issues are the responsibility of local and national government, often arguing that their individual company contribution to emissions and pollution is extremely small. It is therefore not surprising that specific, isolated interventions have failed to address the needs of all types of SMEs. This issue is extremely important for policy makers to consider when they are attempting to gain greater environmental commitment and engagement from SMEs. It is also important not to treat SMEs as a homogeneous group when studying their environmental improvement, but instead group them according to attributes such as business performance commitment.

It is possible to argue that by segmenting SMEs by their business and environmental commitment at least four extreme cases can be identified along a continuum and that a wider range of regulatory tools can be used in combination to achieve greater engagement by each of these types of SME in environmental improvement. In addition, a more holistic regulatory tools programme can perhaps be designed if policy makers and regulatory consider industry-specific and other variables on which SMEs can be categorised in conjunction with the regulatory tools framework outlined earlier (see the Table 4 above). The objective is to understand fully the attitudes and environmental commitment of these firms and to understand how a particular combination of regulatory tools can maximize the number of SMEs engaging in environmental improvement.

Segmenting and categorising SMEs in this way is a multi-dimensional problem and that there exist other categories of SMEs related to environmental commitment or performance (e.g. sectoral and capacity classification listed in part 4 of this report). Nevertheless analysis of SME environmental performance combined with a thorough understanding of the applied and possible regulatory tools for SMEs can result in better mix of regulatory tools and thus greater engagement of these SMEs in environmental improvement.

The logical conclusion is that a broader, possibly tailored, range of regulatory tools will elicit greater engagement overall. Nevertheless, it is possible that some SMEs will remain recalcitrant no matter how many kinds of regulatory tools policy makers adopt. However, in the authors’ view this number of SMEs is likely to be much smaller than is currently the case. It is also our view that policy makers need to understand how SMEs engage with or avoid certain regulatory tools when designing a broader policy mix of regulatory framework. This level of understanding can only be achieved by having a clearer view of the types of SMEs being targeted, their business behaviour and the conditions under which these SMEs operate. Further, governments are in a position to markedly alter the business conditions of SMEs and often do so. However, these policy changes can be crude and ineffective at achieving the goals set, particularly where environmental improvement is concerned, and often have unintended consequences. The authors of this report believe that, a greater understanding of the various categories of SMEs and their respective business behaviour and response to regulatory tools is likely to lead to better policy, a more effective and wider range of regulatory tools and result in significantly greater environmental improvement by SMEs.

Thus, the conclusions of this report are that it is no longer possible to rely on the extremes of voluntary environmental agreements or regulation and legislation to engage SMEs in environmental improvement. Regulatory tools strategies to assist SMEs to engage in environmental improvement need to be holistic and designed for the specific category of SME being targeted. It is argued that a properly coordinated and mixed regulatory tools approach is likely to be more successful in engaging SMEs and it is recommended that such an approach should be developed into a practical tool kit for regulatory authorities in project countries.

- Annex 1 – Air pollution regulation for small businesses in Belarus

НОРМИРОВАНИЕ ВЫБРОСОВ ЗАГРЯЗНЯЮЩИХ ВЕЩЕСТВ В АТМОСФЕРНЫЙ ВОЗДУХ МАЛЫМИ ПРЕДПРИЯТИЯМИ

В соответствии с Законом Республики Беларусь «Об охране атмосферного воздуха» от 16.12.2008 г. (далее - Закон) статьей 19 предусмотрено, что:

1. Нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух устанавливаются для стационарных источников выбросов. К нормативам допустимых выбросов загрязняющих веществ в атмосферный воздух относятся:

1.1. предельная масса выброса загрязняющего вещества в атмосферный воздух в единицу времени (тонн в год, граммов в секунду);

1.2. предельное значение концентрации выброса загрязняющего вещества в атмосферный воздух в миллиграммах в нормальном метре кубическом (температура 273 К, давление 101.3 кПа) без поправок на содержание кислорода и влажности, а для газообразных продуктов горения топлива - в пересчете на сухой газ и определенное содержание кислорода.

2. Нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух устанавливаются Министерством природных ресурсов и охраны окружающей среды Республики Беларусь и его территориальными органами в разрешениях на выбросы загрязняющих веществ в атмосферный воздух, выдаваемых юридическим лицам, индивидуальным предпринимателям для каждого объекта воздействия на атмосферный воздух, имеющего стационарные источники выбросов, или в комплексных природоохранных разрешениях, выдаваемых в соответствии с законодательством.

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

3. Нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух разрабатываются юридическими лицами, индивидуальными предпринимателями, осуществляющими хозяйственную и иную деятельность, связанную с выбросами загрязняющих веществ в атмосферный воздух, или уполномоченными ими юридическими лицами:

3.1. для проектируемых стационарных источников выбросов и объектов воздействия на атмосферный воздух - в составе проектной документации;

(в ред. Закона Республики Беларусь от 14.07.2011 г. № 293-З)

3.2. для действующих стационарных источников выбросов и объектов воздействия на атмосферный воздух - в проекте нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух.

4. Нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух разрабатываются и устанавливаются с учетом:

4.1. нормативов качества атмосферного воздуха;

4.2. фоновых концентраций загрязняющих веществ в атмосферном воздухе;

4.3. концентраций загрязняющих веществ в атмосферном воздухе на границе зоны воздействия источников выбросов;

4.4. прогнозов изменения профиля, объёмов производства продукции, вида выполняемых работ, объема оказываемых услуг, используемых сырья и материалов, внедрения наилучших доступных технических методов;

4.5. технологических нормативов выбросов загрязняющих веществ в атмосферный воздух;

4.6. нормативов допустимой антропогенной нагрузки на атмосферный воздух;

4.7. показателей по сокращению выбросов загрязняющих веществ в атмосферный воздух, устанавливаемых в государственных, отраслевых или территориальных программах в области охраны атмосферного воздуха;

4.8. проведения пусконаладочных работ при вводе в эксплуатацию новых стационарных источников выбросов для каждой очереди строительства и выхода на проектную мощность;

4.9. требований, предусмотренных статьями 30 - 32 настоящего Закона.

5. Обоснованность нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух подтверждается актом инвентаризации выбросов загрязняющих веществ в атмосферный воздух, проводимой в соответствии со статьей 38 настоящего Закона.

6. В случае, когда согласно акту инвентаризации выбросов загрязняющих веществ в атмосферный воздух выбросы загрязняющих веществ в атмосферный воздух превышают нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух и (или) технологические нормативы выбросов загрязняющих веществ в атмосферный воздух с учетом применения наилучших доступных технических методов, Министерством природных ресурсов и охраны окружающей среды Республики Беларусь и его территориальными органами в разрешении на выбросы загрязняющих веществ в атмосферный воздух или в комплексном природоохранном разрешении могут устанавливаться временные нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух и условия осуществления выбросов загрязняющих веществ в атмосферный воздух, к которым относятся требования:

(в ред. Закона Республики Беларусь от 12.12.2012 № 6-З)

6.1. по обеспечению соблюдения или достижения установленных нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух, технологических нормативов выбросов загрязняющих веществ в атмосферный воздух и сроков их достижения, определенных с учетом мероприятий государственных, отраслевых или территориальных программ в области охраны атмосферного воздуха;

6.2. к качеству топлива, сырья, материалов и проверке их качества;

6.3. по оснащению организованных стационарных источников выбросов автоматизированными системами контроля за выбросами загрязняющих веществ в атмосферный воздух в случаях, предусмотренных обязательными для соблюдения требованиями технических нормативных правовых актов;

6.4. по обеспечению отбора проб и производства замеров для определения количественного и качественного состава выбросов загрязняющих веществ в атмосферный воздух, локального мониторинга окружающей среды, объектом наблюдения которого являются выбросы загрязняющих веществ в атмосферный воздух, в случаях, предусмотренных актами законодательства об охране окружающей среды, в том числе обязательными для соблюдения требованиями технических нормативных правовых актов;

(в ред. Закона Республики Беларусь от 14.07.2011 г. № 293-З)

6.5. по разработке и проведению мероприятий по сокращению выбросов загрязняющих веществ в атмосферный воздух на период неблагоприятных метеорологических условий.

7. Министерством природных ресурсов и охраны окружающей среды Республики Беларусь определяется порядок установления нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух и утверждаются перечень загрязняющих веществ, категории объектов воздействия на атмосферный воздух, для которых устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, и перечень объектов воздействия на атмосферный воздух, источников выбросов, видов деятельности, для которых такие нормативы не устанавливаются.

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

Для целей, связанных с объемом требований в отношении объектов воздействия на атмосферный воздух при проведении инвентаризации и определении перечня нормируемых видов деятельности, загрязняющих веществ и источников выбросов загрязняющих веществ в атмосферный воздух статьей 12 вышеуказанного закона установлена классификация и цели классификации. Классификация объектов воздействия на атмосферный воздух по категориям осуществляется для:

1.1. государственного регулирования и управления в области охраны атмосферного воздуха;

1.2. нормирования в области охраны атмосферного воздуха;

1.3. учёта в области охраны атмосферного воздуха;

1.4. контроля (надзора) в области охраны атмосферного воздуха.

(в ред. Закона Республики Беларусь от 14.07.2011 г. № 293-З)

2. Объекты воздействия на атмосферный воздух разделяются по категориям на основании:

2.1. количества источников выбросов, находящихся на объекте воздействия на атмосферный воздух;

2.2. количественного и качественного состава выбросов загрязняющих веществ в атмосферный воздух от источников выбросов, находящихся на объекте воздействия на атмосферный воздух;

2.3. размера зоны воздействия;

2.4. вероятности наступления на объекте воздействия на атмосферный воздух событий, имеющих неблагоприятные последствия для качества атмосферного воздуха, определяемой на основании данных об объемах используемых или хранящихся на этом объекте воздействия на атмосферный воздух легко летучих, взрывоопасных, пожароопасных или токсичных веществ.

3. Министерством природных ресурсов и охраны окружающей среды Республики Беларусь устанавливается порядок отнесения объектов воздействия на атмосферный воздух к определенным категориям и утверждаются категории объектов воздействия на атмосферный воздух.

Источники выбросов загрязняющих веществ в атмосферный воздух в соответствии со статьей 13 Закона классифицируются. Классификация источников выбросов осуществляется для:

1.1. государственного регулирования и управления в области охраны атмосферного воздуха;

1.2. нормирования в области охраны атмосферного воздуха;

1.3. выдачи разрешений на выбросы загрязняющих веществ в атмосферный воздух, комплексных природоохранных разрешений;

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

1.4. учета в области охраны атмосферного воздуха;

1.5. инвентаризации выбросов загрязняющих веществ в атмосферный воздух;

1.6. контроля (надзора) в области охраны атмосферного воздуха.

(в ред. Закона Республики Беларусь от 14.07.2011 г. № 293-З)

2. Источники выбросов классифицируются на стационарные, мобильные и нестационарные.

3. Стационарные источники выбросов подразделяются на организованные стационарные источники выбросов и неорганизованные стационарные источники выбросов.

К организованным стационарным источникам выбросов относятся источники выбросов, оборудованные устройствами, посредством которых производится локализация поступления загрязняющих веществ в атмосферный воздух от источников выделения загрязняющих веществ.

К неорганизованным стационарным источникам выбросов относятся источники выбросов, не оборудованные устройствами, посредством которых производится локализация поступления загрязняющих веществ в атмосферный воздух от источников выделения загрязняющих веществ.

4. Неорганизованные стационарные источники выбросов подразделяются на:

4.1. линейные, если загрязняющие вещества поступают в атмосферный воздух от газопроводов;

4.2. площадные, если загрязняющие вещества поступают в атмосферный воздух от рассредоточенных источников выделения загрязняющих веществ, в том числе от сооружений по очистке сточных вод, площадок хранения сыпучих материалов, отвалов горных пород, объектов захоронения отходов, объектов хранения отходов, объектов тяготения мобильных источников выбросов.

5. Мобильные источники выбросов подразделяются на:

5.1. механические транспортные средства (за исключением приводимых в движение электродвигателями);

5.2. железнодорожные транспортные средства (за исключением приводимых в движение электродвигателями);

5.3. воздушные суда;

5.4. морские суда, суда внутреннего плавания, суда смешанного (река - море) плавания, маломерные суда;

5.5. самоходные машины.

6. Механические транспортные средства классифицируются по экологическим классам.

Экологические классы механических транспортных средств и порядок отнесения механических транспортных средств к экологическим классам механических транспортных средств устанавливаются Советом Министров Республики Беларусь.

7. К нестационарным источникам выбросов относятся источники выбросов, не являющиеся стационарными или мобильными источниками выбросов и включенные в утверждаемый Министерством природных ресурсов и охраны окружающей среды Республики Беларусь перечень нестационарных источников выбросов.

В соответствии со статьей 33 Закона.

Разрешения на выбросы загрязняющих веществ в атмосферный воздух:

1. Эксплуатация стационарных источников выбросов юридическими лицами, индивидуальными предпринимателями, осуществляющими хозяйственную и иную деятельность, связанную с выбросами загрязняющих веществ в атмосферный воздух, допускается только при наличии разрешения на выбросы загрязняющих веществ в атмосферный воздух или комплексного природоохранного разрешения, выдаваемых Министерством природных ресурсов и охраны окружающей среды Республики Беларусь или его территориальными органами.

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

2. Выдача разрешения на выбросы загрязняющих веществ в атмосферный воздух, внесение в него изменений и (или) дополнений, приостановление, возобновление, продление срока действия указанного разрешения, прекращение его действия, а также выдача комплексного природоохранного разрешения, внесение в него изменений и (или) дополнений, продление срока, прекращение его действия и выдача дубликата осуществляются в порядке, установленном Советом Министров Республики Беларусь.

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

3. В разрешении на выбросы загрязняющих веществ в атмосферный воздух или в комплексном природоохранном разрешении устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, включая лимиты допустимых выбросов загрязняющих веществ в атмосферный воздух, и условия осуществления выбросов загрязняющих веществ в атмосферный воздух в соответствии со статьей 19 настоящего Закона.

(п. 3 статьи 33 в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

4. Выбросы загрязняющих веществ в атмосферный воздух сверх нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух, установленных в разрешении на выбросы загрязняющих веществ в атмосферный воздух или в комплексном природоохранном разрешении, либо с нарушением указанных в них условий запрещаются.

(в ред. Закона Республики Беларусь от 12.12.2012 г. № 6-З)

Выдача разрешения на выбросы загрязняющих веществ в атмосферный воздух, внесение в него изменений и (или) дополнений, приостановление, возобновление, продление срока действия указанного разрешения, прекращение его действия, а также выдача комплексного природоохранного разрешения, внесение в него изменений и (или) дополнений, продление срока, прекращение его действия и выдача дубликата осуществляются в порядке, установленном постановлением Совета Министров Республики Беларусь от 21.05.2009 г. № 664 (далее – Положение).

В соответствии с пунктом 4 Положения получение разрешения на выбросы необходимо при эксплуатации заявителем стационарных источников выбросов в случаях, если выбросы загрязняющих веществ в атмосферный воздух составляют:

более 0.1 тонны в год независимо от состава выбросов;

менее 0.1 тонны в год, но в их составе присутствуют загрязняющие вещества, отнесенные в установленном Министерством здравоохранения порядке к 1-му классу опасности загрязняющих веществ, в количестве более 0.0001 тонны в год.

Органами выдачи разрешений в соответствии с пунктом 7 положения являются:

для заявителя, эксплуатирующего или вводящего в эксплуатацию объекты воздействия на атмосферный воздух, имеющие стационарные источники выбросов и (или) находящиеся в городе Минске - Минский городской комитет природных ресурсов и охраны окружающей среды;

(в ред. постановления Совмина от 24.12.2009 № 1692)

для заявителя, эксплуатирующего или вводящего в эксплуатацию объекты воздействия на атмосферный воздух, имеющие стационарные источники выбросов, выбросы от которых составляют более 10 тонн загрязняющих веществ в год, а также в случае, если выбросы загрязняющих веществ в атмосферный воздух составляют менее 0.1 тонны в год, но в их составе присутствуют загрязняющие вещества 1-го класса опасности загрязняющих веществ в количестве более 0.0001 тонны в год, за исключением случая, предусмотренного в абзаце втором, настоящего пункта, - областные комитеты природных ресурсов и охраны окружающей среды;

(в ред. постановления Совмина от 24.12.2009 г. № 1692)

для иных заявителей, эксплуатирующих или вводящих в эксплуатацию объекты воздействия на атмосферный воздух, имеющие стационарные источники выбросов, не указанных в абзацах втором и третьем настоящего пункта, а также для производителей сельскохозяйственной продукции, перешедших на уплату единого налога, крестьянских (фермерских) хозяйств, организаций, финансируемых из бюджета, - городские или районные инспекции природных ресурсов и охраны окружающей среды.

(в ред. постановления Совмина от 24.12.2009 г. № 1692).

В соответствии с законодательством об административных процедурах заявитель представляет в орган выдачи разрешения (установлено постановлением Совета Министров Республики Беларусь от 17.02.2012 г. № 156 «Об утверждении единого перечня административных процедур, осуществляемых государственными органами и иными организациями в отношении юридических лиц и индивидуальных предпринимателей, внесении дополнения в постановление Совета Министров Республики Беларусь от 14.03.2009 г. № 193 и признании утратившими силу некоторых постановлений Совета Министров Республики Беларусь"(с изменениями и дополнениями, вступившими в силу с 26.06.2014 г.).

В соответствии с пунктом 6.19 заявитель представляет в орган выдачи разрешений в случае ввода в эксплуатацию законченного строительством объекта заявление и экологический паспорт проекта. Срок действия такого разрешения до 2-х лет. По истечении срока, установленного в разрешении, природопользователь проводит инвентаризацию выбросов загрязняющих веществ в атмосферный воздух и представляет в орган выдачи разрешений проект нормативов допустимых выбросов с приложением акта инвентаризации для получения разрешения на объект(-ы) воздействия IV и V категории воздействия сроком на 10 лет.

При проведении инвентаризации выбросов загрязняющих веществ в атмосферный воздух постановлением Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 23.06.2009 г. № 42 "Об утверждении Инструкции о порядке инвентаризации выбросов загрязняющих веществ в атмосферный воздух и признании утратившими силу постановления Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 28.12.2006 г. № 80 и подпункта 1.2 пункта 1 постановления Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 30.11.2007 г. № 97" для малых объектов при отсутствии существенных изменений (более 10% от существующего уровня) качественных и (или) количественных характеристик выбросов загрязняющих веществ в атмосферный воздух или параметров источников выбросов, подтвержденных результатами производственного и (или) государственного контроля в области охраны атмосферного воздуха, природопользователь переутверждает действующий акт инвентаризации путем замены его титульного листа и дополняет раздел "Введение" соответствующим обоснованием.

Количество выбрасываемых в атмосферный воздух загрязняющих веществ определяется разработчиком на основании данных о расходе топлива, сырья, материалов и времени работы источников выделения загрязняющих веществ и источников выбросов за предшествующий год, подтверждаемых данными, содержащимися в формах учетной документации в области охраны окружающей среды, утвержденных Министерством природных ресурсов и охраны окружающей среды Республики Беларусь.

Для объектов воздействия независимо от категории воздействия объекта при проведении инвентаризации используются:

инструментально-расчетные и расчетные методы для определения выбросов от:

неорганизованных стационарных источников выбросов загрязняющих веществ;

факельных установок;

источников открытого хранения топлива, сырья, веществ и отходов, в том числе прудов-отстойников и накопителей, нефтеловушек, шламо- и хвостохранилищ, золоотвалов, отвалов горных пород, открытых поверхностей испарения;

источников взрывных и погрузочно-разгрузочных работ, маршрутов перемещения сыпучих материалов;

карьеров добычи полезных ископаемых, открытых участков их дробления и рассева на фракции;

оборудования и технологических процессов, расположенных в производственных помещениях, не оснащенных вентиляционными установками;

резервуарных парков, сливно-наливочных железно- и автодорожных эстакад и терминалов речных портов;

источников пылевых выбросов и открытых поверхностей орошаемых или водных объектов, массовый выброс которых существенно зависит от гидрометеорологических показателей;

источников вспомогательных производств, в том числе расположенных на открытом воздухе (передвижные сварочные и окрасочные посты, пилорамы, механическая обработка материалов, нанесение металлопокрытий гальваническим способом и т.д.).

Расчетные методы используются для определения выбросов:

от неработающих, резервных, находящихся на ремонте источников выделения загрязняющих веществ и источников выбросов на основании требований технических нормативных правовых актов;

в случае, если валовые выбросы от источников выделения загрязняющих веществ и источников выбросов природопользователя составляют менее 5 тонн в год с учетом обязательного проведения обследования состояния газоочистных установок инструментальными методами и обязательного подтверждения инструментальными методами соответствия источников выделения загрязняющих веществ и источников выбросов требованиям технических нормативных правовых актов или действующих для Республики Беларусь международных договоров.

Для малых предприятий, как правило, при проведении инвентаризации не учитывается фактор нестационарности выбросов загрязняющих веществ в атмосферный воздух.

Ниже приведен перечень оборудования для которых учет нестационарности выбросов является обязательным:

1. Котельные установки и иные устройства по сжиганию топлива мощностью свыше 1 МВт;

2. Установки термического обезвреживания отходов и хвостовых газов химического и нефтехимического производств;

3. Стекловаренные печи и оборудование для химической полировки хрусталя;

4. Вагранки производительностью от 2.5 т/ч;

5. Электропечи для плавки чугуна и стали производительностью от 1.0 т/ч;

6. Печи цветного литья производительностью от 0.5 т/ч;

7. Установки сушки различных материалов;

8. Выбивные решетки;

9. Установки по изготовлению стержней в горячей оснастке и с использованием холодно-твердеющих смесей;

10. Технологические печи химического и нефтехимического производств;

11. Грануляционные башни карбамида;

12. Скруббера-нейтрализаторы цеха слабой азотной кислоты и аммонийной селитры;

13. Узлы абсорбции-десорбции производства карбамида;

14. Абсорбционные колонны азотной кислоты;

15. Абсорберы циклогексанона и установки очистки газов от оксида углерода производства циклогексанона;

16. Печи и установки сжигания цеха восстановления едкого натра;

17. Регенераторы установок каталитического крекинга;

18. Магний-регенерационные котлы;

19. Аммонизаторы-грануляторы;

20. Нейтрализаторы-грануляторы;

21. Контактные аппараты цеха серной кислоты;

22. Печи пиролиза производства полиэтилена;

23. Абсорберы производства мономеров;

24. Мерсеризаторы и гомогенизаторы химических цехов;

25. Производство углеродных и полимерных материалов.

В соответствии с постановлением Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 23.06.2009 № 43 (ред. от 23.12.2011) "Об утверждении Инструкции о порядке установления нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух и признании утратившими силу некоторых постановлений Министерства природных ресурсов и охраны окружающей среды Республики Беларусь":

Нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух (далее - нормативы выбросов) и временные нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух (далее - временные нормативы выбросов) устанавливаются для:

стационарных источников выбросов и (или) совокупности стационарных источников выбросов, сгруппированных по отдельным цехам и производствам природопользователя (далее - нормируемые источники выбросов), объектов воздействия на атмосферный воздух и (или) совокупности объектов воздействия на атмосферный воздух, имеющих стационарные источники выбросов, при условии пересечения их зон воздействия (далее - нормируемые объекты воздействия);

загрязняющих веществ, включенных в перечень загрязняющих веществ, для которых устанавливаются нормативы допустимых выбросов в атмосферный воздух, согласно приложению 1 к постановлению Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 29.05.2009 г. № 31 "Об утверждении перечня загрязняющих веществ, категорий объектов воздействия на атмосферный воздух, для которых устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, и перечня объектов воздействия на атмосферный воздух, источников выбросов, для которых не устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, и признании утратившим силу постановления Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 28.02.2005 г. № 10" (Национальный реестр правовых актов Республики Беларусь, 2009 г., № 162, 8/21121).

Нормативы выбросов не устанавливаются для:

нестационарных источников выбросов и стационарных источников выбросов, связанных с выбросами загрязняющих веществ в атмосферный воздух от мобильных источников выбросов;

источников выделения загрязняющих веществ и источников выбросов, находящихся на консервации;

мобильных источников выбросов;

объектов воздействия на атмосферный воздух, источников выбросов, включенных в перечень объектов воздействия на атмосферный воздух, источников выбросов, для которых не устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, согласно приложению 2 к постановлению Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 29 мая 2009 г. № 31.

Проекты нормативов допустимых выбросов загрязняющих веществ в атмосферный воздух (далее - проект нормативов) разрабатываются природопользователями в случаях, если выбросы загрязняющих веществ в атмосферный воздух составляют:

более 0.1 тонны в год независимо от состава выбросов;

менее 0.1 тонны в год, но в их составе присутствуют загрязняющие вещества, отнесенные к 1-му классу опасности загрязняющих веществ, в количестве более 0.0001 тонны в год.

Для объектов воздействия относящихся к IV категории со значением относительного показателя опасности объекта воздействия на атмосферный воздух менее 0.1 и к V категории расчет максимальных приземных концентраций загрязняющих веществ или групп суммации, создаваемых стационарными источниками выбросов на границе СЗЗ и в жилой зоне, полученных на основании расчета рассеивания загрязняющих веществ в атмосферном воздухе не проводится.

Как было указано ранее нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух не устанавливаются для веществ и источников выбросов и объектов установленных постановлением Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 29.05.2009 № 31 "Об утверждении перечня загрязняющих веществ, категорий объектов воздействия на атмосферный воздух, для которых устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, и перечня объектов воздействия на атмосферный воздух, источников выбросов, для которых не устанавливаются нормативы допустимых выбросов загрязняющих веществ в атмосферный воздух, и признании утратившим силу постановления Министерства природных ресурсов и охраны окружающей среды Республики Беларусь от 28 февраля 2005 г. № 10"

Постановлением определён перечень загрязняющих веществ для которых устанавливаются нормативы допустимых выбросов независимо от категории воздействия объекта воздействия на атмосферный воздух.

ПЕРЕЧЕНЬ ЗАГРЯЗНЯЮЩИХ ВЕЩЕСТВ, ДЛЯ КОТОРЫХ УСТАНАВЛИВАЮТСЯ НОРМАТИВЫ ДОПУСТИМЫХ ВЫБРОСОВ В АТМОСФЕРНЫЙ ВОЗДУХ

| №№ п/п | Код | Наименование загрязняющего вещества |
| --- | --- | --- |
| 1 | 2 | 3 |
| Металлы и их соединения |  |  |
| 1 | 0101 | Алюминий оксид (в пересчете на алюминий) |
| 2 | 0118 | Титан диоксид |
| 3 | 0130 | Железо и его соединения (в пересчете на железо) (ОБУВ - 70.0; КО - 3)<*> |
| 4 | 0124 | Кадмий и его соединения (в пересчете на кадмий) |
| 5 | 0125 | диКалий карбонат (калий карбонат, поташ) |
| 6 | 0126 | Калий хлорид (калий хлористый) |
| 7 | 0128 | Кальций оксид (известь негашеная) |
| 8 | 0133 | Кобальт и его соединения (в пересчете на кобальт) (ОБУВ - 0.4; КО - 2) <*> |
| 9 | 0138 | Магний оксид |
| 10 | 0140 | Медь и ее соединения (в пересчете на медь) |
| 11 | 0143 | Марганец и его соединения в пересчете на марганец (IV) оксид |
| 12 | 0158 | Натрий и его соединения (в пересчете на натрий) |
| 13 | 0150 | Натрий гидрооксид (натр едкий, сода каустическая) |
| 14 | 0152 | Натрий хлорид (поваренная соль) |
| 15 | 0160 | Никель и его соединения (в пересчете на никель) (ОБУВ - 2.0; КО - 1) <*> |
| 16 | 0168 | Олово и его соединения (в пересчете на олово) |
| 17 | 0183 | Ртуть и ее соединения (в пересчете на ртуть) |
| 18 | 0184 | Свинец и его неорганические соединения (в пересчете на свинец) |
| 19 | 0194 | Сурьма и ее соединения (в пересчете на сурьму) (ОБУВ - 40.0; КО - 3) <*> |
| 20 | 0191 | Таллий карбонат (в пересчете на таллий) |
| 21 | 0282 | Таллий йодид (в пересчете на таллий) |
| 22 | 0203 | Хром (VI) |
| 23 | 0211 | Калий гидросульфат (калий бисульфат, калий сульфат однозамещенный) |
| 24 | 0214 | Кальций гидрооксид |
| 25 | 0228 | 3+ Хрома трехвалентные соединения (в пересчете на Cr ) |
| 26 | 0229 | Цинк и его соединения (в пересчете на цинк) |
| 27 | 0231 | Барий и его соединения (в пересчете на барий) |
| Неметаллы и их соединения |  |  |
| 28 | 0301 | Азот (IV) оксид (азота диоксид) |
| 29 | 0302 | Азотная кислота |
| 30 | 0303 | Аммиак |
| 31 | 0304 | Азот (II) оксид (азота оксид) |
| 32 | 0305 | Аммоний нитрат (аммиачная селитра) |
| 33 | 0351 | диАммоний сульфат |
| 34 | 0308 | Ортоборная кислота (борная кислота) |
| 35 | 0310 | Бор нитрид |
| 36 | 0315 | Фосфин (водород фосфористый) |
| 37 | 0316 | Гидрохлорид (водород хлорид, соляная кислота) |
| 38 | 0317 | Гидроцианид (муравьиной кислоты нитрил, циановодород, синильная кислота) |
| 39 | 0322 | Серная кислота |
| 40 | 0325 | Мышьяк, неорганические соединения (в пересчете на мышьяк) |
| 41 | 0326 | Озон |
| 42 | 0328 | Углерод черный (сажа) |
| 43 | 0329 | Селен диоксид (селен (IV) оксид) (в пересчете на селен) |
| 44 | 0330 | Сера диоксид (ангидрид сернистый, сера (IV) оксид, сернистый газ) |
| 45 | 0331 | Сера элементная |
| 46 | 0333 | Сероводород |
| 47 | 0334 | Сероуглерод |
| 48 | 0337 | Углерод оксид (окись углерода, угарный газ) |
| 49 | 0342 | Фтористые газообразные соединения (в пересчете на фтор) - гидрофторид |
| 50 | 0344 | Фториды неорганические плохо растворимые (алюминия фторид, кальция фторид, натрия гексафторалюминат) |
| 51 | 0348 | Ортофосфорная кислота |
| 52 | 0349 | Хлор |
| Углеводороды предельные |  |  |
| 53 | 0401 | Углеводороды предельные алифатического ряда C1 - C10 |
| 54 | 0410 | Метан |
| 55 | 0418 | Этан |
| 56 | 0417 | Пропан |
| 57 | 0402 | Бутан |
| 58 | 0412 | Изобутан (2-Метилпропан) |
| 59 | 0403 | Гексан |
| 60 | 0405 | Пентан |
| 61 | 2754 | Углеводороды предельные алифатического ряда C11 - C19 |
| Углеводороды непредельные |  |  |
| 62 | 0550 | Углеводороды непредельные алифатического ряда |
| 63 | 0501 | Пентилены (амилены - смесь изомеров) |
| 64 | 0502 | Бут-1-ен (бутилен) |
| 65 | 0503 | Бута-1,3-диен (1,3-бутадиен, дивинил) |
| 66 | 0514 | Изобутилен (2-Метилпропен-1) |
| 67 | 0516 | 2-Метилбута-1,3-диен (изопрен, 2-метилбутадиен-1,3) |
| 68 | 0521 | Пропен (пропилен) |
| 69 | 0526 | Этилен |
| 70 | 0528 | Этин (ацетилен) |
| 71 | 0536 | Метилацетилен (проп-1-ин) |
| 72 | 0551 | Углеводороды алициклические |
| 73 | 0539 | Метилциклогексан |
| 74 | 0408 | Циклогексан |
| Углеводороды ароматические |  |  |
| 75 | 0655 | Углеводороды ароматические |
| 76 | 0602 | Бензол |
| 77 | 0603 | 2-Этенилтолуол (о-винилтолуол) |
| 78 | 0609 | 1,2-Диэтилбензол |
| 79 | 0612 | Изопропилбензол (кумол) |
| 80 | 0616 | Ксилолы (смесь изомеров о-, м-, п-) |
| 81 | 0620 | Винилбензол (стирол) |
| 82 | 0621 | Толуол (метилбензол) |
| 83 | 0623 | 1,3,5-Триметилбензол (мезитилен) |
| 84 | 0626 | 1,2,4-Триметилбензол (псевдокумол) |
| 85 | 0685 | 1,2,3-Триметилбензол (гемилеллитен) |
| 86 | 0624 | Пропилбензол |
| 87 | 0627 | Этилбензол |
| 88 | 0628 | Этилтолуолы (смесь изомеров о-, м-, п-) |
| 89 | 0631 | 1-Метил-4-изопропилбензол (п-цимол) |
| Углеводороды полициклические ароматические |  |  |
| 90 | 0703 | Бенз(а)пирен |
| 91 | 0708 | Нафталин |
| 92 | 0711 | Антрацен |
| 93 | 0716 | Фенантрен |
| 94 | 0727 | Бензо(в)флюоратен |
| 95 | 0728 | Бензо(к)флюоратен |
| 96 | 0729 | Индено(1,2,3-cd)пирен |
| Галогенопроизводные углеводородов |  |  |
| 97 | 0801 | 3-Хлорпроп-1-ен (аллил хлористый) |
| 98 | 0827 | Хлорэтилен (винилхлорид, этиленхлорид, хлорэтен) |
| 99 | 0830 | Гексахлорбензол |
| 100 | 0856 | 1,2-Дихлорэтан (дихлорэтан) |
| 101 | 0858 | Дихлорфторметан |
| 102 | 0859 | Дифторхлорметан |
| 103 | 0869 | Дихлорметан (метиленхлорид, метилен хлористый) |
| 104 | 0882 | Тетрахлорэтилен (перхлорэтилен) |
| 105 | 0898 | Трихлорметан (хлороформ) |
| 106 | 0899 | 1,1,1-Трихлорэтан (метилхлороформ) |
| 107 | 0902 | Трихлорэтилен |
| 108 | 0906 | Тетрахлорметан (углерод тетрахлорид, четыреххлористый углерод) |
| 109 | 0930 | 2-Хлорбута-1,3-диен (бета-хлоропрен) |
| 110 | 0931 | (Хлорметил) оксиран (1-хлор-2,3-эпоксипропан, эпихлоргидрин) |
| Спирты и фенолы |  |  |
| 111 | 1002 | 1,4-Бутандиол (бутиленгликоль) |
| 112 | 1078 | Этан-1,2-диол (гликоль, этиленгликоль) |
| 113 | 1034 | Пропан-1,2-диол (пропиленгликоль) |
| 114 | 1039 | Пентан-1-ол (амиловый спирт) |
| 115 | 1041 | Бензилкарбинол (бензиловый спирт) |
| 116 | 1042 | Бутан-1-ол (бутиловый спирт) |
| 117 | 1046 | 4-Гидрокси-4-метилпентан-2-он (диацетон, диацетоновый спирт) |
| 118 | 1048 | 2-Метилпропан-1-ол (изобутиловый спирт) |
| 119 | 1050 | 2-Этилгексанол (изооктиловый спирт) |
| 120 | 1051 | Пропан-2-ол (изопропиловый спирт) |
| 121 | 1052 | Метанол (метиловый спирт) |
| 122 | 1053 | Октан-1-ол (н-октиловый спирт) |
| 123 | 1054 | Пропан-1-ол (пропиловый спирт) |
| 124 | 1061 | Этанол (этиловый спирт) |
| 125 | 1077 | Циклогексанол |
| 126 | 1069 | Крезол (смесь изомеров о-, м-, п-) (трикрезол) |
| 127 | 1071 | Фенол (гидроксибензол) |
| 128 | 1094 | 2-Изопропил-5-метилфенол (тимол) |
| Простые эфиры |  |  |
| 129 | 1114 | Диметиловый эфир |
| 130 | 1105 | Этоксиэтан (диэтиловый эфир) |
| 131 | 1107 | 2-Метокси-2-метилпропан (метил-трет-бутиловый эфир) |
| 132 | 1109 | 2-(2-Бутокси) этоксиэтанол (бутилкарбитол, монобутиловый эфир диэтиленгликоля) |
| 133 | 1110 | 2-(Изобутокси)этанол (бутилцеллозольв, моноизобутиловый эфир этиленгликоля) |
| 134 | 1111 | 2-(Изопропокси)этанол (изопропилцеллозольв, моноизопропиловый эфир этиленгликоля) |
| 135 | 1112 | 2-(2-Этоксиэтокси)этанол (моноэтиловый эфир диэтиленгликоля, этилкарбитол) |
| 136 | 1117 | 1-Метоксипропан-2-ол (альфа-метиловый эфир пропиленгликоля) |
| 137 | 1119 | 2-Этоксиэтанол (этиловый эфир этиленгликоля, этилцеллозольв) |
| 138 | 1124 | Дибутиловый эфир |
| Сложные эфиры (кроме эфиров кислот фосфора) |  |  |
| 139 | 1202 | Пентилацетат (н-амилацетат, уксусной кислоты н-пентиловый эфир) |
| 140 | 1206 | Бутилакрилат (акриловой кислоты бутиловый эфир) |
| 141 | 1210 | Бутилацетат (уксусной кислоты бутиловый эфир) |
| 142 | 1211 | Диметил-1,4-бензолдикарбонат (диметилтерефталат) |
| 143 | 1213 | Этенилацетат (винилацетат, уксусной кислоты виниловый эфир) |
| 144 | 1215 | Дибутилфталат (фталевой кислоты дибутиловый эфир) |
| 145 | 1217 | Диоктилфталат (1,2-бензилдикарбоновой кислоты диоктиловый эфир) |
| 146 | 1219 | Изоамилацетат (уксусной кислоты изопентиловый эфир) |
| 147 | 1221 | Изобутилацетат (уксусной кислоты изобутиловый эфир) |
| 148 | 1224 | Метилацетат (уксусной кислоты метиловый эфир) |
| 149 | 1225 | Метилакрилат (акриловой кислоты метиловый эфир) |
| 150 | 1232 | Метил-2-метилпроп-2-еноат (метакриловой кислоты метиловый эфир, метилметакрилат) |
| 151 | 1240 | Этилацетат (уксусной кислоты этиловый эфир) |
| 152 | 1248 | 3,5-Ди-трет-бутил-4-гидроксифенилпропионовой кислоты 2-(2- гидроксиэтокси) этиловый эфир (фенозан 28) |
| 153 | 1260 | 2-Этоксиэтилацетат (уксусной кислоты 2-этоксиэтиловый эфир, целлозольвацетат) |
| 154 | 1261 | Метилпропионат (пропионовой кислоты метиловый эфир) |
| 155 | 1262 | Метилэтилацетат (изопропилацетат, уксусной кислоты изопропиловый эфир) |
| 156 | 1270 | Диизододецифталат (фталевой кислоты диизододециловый эфир) |
| Альдегиды |  |  |
| 157 | 1301 | Проп-2-ен-1-аль (акролеин) |
| 158 | 1302 | Бензальдегид (альдегид бензойный) |
| 159 | 1303 | Пентаналь (валериановый альдегид) |
| 160 | 1304 | 2-Метилпропаналь (изобутиральдегид, изомасляный альдегид) |
| 161 | 1310 | Бутаналь (бутиральдегид, масляный альдегид) |
| 162 | 1314 | Пропиональдегид (пропаналь, пропионовый альдегид) |
| 163 | 1317 | Ацетальдегид (уксусный альдегид, этаналь) |
| 164 | 1325 | Формальдегид (метаналь) |
| Кетоны |  |  |
| 165 | 1401 | Пропан-2-он (ацетон) |
| 166 | 1408 | 4-Метилпентан-2-он (метилизобутилкетон) |
| 167 | 1409 | Метилэтилкетон (бутан-2-он) |
| 168 | 1410 | 3,5,5-Триметилциклогекс-2-ен-1-он (изофорон) |
| 169 | 1411 | Циклогексанон |
| Органические кислоты |  |  |
| 170 | 1507 | Уксусный ангидрид |
| 171 | 1508 | 1,3-Изобензофурандион (фталевый ангидрид) |
| 172 | 1512 | Акриловая кислота |
| 173 | 1519 | Пентановая кислота (валериановая кислота) |
| 174 | 1531 | Гексановая кислота (капроновая кислота) |
| 175 | 1532 | Мочевина (диамид угольной кислоты, карбамид) |
| 176 | 1534 | Бутановая кислота (масляная кислота) |
| 177 | 1546 | Пропионовая кислота |
| 178 | 1549 | Сульфаминовая кислота |
| 179 | 1551 | 1,4-Бензолдикарбоновая кислота (терефталевая кислота) |
| 180 | 1555 | Уксусная кислота |
| 181 | 1565 | Жирные синтетические кислоты фракций С10-С16 |
| Органические окиси и перекиси |  |  |
| 182 | 1610 | Диоксан-1,4 (диэтилендиоксид) |
| 183 | 1611 | Оксиран (эпоксиэтилен, этилена оксид) |
| Соединения, содержащие серу |  |  |
| 184 | 1703 | Дифенилсульфид |
| 185 | 1707 | Диметилсульфид |
| 186 | 1715 | Метантиол (метилмеркаптан) |
| 187 | 1728 | Этантиол (этилмеркаптан) |
| Амины |  |  |
| 188 | 1819 | Диметиламин |
| 189 | 1824 | 2-(Диметиламино)этанол (N,N-диметилэтаноламин) |
| 190 | 1852 | 2-Аминоэтанол (коламин, моноэтаноламин; этаноламин) |
| 191 | 1862 | Триметиламин |
| 192 | 1863 | Триэтиламин |
| 193 | 1864 | Три(2-гидроксиэтил)амин (триэтаноламин) |
| Прочие азотсодержащие |  |  |
| 194 | 2001 | Акрилонитрил (акриловой кислоты нитрил, проп-2-еннитрил) |
| 195 | 2002 | Ацетонитрил (цианистый метан, цианометан) |
| 196 | 2031 | Толуилеидиизоцианат |
| 197 | 2044 | 1-Изоцианато-4-(4-изоцианатофенил)метилбензол (4,4- дифенилметандиизоцианат, 4,4-метилендифенилизоцианат) |
| Гетероциклические соединения |  |  |
| 198 | 2425 | 2-Фурфуральдегид (2-фуральдегид, фурфураль, фурфурол) |
| 199 | 3620 | Диоксины (в пересчете на 2,3,7,8, тетрахлордибензо-1,4-диоксин) |
| Микроорганизмы |  |  |
| 200 | 2602 | Белково-витаминный концентрат (БВК) (по белку) |
| 201 | 2603 | Микроорганизмы и микроорганизмы-продуценты (отраслей промышленности: мукомольной, комбикормовой, дрожжевой, пивоваренной, кормовых дрожжей, аминокислот, ферментов, биопрепаратов на основе молочнокислых бактерий) (по общему бактериальному счету) |
| Пыль |  |  |
| 202 | 0008 | Твердые частицы фракции РМ 10 |
| 203 | 0010 | Твердые частицы фракции РМ 2.5 |
| 204 | 2902 | Твердые частицы суммарно |
| 205 | 2907 | Пыль неорганическая, содержащая двуокись кремния более 70% (динас и др.) |
| 206 | 2908 | Пыль неорганическая, содержащая двуокись кремния менее 70% (шамот, цемент, пыль цементного производства, глина, глинистый сланец, доменный шлак, песок, клинкер, зола, кремнезем, доломит, известняк, мел, огарки, сырьевая смесь, пыль вращающихся печей, боксит и др.) |
| 207 | 2911 | Пыль комбикормовая (в пересчете на белок) |
| 208 | 2912 | Пыль костной муки (в пересчете на белок) |
| 209 | 2913 | Пыль мясокостной муки (в пересчете на белок) |
| 210 | 2915 | Пыль стекловолокна |
| 211 | 2916 | Пыль стеклопластика |
| 212 | 2917 | Пыль хлопковая |
| 213 | 2919 | Пыль капрона |
| 214 | 2920 | Пыль меховая (шерстяная, пуховая) |
| 215 | 2921 | Пыль поливинилхлорида |
| 216 | 2922 | Пыль полипропилена |
| 217 | 2930 | Пыль абразивная (корунд белый, монокорунд) |
| 218 | 2931 | Пыль асбестосодержащая (с содержанием хризотил-асбеста до 10%) (по асбесту) |
| 219 | 2934 | Пыль аминопластов |
| 220 | 2936 | Пыль древесная |
| 221 | 2937 | Пыль зерновая (по массе) |
| 222 | 2938 | Пыль желатина |
| 223 | 2952 | Пыль текстолита |
| 224 | 2953 | Пыль фенопластов резольного типа (Э2-330-02; У2-301-07) |
| 225 | 2962 | Пыль бумаги |
| 226 | 2966 | Пыль крахмала |
| 227 | 2969 | Пыль полиамида ПА-610 |
| 228 | 2973 | Пыль сахара, сахарной пудры (сахарозы) |
| 229 | 2977 | Пыль талька |
| 230 | 2978 | Пыль тонко измельченного резинового вулканизата из отходов подошвенных резин |
| 231 | 2981 | Пыль ферросплавов (железо - 51%, кремний - 47%) (по железу) |
| 232 | 2989 | Пыль полиамида |
| 233 | 2990 | Пыль полистирола |
| 234 | 1544 | Полиэтилентерефталат |
| 235 | 2026 | Полиизоцианат |
| 236 | 2904 | Мазутная зола теплоэлектростанций (в пересчете на ванадий) |
| 237 | 2928 | Каучук СКТН (пыль) |
| 238 | 2942 | Полимер проп-2-енонитрила с проп-2-ен-1,2-дикарбоновой кислоты (акриловой кислоты нитрил полимер с проп-2-ен-1,2 дикарбоновой кислоты, нитрона пыль) |
| 239 | 2933 | Алюмосиликаты (цеолиты; цеолитовые туфы) |
| 240 | 2781 | Стеарин |
| 241 | 0323 | Кремния диоксид аморфный |
| Прочие соединения |  |  |
| 242 | 3920 | Полихлорированные бифенилы (по сумме ПХБ (ПХБ 28, ПХБ 52, ПХБ 101, ПХБ 118, ПХБ 138, ПХБ 153, ПХБ 180) |

--------------------------------

<*> Значения ориентировочно безопасных уровней воздействия загрязняющих веществ в атмосферном воздухе населенных пунктов и мест отдыха населения не являются нормативами качества атмосферного воздуха и применяются как ориентировочные для целей инвентаризации и нормирования выбросов загрязняющих веществ в атмосферный воздух.

Примечание. Коды загрязняющих веществ определены согласно СТБ 17.08.02-01-2009 "Охрана окружающей среды и природопользование. Атмосферный воздух. Вещества, загрязняющие атмосферный воздух. Коды и перечень".

Также постановлением утверждён перечень объектов воздействия на атмосферный воздух, источников выбросов, для которых не устанавливаются нормативы допустимых выбросов независимо от категории объекта воздействия на атмосферный воздух.

ПЕРЕЧЕНЬ

ОБЪЕКТОВ ВОЗДЕЙСТВИЯ НА АТМОСФЕРНЫЙ ВОЗДУХ, ИСТОЧНИКОВ ВЫБРОСОВ, ДЛЯ КОТОРЫХ НЕ УСТАНАВЛИВАЮТСЯ НОРМАТИВЫ ДОПУСТИМЫХ ВЫБРОСОВ ЗАГРЯЗНЯЮЩИХ ВЕЩЕСТВ В АТМОСФЕРНЫЙ ВОЗДУХ

(в редакции постановлений Минприроды от 26.02.2010 № 10, от 24.01.2011 № 4)

1. Установки мощностью менее 100 кВт: газопоршневые, газотурбинные, когенерационные, котельные и иные топливосжигающие, отопительные и технологические печи, теплогенераторы.

(п. 1 в ред. постановления Минприроды от 24.01.2011 № 4)

(см. текст в предыдущей редакции)

2. Банные печи мощностью менее 100 кВт.

3. Сушилки древесины электрические.

4. Аккумуляторные участки предприятий.

5. Оборудование для маркировки и упаковки, машины для сварки полимерных пленок.

6. Оборудование для стирки, отжима, сушки и глажения белья в прачечных.

7. Оборудование для мойки, инструменты для полировки механических транспортных средств.

8. Участки, на которых источники выделения оборудованы газоочистными установками и выброс загрязняющих веществ после газоочистных установок осуществляется в рабочую зону.

9. Отопительное оборудование, работающее на твердом, жидком и газообразном топливе, установленное на мобильных источниках, предусмотренное конструкцией данного транспортного средства (вагоны, баржи, суда и другое) и (или) буксируемое им.

10. Нестационарное оборудование и находящееся в резерве стационарное оборудование для получения электрической энергии (электрогенераторы, дизель-генераторы электрического тока, железнодорожные электростанции, дизельные электростанции на автомобильных прицепах, а также оборудование, оснащенное двигателями внутреннего сгорания).

11. Инструмент и оборудование, использующие топливо (бензопилы, бензорезы, кусторезы, газонокосилки, насосы и станции насосные, компрессоры и компрессорные станции и установки специальные, агрегаты сварочные, моечные машины и другое).

12. Оборудование, работающее на объектах строительства и ремонта, в том числе:

свайных работ;

приготовления, разогрева и нанесения битумов, мастик и других изоляционных материалов;

приёмки и хранения цемента, извести, гипса и других пылящих строительных материалов;

очистки конструкций с применением пескоструйных аппаратов;

резки и сварки металлов;

нанесения покрытий с использованием материалов, имеющих в составе летучие органические соединения, внутри и снаружи зданий, сооружений, коммуникаций, оборудования, транспортных средств, автомобильных дорог.

13. Оборудование для выполнения научно-исследовательских, опытно-конструкторских и опытно-технологических работ.

14. Физико-химические лаборатории при входном или выходном контроле качества продукции.

15. Оборудование лабораторий, осуществляющих измерения в области охраны окружающей среды.

16. Оборудование, используемое для газации, фумигации, обработки растений, зерна, семян и товаров (опрыскиватели, опыливатели, протравливатели, фулинаторы, разбрасыватели, смесители, аппараты аэрозольные и другое).

17. Оборудование, установленное на мобильных источниках (имеющих колесную/гусеничную базу) при следующих условиях:

время работы оборудования на одной производственной площадке составляет не более 3 месяцев;

оборудование соответствует действующим в данной области техническим нормативным правовым актам.

18. Эксплуатация холодильного оборудования, работающего на хладагентах, за исключением аммиака.

(в ред. постановления Минприроды от 26.02.2010 № 10)

(см. текст в предыдущей редакции)

19. Растениеводство.

20. Предоставление услуг в области растениеводства и животноводства, кроме ветеринарных услуг.

21. Охота и разведение дичи, включая предоставление услуг в этих областях.

22. Рыболовство, рыбоводство и предоставление услуг в этих областях.

23. Копирование записанных материалов:

копирование звукозаписей (копирование на грампластинки, компакт-диски, магнитные ленты и прочие носители музыкальных и других звукозаписей с оригинальной матрицы);

копирование видеозаписей (копирование на компакт-диски, магнитные ленты и прочие носители фильмов других видеозаписей с оригинальной матрицы);

копирование программных средств (копирование на диски, магнитные ленты и прочие носители программ и данных с оригинальной матрицы).

24. Монтаж приборов контроля и регулирования технологических процессов.

25. Производство часов.

26. Производство ювелирных изделий, монет и медалей.

27. Производство спортивных товаров.

28. Производство игр и игрушек.

29. Производство различной продукции (ювелирных изделий из недрагоценных металлов, метелок и щеток, канцелярских изделий, детских колясок), кроме производства щеток для обуви и одежды, спичек, линолеума и прочих твёрдых покрытий для пола.

30. Передача электроэнергии.

31. Распределение и продажа электроэнергии.

32. Снабжение паром и горячей водой, за исключением производства тепловой энергии тепловыми электростанциями, самостоятельными котельными, прочими источниками.

(в ред. постановления Минприроды от 26.02.2010 № 10)

(см. текст в предыдущей редакции)

33. Сбор, очистка и распределение воды.

34. Подготовка строительного участка (разборка и снос зданий, земляные работы, взрывные работы, разведочное бурение), предоставление услуг по добыче нефти и газа, строительные работы, непосредственно связанные с добычей нефти и природного газа.

(в ред. постановления Минприроды от 26.02.2010 № 10)

(см. текст в предыдущей редакции)

35. Строительство зданий и сооружений (общестроительные работы, строительство инженерных сооружений, устройство покрытий зданий и сооружений, строительство дорог, аэродромов и спортивных сооружений, строительство водных сооружений, строительство шахт).

36. Установка инженерного оборудования зданий и сооружений (оборудование всеми видами коммунальных удобств, несущими функциональную нагрузку в здании, электромонтажные работы, изоляционные работы, санитарно-технические работы).

37. Отделочные работы (штукатурные работы, столярные и плотничные работы, устройство покрытий пола и облицовка стен, малярные и стекольные работы и прочие).

38. Торговля автомобилями; техническое обслуживание и ремонт автомобилей; торговля автомобильными деталями, узлами и принадлежностями; торговля мотоциклами, их деталями, узлами и принадлежностями; техническое обслуживание и ремонт мотоциклов, ремонт бытовых изделий и предметов личного пользования, розничная торговля, кроме торговли автомобилями и мотоциклами.

(п. 38 в ред. постановления Минприроды от 26.02.2010 № 10)

(см. текст в предыдущей редакции)

39. Предоставление услуг гостиницами и ресторанами.

40. Деятельность железнодорожного транспорта (пассажирские и грузовые перевозки по магистральным линиям железной дороги во внутриреспубликанском и международном сообщениях, подчиняющиеся или не подчиняющиеся расписанию, в том числе перевозки принадлежащего пассажирам багажа; маневровые и буксировочные операции (включая деятельность на подъездных путях) и диспетчерская работа, услуги по перевозкам грузов на железнодорожных путях необщего пользования).

41. Деятельность прочего сухопутного транспорта (деятельность, обеспечивающая внутригородские, пригородные, междугородные и международные перевозки пассажиров по установленным маршрутам, подчиняющиеся расписанию, посадку и высадку пассажиров на указанных в расписании остановках, перевозки автобусами, троллейбусами, трамваями, метрополитеном и другими видами транспорта, деятельность прочего автомобильного пассажирского транспорта, не подчиняющегося расписанию: чартерные перевозки, предоставление услуг по перевозке с экскурсионными и прочими целями, аренда автобусов с водителем).

42. Деятельность водного транспорта (пассажирские и грузовые перевозки по водным путям, а также услуги экскурсионных, прогулочных и туристических судов, паромов, водных такси, буксиров или буксиров-толкачей, барж и тому подобное, аренда кораблей и судов с экипажем, маневровые и буксировочные операции).

43. Деятельность воздушного транспорта.

44. Транспортная обработка грузов (погрузка и разгрузка грузов и багажа пассажиров независимо от вида транспорта, используемого для перевозки, погрузка, включая крепление груза, и разгрузка судов (стивидорные работы).

45. Туристическая деятельность.

46. Организация перевозок грузов.

47. Связь (почтовая и курьерская деятельность, электросвязь).

48. Финансовая деятельность.

49. Операции с недвижимым имуществом, аренда и предоставление услуг.

50. Государственное управление.

51. Образование.

52. Здравоохранение и предоставление социальных услуг (деятельность по охране здоровья и предоставлению социальных услуг населению, а также в соответствии с международной практикой ветеринарная деятельность).

53. Уборка территории и аналогичная деятельность.

54. Деятельность общественных и религиозных организаций (объединений).

55. Деятельность по организации отдыха и развлечений, культуры и спорта, в том числе проведение салютов и фейерверков.

56. Предоставление услуг парикмахерскими и салонами красоты.

57. Физкультурно-оздоровительная деятельность.

58. Предоставление прочих индивидуальных услуг (биолокация, предоставление социальных услуг, например предоставление сопровождения (эскорта), деятельность службы знакомств, предоставление услуг брачными агентствами, тамадой, деятельность организаций по генеалогическим и графологическим исследованиям, предоставление услуг чистильщиками обуви, носильщиками, предоставление индивидуальных услуг, оказываемых при помощи машин-автоматов, таких как фотокабины, автоматы для измерения веса, роста и т.п., услуги по содержанию, уходу и дрессировке домашних животных, кроме сельскохозяйственных, дрессировке собак-сопровождающих, услуги общественных туалетов, предоставление прочих индивидуальных услуг).

59. Деятельность частных домашних хозяйств, нанимающих работников.

60. Деятельность частных домашних хозяйств по производству товаров для собственного потребления.

61. Деятельность частных домашних хозяйств по производству услуг для собственного потребления.

62. Многотопливные автозаправочные станции, автогазозаправочные станции, автозаправочные станции, контейнерные, блочные и передвижные автозаправочные станции, автомобильные газонаполнительные компрессорные станции.

(п. 62 введен постановлением Минприроды от 26.02.2010 № 10; в редакции постановления Минприроды от 24.01.2011 № 4)

(см. текст в предыдущей редакции)

63. Зверофермы, объекты по выращиванию и откорму крупного рогатого скота мощностью менее 400 условных голов, объекты для выращивания свиней мощностью менее 2 тысяч условных голов, объекты для выращивания птиц мощностью менее 40 тысяч мест для птиц.

(п. 63 введен постановлением Минприроды от 26.02.2010 г. № 10)

64. Содержание памятников, монументов, стел, мемориальных комплексов, памятных сооружений, пунктов "Вечный огонь".

(п. 64 введен постановлением Минприроды от 26.02.2010 г. № 10)

65. Источники выделения загрязняющих веществ в атмосферный воздух, расположенные внутри помещений, выбросы загрязняющих веществ от которых не поступают непосредственно в атмосферный воздух через аэрационные фонари, крышные вентиляторы, дефлекторы, другие устройства, посредством которых производится локализация поступления загрязняющих веществ в атмосферный воздух от источников выделения загрязняющих веществ.

(п. 65 введен постановлением Минприроды от 26.02.2010 № 10; в редакции постановления Минприроды от 24.01.2011 г. № 4)

(см. текст в предыдущей редакции)

66. Газопроводы и промысловые трубопроводы.

(п. 66 введён постановлением Минприроды от 24.01.2011 № 4)

Постановлением определено, что для загрязняющих веществ, не включенных в перечень загрязняющих веществ, для которых устанавливаются нормативы допустимых выбросов в атмосферный воздух (далее - перечень), и выбрасываемых (планируемых к выбросу) стационарными источниками выбросов юридического лица, индивидуального предпринимателя, осуществляющего хозяйственную и иную деятельность, связанную с выбросами загрязняющих веществ (далее - природопользователь), при проведении проектных работ или инвентаризации выбросов загрязняющих веществ в атмосферный воздух рассчитывается значение критерия значимости по формуле

160 x K  x (M )

j     j

F  = ---------------------,

j             n   i    i

ПДК    x SUM(H  x M )

м.р    i   j    j

где K - безразмерный коэффициент, учитывающий скорость оседания j загрязняющих веществ в атмосферном воздухе, равный для газообразных и жидких загрязняющих веществ 1; для твердых - 3;

M - суммарный выброс j-го загрязняющего вещества от объекта j воздействия на атмосферный воздух, имеющего стационарные источники выбросов, соответствующий наиболее неблагоприятным режимам работы, г/с;

ПДК - значение максимальной разовой предельно допустимой м.р концентрации (ориентировочно безопасного уровня воздействия) j-го загрязняющего вещества в атмосферном воздухе населённых пунктов и мест;

отдыха населения, миллиграммов в кубическом метре (мг/куб.м), определяемое согласно нормативам качества атмосферного воздуха.

В случае отсутствия установленного для загрязняющего вещества значения предельно допустимой концентрации (ориентировочно безопасного уровня воздействия) используется

наиболее низкое значение из:

среднесуточной предельно допустимой концентрации загрязняющего вещества в атмосферном воздухе, умноженной на 2.5;

предельно допустимой концентрации загрязняющих веществ в воздухе рабочей зоны, делённой на 4;

Hi - высота i-го стационарного источника выброса, из которого j выбрасывается j-е загрязняющее вещество в атмосферный воздух;

Mi - выброс j-го загрязняющего вещества от i-го стационарного j источника выброса, г/с.

В случае если значение критерия значимости загрязняющего вещества менее 20, то вещество не является значимым для данного объекта воздействия на атмосферный воздух, имеющего стационарные источники выбросов, и не подлежит включению в перечень.

В случае если значение критерия значимости загрязняющего вещества более 20, то определяется значение расчетных приземных концентраций j-го загрязняющего вещества, создаваемых стационарными источниками выбросов в жилой зоне в долях предельно допустимой концентрации или ориентировочно безопасного уровня воздействия загрязняющих веществ в атмосферном воздухе населенных пунктов и мест отдыха населения без учета фоновых концентраций. В случае если рассчитанное значение приземной концентрации j-го загрязняющего вещества:

менее 0.1, то вещество не является значимым для данного объекта воздействия на атмосферный воздух, имеющего стационарные источники выбросов, и не подлежит включению в перечень;

более 0.1, то вещество является значимым для данного объекта воздействия на атмосферный воздух, имеющего стационарные источники выбросов, и подлежит включению в разрешение на выбросы загрязняющих веществ в атмосферный воздух или в комплексное природоохранное разрешение, выдаваемое природопользователю территориальным органом Министерства природных ресурсов и охраны окружающей среды Республики Беларусь, и в случае, если рассчитанное значение приземной концентрации загрязняющего вещества более 1.0, для этого вещества устанавливается временный норматив допустимых выбросов загрязняющих веществ в атмосферный воздух. (в ред. постановлений Минприроды от 26.02.2010 № 10, от 15.12.2011 № 49) (см. текст в предыдущей редакции).

Результаты расчета включаются в проектную документацию на строительство или акт инвентаризации выбросов загрязняющих веществ в атмосферный воздух, на основании которых Министерство природных ресурсов и охраны окружающей среды Республики Беларусь по представлению его территориального органа или природопользователя принимает окончательное решение о включении загрязняющего вещества в перечень.

В настоящее время Министерством природных ресурсов и охраны окружающей среды проводится работа по уточнению положений постановления № 42 и запланирована разработка отдельного пособия и программного продукта по проведению инвентаризации и нормированию выбросов загрязняющих веществ для объектов воздействия, относящихся к IV и V категории воздействия на атмосферный воздух.

- Annex 2 – Tool for facility register

Разработка пособия по упрощённой инвентаризации выбросов загрязняющих веществ в атмосферный воздух для объектов воздействия на атмосферный воздух, выбрасывающих до 25 тонн в год загрязняющих веществ

Целью работы является разработка методологии и научного обоснования по оценке и проведению упрощенной инвентаризации выбросов загрязняющих веществ в атмосферный воздух для объектов воздействия на атмосферный воздух, выбрасывающих до 25 тонн в год загрязняющих веществ.

Для достижения поставленной цели в рамках выполнения работы будут решены следующие задачи:

1. Проведение анализа требований ТНПА, в том числе региональных и международных, опубликованных материалов в области проведения инвентаризации выбросов загрязняющих веществ в атмосферный воздух.

2. Разработка перечня источников выбросов загрязняющих веществ в атмосферный воздух с учетом проведения анализа имеющихся (действующих) инвентаризаций выбросов загрязняющих веществ в атмосферный воздух для предприятий, выбрасывающих до 25 тонн в год загрязняющих веществ и получающих разрешения на выбросы

3. При разработке перечня источников детально рассмотреть следующие источники выбросов:

автотранспорт;

отопление предприятий;

процессы сварки;

процессы газовой резки;

металлообработка, в том числе, кузнечно-прессовое, штамповочное производство, термическая обработка металлов, механическая обработка металлов, механическая подготовка поверхностей деталей;

деревообработка;

процессы, связанные с производством изделий из пластмасс, полимерных композиционных материалов;

прачечная и стирка;

химчистка;

механическая обработка (переработка) полимерных материалов;

пересыпка и хранение сыпучих материалов;

оборудование для нанесения лаков и красок, в т.ч исходя из процента содержания летучих органических соединений;

сельскохозяйственное производство и агропромышленный комплекс, включая зерноперерабатывающие предприятия и элеваторы;

участки изготовления резинотехнических изделий;

участки герметизации изделий радиоэлектронной аппаратуры полимерными материалами, сборочно-монтажное производство узлов и блоков радиоэлектронной аппаратуры;

вспомогательные и бытовые нужды, общезаводские лаборатории;

строительные участки;

автозаправочные станции.

4. Для указанных выше источников выбросов определить критерии, при которых возможно проведение упрощенной инвентаризации, с учетом необходимости соблюдения нормативов качества атмосферного воздуха. Следует определить, как для организованных, так и для неорганизованных источников выбросов допустимые параметры, которые на различных расстояниях от объекта (50, 100, 200, 300 м и т.д.) приводят к соблюдению нормативов качества атмосферного воздуха, в том числе и с использованием газоочистного оборудования с определенной степенью очистки выбросов, и соответственно, определить критерии, при которых выбросы от конкретного технологического процесса будут превышать пороговое значение в 25 тонн в год загрязняющих веществ.

5. Выбрать из указанных выше источников выбросов не менее трех, для которых разработать комплексные требования по трем компонентам окружающей среды – требования к выбросам загрязняющих веществ в атмосферный воздух, сбросам загрязняющих веществ в водные объекты, образованию и использованию отходов.

Исходными данными для выполнения работы являются нормативные правовые акты, международные и региональные стандарты, технические нормативные правовые акты Республики Беларусь, Российской Федерации, Украины, устанавливающие требования к инвентаризации выбросов загрязняющих веществ в атмосферный воздух; опубликованные материалы в области научного обоснования инвентаризации выбросов загрязняющих веществ в атмосферный воздух.

По окончании работ представляются:

1. Методология по оценке и проведению инвентаризации выбросов загрязняющих веществ в атмосферный воздух в виде аналитической записки.

2. Образцы методик комплексного предотвращения и контроля загрязнений для двух источников выбросов.

- Annex 3 – Content of official collection of emission factors in Ukraine

Украинский научный центр технической экологии

СБОРНИК ПОКАЗАТЕЛЕЙ ЭМИССИИ

(УДЕЛЬНЫХ ВЫБРОСОВ)

ЗАГРЯЗНЯЮЩИХ ВЕЩЕСТВ В АТМОСФЕРНЫЙ

ВОЗДУХ РАЗЛИЧНЫМИ ПРОИЗВОДСТВАМИ

ВТОРАЯ РЕДАКЦИЯ

Том 1 - 3

Донецк - 2010

З 41 Збірник показників емісії (питомих викидів) забруднюючих речовин в атмосферне повітря різними виробництвами. Друга редакція.

Т. 1 - 3 – Донецьк: ІАЦ ВАТ "УкрНТЕК" / ІЕТ "Чисте повітря"

2008, 2010. – 466 с.

Во второй редакции "Сборника показателей эмиссии (удельных выбросов) загрязняющих веществ в атмосферный воздух различными производствами" учтён отечественный опыт оценки выбросов загрязняющих веществ последних лет, прежде всего, результаты отраслевых разработок по заданию Минприроды Украины, требования к отчётности по форме государственного статистического наблюдения № 2 ТП (воздух), а также данные Руководства по инвентаризации атмосферных выбросов CORINAIR и реестра выделения и переноса загрязнения РВПЗ (PRTR).

Информационно-аналитический центр ОАО "УкрНТЭК" /

Информационно -экологическое общество "Чистый воздух"

2008, 2010 гг.

СОДЕРЖАНИЕ

I. СЖИГАНИЕ ОРГАНИЧЕСКОГО ТОПЛИВА	.............................................................5

1. Источники выбросов									5

2. Выбросы											7

3. Определение выбросов (Упрощённая методика)						7

4. Определение показателей выбрсов								9

Приложения											17

II. ЧЁРНАЯ МЕТАЛЛУРГИЯ	                                                                                      37

Агломерационное производство								37

Обжиг окатышей										38

Доменное производство									39

Мартеновское производство стали								42

Конвертерное производство стали								44

Электросталеплавильное производство							45

Производство ферросплавов									47

Прокатное производство									49

Производство огнеупоров									54

Производство извести									56

Литейное производство предприятий чёрной металлургии					59

III. КОКСОХИМИЧЕСКОЕ ПРОИЗВОДСТВО	..............................................................61

«Показатели эмиссии (удельные выбросы) загрязняющих веществ в атмосферный воздух основных производств коксохимических предприятий»					61

I.  Показатели эмиссии (удельные  выбросы) основныхпроизводств коксохимического производства											65

Ii. Рекомендации по установлению технологических нормативов выбросов загрязняющих веществ											87

Приложения											90

IV. ЦВЕТНАЯ МЕТАЛЛУРГИЯ	 .....................................................................................93

V. СВАРОЧНОЕ ПРОИЗВОДСТВО	 ....................................................................................98

Показатели эмиссии (удельные выбросы)загрязняющих веществ от процессов электро-, газосварки,наплавки, электро-, газорезки и напыления металлов				98

VI. ЗАПРАВКА АВТОТРАНСПОРТА	........................................................................120

VII. ПРОИЗВОДСТВО СИНТЕТИЧЕСКИХ ВОЛОКОН	……….....................................121

VIII. ГАЗОВАЯ ПРОМЫШЛЕННОСТЬ	........................................................................123

Газы углеводородные сжиженные топливные для коммунально-бытового потребления	123

Газораспределительные сети									136

Сжигание газов в факельных устройствах							148

IX. ПРОИЗВОДСТВО СТРОИТЕЛЬНЫХ ДОРОЖНЫХ МАТЕРИАЛОВ, ЖЕЛЕЗОБЕТОННЫХ ИЗДЕЛИЙ И СТРОИТЕЛЬНЫЕ УЧАСТКИ	...................................149

Производство асфальтобетона								149

Производство железобетонных изделий и строительные участки				161

X. МАШИНОСТРОЕНИЕ И МЕТАЛЛООБРАБОТКА	................................................165

Литейное производство машиностроительных и металлообрабатывающих предприятий	165

Производство деталей методом порошковой металлургии					179

Кузнечно-прессовое, штамповочное производство и термическая обработка металлов	180

Механическая обработка металлов								183

Механическая подготовка поверхностей деталей						190

Участки электрохимических покрытий							193

Производство лакокрасочных покрытий							234

Производство эмалевых покрытии								262

Деревообрабатывающее производство							264

Участки остеклования									283

Производство по переработке пластмасс							285

Производство деталей из полимерных композиционных материалов			289

Участки изготовления резинотехнических изделий						293

Участки герметизации изделий радиоэлектронной аппаратуры полимерными материалами												302

Производство печатных плат									309

Микроэлектронное производство								331

Сборочно-монтажное производство узлов и блоков радиоэлектронной аппаратуры		344

Вспомогательные и бытовые службы							352

Общезаводские лаборатории									356

XI. АГРОПРОМЫШЛЕННЫЙ КОМПЛЕКС	.......................................................................360

Зерноперерабатывающие предприятия и элеваторы						360

Сахарная промышленность									366

Кондитерская промышленность								371

Комплексная переработка мелассы								385

Дрожжевая и спиртовая промышленность							391

Масложировая промышленность								393

Табачная промышленность									404

Парфюмерно-косметическая промышленность						410

Консервная плодоовощная промышленность						427

Пиво-безалкогольная промышленность							430

Соляная промышленность									435

Молочно-консервная промышленность							437

XII. СЕЛЬСКОЕ ХОЗЯЙСТВО	....................................................................................438

Животноводческие комплексы и зверофермы						438

XIII. СЖИГАНИЕ ОТХОДОВ	....................................................................................448

Сжигание бытовых, муниципальных отходов						448

- Annex 4. Approaches for ELVs establishing for the small combustion installations

The small combustion installations (SCI) including applied for residential heating — fireplaces, stoves, cookers, small boilers (< 50 MWth) are the noticeable sources of ambient air pollution, especially of particulate emissions.

Figure 1 from the report presents PM2.5 emissions in the GAINS Europe area for the years 2000, 2005 and 2020. We can see that the share of small combustion installations is nearby 25% in total PM2.5  emissions and is higher than the share of large combustion plants.

Figure 1 — PM2.5 emissions in the GAINS Europe area

Mainly (nearby 80%) PM2.5 emissions were due the biofuel combustion. SCI in domestic sector are responsible for quite significant share of the total PM2.5 emissions in EU countries. Estimations made in the Report shows that the shares of SCI PM2.5 emissions are as much as 76% in Latvia, 74% in Poland, 57% in Denmark, 38% in Estonia. But for some countries this share is rather small: Germany – 16%, Sweden – 12% and UK – 9%. The last figure is particularly surprising, and shifts our thinking about the “chimneys country”, but such low emissions are due to the usage of the best available practices and techniques in the sector.

In the work of Thomas Nussbaumer some techniques of biofuel combustion were analysed and wide ranges of PM2.5 emission factors have been reported. Analysis of the data and costs gives ability to determine best available practices (BAP) for biomass combustion in SCI and corresponding emission levels.

For example, for wood stoves, low PM2.5 emissions of less than 25 mg/MJ are possible by utilization of small logs of dry wood being added in small batches and igniting the fuel from the top. The same figures can be achieved for modern wood boilers with forced downdraft combustion and electronic combustion control. The table 1 from the June 2010 report presents some PM2.5 best practice emission levels for various types of SCI. The values can be used as ELVs for wood combustion in SCI.

| The type of the small combustion installation | Abatement technique | Achievable PM2.5 emission level (mg/MJ) | Corresponding ELV (mg/m3 at 13% O2) |
| --- | --- | --- | --- |
| Open fireplaces |  | 50 — 100 | 75 —150 |
| Wood stoves and closed inset appliances |  | 15 — 25 | 20 — 40 |
| Log wood boilers (with heat storage tank) |  | 10 — 20 | 15 — 30 |
| Pellet stoves and boilers |  | 10 — 20 | 15 — 30 |
| Automatic combustion plants | Multicyclone | 50 — 100 | 75 —150 |
|  | Simple ESP | 15 — 35 | 20 — 50 |
|  | Improved ESP | 5 — 15 | < 10 — 20 |
|  | Fabric filter | < 5 | < 10 |

Table 1 — Achievable best practice PM2.5 emissions for various small combustion installations (<50 MWth)

The total number of combustion plants with thermal capacity less than 50 MWth, as reported by Member States in 2011, is 142,542 with total capacity of 689 GWth. In the capacity range 1-5 MWth there are 107,506 installations, 5-20 MWth – 29 958 installations, and 20-50 MWth – 5,078 installations. The dominant fuel for reported plants is natural gas (92% for plants 1-5 MWth, 87% for plants 5-20 MWth, and 54% for plants 20-50 MWth). The usage of biomass is less than 15% for all capacity classes, but with the growth of capacity the share of biomass and other solid fuels is growth.

There are some standards used within the European Union for the testing of wood fired boilers and stoves. Among them:

- EN 14785 — Residential space heating appliances fired by wood pellets – requirements and test methods. The thresholds for CO emissions are established in the standard

- EN 303-5 —Heating boilers for solid fuels, hand and automatically stoked, nominal heat output of up to 300 kW. In this standard the thresholds for CO and dust are pre-defined.

But mainly, the values of ELV for SCI in Member States are established by national legislation. All-European standards in this field are nowadays in discussion. In the table 2 below (AMEC Report, July 2012) the data for other important pollutant NOx for several EU member states are collected.

| Member State | Capacity class (MWth) | Fuel | Fuel detail | Notes | ELV for NOx (mg/Nm3) |
| --- | --- | --- | --- | --- | --- |
| Austria | 1-5 | Biomass | 13%O2 | Steam boilers. Average 250-500 range | 375 |
|  | 5-20 |  |  | Steam boilers. Average 250-350 range | 275 |
|  | 20-50 |  |  | Steam boilers. Average 250-350 range | 275 |
|  | 1-5 | Other solid fuels | 6%O2 | New steam boilers or existing ones from 2018 | 400 |
|  | 5-20 |  |  |  | 350 |
|  | 20-50 |  |  |  | 350 |
|  | 1-5 | Liquid fuels | 11%O2 | Steam boilers. 150 gasoil | 400 |
|  | 5-20 |  |  |  | 350 |
|  | 20-50 |  |  |  | 350 |
|  | 1-5 | Natural gas | 6%O2 | New steam boilers or existing ones from 2018 | 125 |
|  | 5-20 |  |  |  | 100 |
|  | 20-50 |  |  |  | 100 |
| Czech Republic | 1-50 | Biomass |  | Other furnace, biomass. 400 mg/Nm3 for fluid combustion chamber | 650 |
|  | 1-50 | Other solid fuels |  | Other furnace, biomass. 400 mg/Nm3 for fluid combustion chamber | 650 |
|  | 1-5 | Liquid fuels |  |  | 500 |
|  | 5-20 |  |  |  | 450 |
|  | 20-50 |  |  |  | 450 |
|  | 1-50 | Natural gas |  |  | 200 |
| Germany | 1-5 | Other solid fuels |  | 300 mg/Nm3 for fluidized bed furnaces | 500 |
|  | 5-20 |  |  |  | 400 |
|  | 20-50 |  |  |  | 400 |
|  | 1-50 | Liquid fuels |  | 180 mg/Nm3 for boiler temperatures <110 C and pressure <0.05 MPa; 200 mg/Nm3 for boiler temperatures 110-210 C and pressure 0.05-1.8 MPa; 250 mg/Nm3 for boiler temperatures >210 C and pressure >1.8 MPa | 200 |
|  | 1-50 | Natural gas |  | 180 mg/Nm3 for boiler temperatures <110C and pressure <0.05 MPa; 200 mg/Nm3 for boiler temperatures 110-210 C and pressure 0.05-1.8 MPa; 250 mg/Nm3 for boiler temperatures >210 C and pressure >1.8 MPa | 110 |
| Finland | 1-50 | Biomass |  | 375 mg/Nm3 for new plants | 450 |
|  | 1-50 | Other solid fuels |  | 270 mg/Nm3 for new plants | 420 |
|  | 1-5 | Liquid fuels |  | 800 mg/Nm3 for new plants | 900 |
|  | 5-20 |  |  | 800 mg/Nm3 for new plants | 900 |
|  | 20-50 |  |  | 500 mg/Nm3 for new plants | 600 |
|  | 1-5 | Natural gas or other gaseous fuel |  | 340 mg/Nm3 for new plants | 400 |
|  | 5-20 |  |  | 340 mg/Nm3 for new plants | 400 |
|  | 20-50 |  |  | 200 mg/Nm3 for new plants | 300 |
| France | 1-5 | Biomass |  |  | 500 |
|  | 5-20 |  |  |  | 500 |
|  | 20-50 |  |  |  | 600 |
|  | 1-5 | Liquid fuels |  |  | 550 |
|  | 5-20 |  |  |  | 500 |
|  | 20-50 |  |  |  | 600 |
|  | 1-5 | Natural gas |  |  | 200 |
|  | 5-20 |  |  |  | 150 |
|  | 20-50 |  |  |  | 225 |
| Latvia | 20-50 | Solid fuels |  |  | 600 |
|  | 20-50 | Liquid fuels |  |  | 450 |
|  | 20-50 | Natural gas |  |  | 350 |
| United Kingdom | 20-50 | Biomass and solid fuels |  | 450 mg/Nm3 for stocker firing; 650 mg/Nm3 for other methods | 550 |
|  | 20-50 | Liquid fuels |  | 450 mg/Nm3 for heavy fuel oil; 200 mg/Nm3 for distillates | 325 |
|  | 20-50 | Natural gas |  |  | 140 |

Table 2 — Emission limit values for NOx in some EU Member States national legislation (from AMEC Report, July 2012)

The last requirements for ambient air emissions can be found in Eco-design standards. These standards are worked out in line with Eco-Design Directive and are applicable for all Member States. The recent Regulation establishes concrete requirements for space heaters and combination heaters. Among them the requirements for water heating energy efficiency, for sound power level, and for emissions of NOx. The last are presented as technical standards normalized by the fuel input in terms of gross calorific value. For example, for fuel boiler combination heaters using gaseous fuels the emission of NOx shall not exceed 56 mg/kWh, for the same heaters using liquid fuels — 120 mg/kWh.

See http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2010:334:0017:0119:en:PDF

SMEs and the environment in the European Union, 2010. Danish Technological Institute and PLANET S.A., Greece

ENV/EPOC/WPIEEP(2012)1/FINAL: Green transformation of small businesses: achieving and going beyond environmental requirements

http://servicii.gov.md

http://www.eea.europa.eu/themes/air/emep-eea-air-pollutant-emission-inventory-guidebook

http://likumi.lv/doc.php?id=256088

ENV/EPOC/WPIEEP(2012)1/FINAL: Green transformation of small businesses: achieving and going beyond environmental requirements

http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=CELEX:31994L0063:EN:HTML

http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2009:285:0036:01:EN:HTML

Options for limit values for emissions of dust from small combustion installations < 50 MWth. UNECE Convention on Long-range Transboundary Air Pollution Subgroup on Small Combustion Installations under EGTEI. June 2010 http://www.citepa.org/old/forums/egtei/Options%20for%20PM%20ELVs%20for%20SCI%20%20final.pdf

http://gains.iiasa.ac.at/index.php/gains-europe

Emissions from households and other small combustion sources and their reduction potential. TSAP Report #5,  Janusz Cofala, Zbigniew Klimont, IIASA, June 2012 http://gains.iiasa.ac.at/images/stories/reports/TSAP/TSAP-SMALL%20SOURCES-20120612.pdf

Thomas Nussbaumer. Overview on Technologies for Biomass Combustion and Emission Levels of Particulate Matter // Swiss Federal Office for the Environment (FOEN), Zürich, June 2010 http://www.citepa.org/old/forums/egtei/Nussbaumer_EGTEI-Report_final.pdf

Small-scale electrostatic precipitators

Collection and analysis of data to support the Commission in reporting in line with Articles 73(2)(a) of Directive 2010/75/EU on industrial emissions from the combustion of fuels in installations with a total rated thermal input below 50 MW. European Commission. Final Report. AMEC Environment & Infrastructure, July 2012 http://www.endseurope.com/docs/120723a.pdf

General Guidance Manuals (revised April 2012) for combustion and incineration processes and for other sectors can be found at DEFRA site http://www.defra.gov.uk/industrial-emissions/las-regulations/guidance/

Directive 2009/125/EC of the EU Parliament and of the Council of 21 October 2009 establishing a framework for the setting of ecodesign requirements for energy-related products. 
http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2009:285:0010:0010:EN:PDF

Commission Regulation (EU) No 813/2013 of 2 August 2013 implementing Directive 2009/125/EC of the European Parliament and of the Council with regard to ecodesign requirements for space heaters and combination heaters. http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2013:239:0136:0161:EN:PDF