EuropeAid/129522/C/SER/Multi Contract number 2010/232-231

Air Quality Governance in the ENPI East Countries

Implementing legislation recommendations for integrated permitting, registration procedure and GBR permitting system

20 June 2014

| Version |  | Date | Description | Prepared by | Reviewed by |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | 20 June 2014 | Subtask 2.1.2.4 Develop and consult implementing legislation recommendations for integrated permitting and registration procedure, and Subtasks 2.1.2.5 Develop and consult implementing recommendations for GBR permitting system legislation | Short-term experts of Component 2 | Aiga Kāla, Key Expert 2 – Air Quality Assessment and Management Vladimir Morozov, Key Expert 3 – Industry/IPPC |  |

# SUMMARY

Project Title:	Air Quality Governance in ENPI East Countries

Contract Number:	2010/232-231

Countries:	Armenia, Azerbaijan, Belarus, Georgia, the Republic of Moldova, the Russian Federation, and Ukraine

Lead Contractor

Name	MWH

Address	Nysdam Office Park
Avenue Reine Astrid, 92
B-1310 La Hulpe
Belgium

Tel. number	+32 2 655 2230

Fax number	+32 2 655 2280

Contact person	Aïda Yassine, Project Manager

Signature

Date of report:	20 June 2014

Experts:

Monika Přibylová, International Non-Key IPPC Expert

Michael Begak, International Non-Key IPPC Expert

Oleg Bely, Local Non-Key IPPC Expert

Tatiana Guseva, Local Non-Key IPPC Expert

Viktar Khodzin, Local Non-Key IPPC Expert

Dmytro Laznenko, Local Non-Key IPPC Expert

Content

SUMMARY	3

List of abbreviations	5

1.	BACKGROUND	6

2.	Implementing legislation recommendations for integrated permitting and registration procedure	7

3.1.	Key features of GBR permitting system	12

3.2.	Definition of General Binding Rules	13

3.3.	Scope of Application of GBRs	14

3.4.	Example of GBR practice	15

3.5.	Development of GBRs	16

3.6.	Permitting Procedure under GBRs	18

3.6.1.	Proposal of new permitting procedure with the use of GBRs	18

3.6.2.	Assessment of Application and Determination of Permit Conditions	20

3.6.3.	Timeline of the Procedure	21

3.7.	Transition to GBR’s permitting system	22

3.8.	Role of Single-Medium Permitting	22

3.9.	Legal and Institutional Issues	23

3.10.	Timing of the Transition	24

3.	implementing recommendations for GBR permitting system legislation	12

Annex 1. Form for integrated permit application	29

Annex 2. Integrated permit template	67

Annex 3. Model administrative regulations for Designated Administrator responsible for integrated environmental permitting	77

Annex 4 Registration form for low polluting activities	103

# List of abbreviations

BAT 		– Best Available Techniques

BREF 		– BAT Reference document

DA 		– Designated Administrator

EECCA	– Eastern Europe, Caucasus and Central Asia

EIA			 – Environmental Impact Assessment

ELV		– Emission Limit Value

ENPI 			– The European Neighbourhood and Partnership Instrument (ENPI) that supports the European Neighbourhood Policy (ENP)

EP			– Environmental Permitting

EU 			– European Union

GBR 		– General Binding Rules

IED 		– Industrial Emissions Directive

IPPC 		– Integrated Pollution Prevention and Control

MPAs		– medium polluting activities

NGO 		– Nongovernmental Organisation

OECD 		– Organisation for Economic Cooperation and Development

RO 		– Responsible Official

# BACKGROUND

This report is prepared in the frame of Air Quality Governance project, Component 2 - Activities addressing the Industrial Sector, including Energy, Task 2.1 Support towards the implementation of integrated permitting, Subtasks 2.1.2.4 Develop and consult implementing legislation recommendations for integrated permitting and registration procedure and 2.1.2.5 Develop and consult implementing recommendations for GBR permitting system legislation.

The aim of this report is to provide necessary part of comprehensive recommendations for pending reforming the permitting system and IPPC implementation in countries participating in the Air-Q-Gov project (Armenia, Azerbaijan, Belarus, Georgia, the Republic of Moldova, the Russian Federation, and Ukraine).

The report refers to specific legislation of transitional period ensuring necessary steps for implementation of new instruments and increased efficiency of environmental permitting.

Presented in this report recommendations are based on:

- EU legislation and guidance related to IPPC

- The experience with the implementation of the IPPC Directive and Industrial Emissions Directive (IED), including the new EU member states

- Conclusions and findings of similar IPPC projects and guides like OECD Integrated environmental permitting guidelines for EECCA countries, 2005 or the Handbook for Implementation of EU Environmental Legislation, 2008

- Discussions at IPPC & Permitting workshops of the Air-Q-Gov project.

The overall scope of the implementing legislation should be defined in the environmental permitting law as shown in separate report for Subtask 2.1.2.3 Develop and consult Law on Environmental permitting recommendations.

In minimum the implementing legislation shall cover the following:

- Application form and instructions how to fill it in

- Scope of the statement from concerned authorities

- Permit form

- Registration form

- Compulsory methodologies, e.g. the BAT guidance.

# Implementing legislation recommendations for integrated permitting and registration procedure

  - Key features of integrated permitting system (IPS)

The key features of IPS define the scope of aspects which have to be included either in the primary or secondary (implementing) legislation for introduction and functioning of the new permitting system. The following features are based on principles defined in the Concept for transformation of environmental permitting system under the task 2.1.2.1:

- Operations of large polluting activities should be regulated by integrated environmental permits based on BAT

- The level of integration in the integrated permit shall cover all environmental aspects, including installation level monitoring and reporting

- Consideration of both technological and environmental aspects during determination of permit conditions

- The procedure for issuing integrated permits has to be transparent and provide tools for public participation and ensure access to information during the permitting procedure

- Involvement of all relevant administrative bodies during the permitting procedure – consultations with the concerned state and regional public administration bodies and local authorities

- The permitting authority shall consider local situation during determination of permit conditions and shall have the right to set permit conditions stricter than minimum legally binding requirements

- Use of sectoral BAT guidance based on EU BAT reference documents and other international experience

- Integrated permits should have long validity periods and clear procedures for permit revision and termination

- Information and methodology support to the regulatory system

These features were elaborated in the report on task 2.1.2.3 “Develop and consult recommendations on environmental permitting law”, which defines the scope of a law and proposes aspects to be covered in the primary legislation.

  - Scope of primary and secondary legislation

The law on environmental permitting shall cover the following key aspects of the integrated permitting system:

•	Scope of the regulatory system - level of integration in the integrated permit

•	Designation of competent authority/ies for granting of integrated permits (at different public administration levels) and definition of their responsibilities

•	Designation of competent authority/ies for integrated inspection and definition of their responsibilities

•	Specification of industrial and agricultural activities with high pollution potential in concern

•	Obligations of operators under integrated permitting regulatory system

•	Steps of the integrated permitting procedure (from elaboration of a permit application till permit issuance)

•	Content of a permit and procedure for setting permit conditions

•	Exchange of information on BAT

•	Validity of a permit, procedure for revision and termination of permits

•	Procedure for integrated inspections

•	Penalties for noncompliance with the permit conditions or other obligations imposed on operators by environmental legislation

•	Transitory provisions specifying dates when new and existing installations shall obtain integrated permits.

The following subchapters describe only those aspects, which are recommended to be covered by the implementing (secondary) legislation, since the main aspects, which need to be included in the primary legislation, are elaborated in the report on task 2.1.2.3 “Develop and consult recommendations on environmental permitting law”.

The division between aspects to be included in the environmental permitting law or in the implementing regulations is always based on the national legal system, the experience with drafting and approval of the legislation and requirements for flexibility. Usually the implementing regulations can be updated faster and do not require wide inter-ministerial consultations.  The following subchapters give suggestions of areas which are commonly prescribed by the implementing legislation in EU member states.

  - Determination of the regulated polluting activities

Activities regulated by integrated permitting regime are usually defined in a listed form, which is either an annex to the environmental permitting law or an annex to the implementing regulation. The latter case is used for example in the UK, where the regulation contains also general interpretation of terminology used and capacity thresholds.

In most of the new EU member countries the list of the regulated activities is provided as an annex to permitting law since it was provided as the annex of the Directive 2008/1/EC on IPPC as well as to the Industrial Emissions Directive. By specification of the regulated activities in a form of an annex to the integrated permitting law it was more easily to prove to the European Commission the correct transposition of the IPPC Directive during the accession process.

The example of the list of activities regulated by integrated environmental permit is provided in the Annex A of the report on task 2.1.1.6 „Integrated Permitting guidance“.

  - Integrated permitting procedure steps

The main steps of permitting procedure have to be specified in the environmental permitting law because the permitting authority/ies is responsible for carrying out the procedure. The following figure presents the sequence of main steps of permitting procedure.

BACKGROUND TO APPLICATION

The basic steps of the permitting process can be described as follows. In the first step the application will be drafted. The operator will consult the permitting authority and the application will be submitted. This step is called “background to application”. In the second step “consideration of the application” the application will be verified and there will be an assessment and a statement. In the third step it will be decided if a permit is issued, or if the application is to be rejected. The last two steps of the process, “modification of installation, substantial change”, and “permit review “ describe the situations when an existing permit can be changed. Figure 1 shows these basic steps of permitting.

CONSIDERATION OF APPLICATION

The basic steps of the permitting process can be described as follows. In the first step the application will be drafted. The operator will consult the permitting authority and the application will be submitted. This step is called “background to application”. In the second step “consideration of the application” the application will be verified and there will be an assessment and a statement. In the third step it will be decided if a permit is issued, or if the application is to be rejected. The last two steps of the process, “modification of installation, substantial change”, and “permit review “ describe the situations when an existing permit can be changed. Figure 1 shows these basic steps of permitting.

ISSUING PERMIT

The basic steps of the permitting process can be described as follows. In the first step the application will be drafted. The operator will consult the permitting authority and the application will be submitted. This step is called “background to application”. In the second step “consideration of the application” the application will be verified and there will be an assessment and a statement. In the third step it will be decided if a permit is issued, or if the application is to be rejected. The last two steps of the process, “modification of installation, substantial change”, and “permit review “ describe the situations when an existing permit can be changed. Figure 1 shows these basic steps of permitting.

PERMIT REVIEW

The basic steps of the permitting process can be described as follows. In the first step the application will be drafted. The operator will consult the permitting authority and the application will be submitted. This step is called “background to application”. In the second step “consideration of the application” the application will be verified and there will be an assessment and a statement. In the third step it will be decided if a permit is issued, or if the application is to be rejected. The last two steps of the process, “modification of installation, substantial change”, and “permit review “ describe the situations when an existing permit can be changed. Figure 1 shows these basic steps of permitting.

MODIFICATION OF INSTALLATION /SUBSTANTIAL CHANGE

The basic steps of the permitting process can be described as follows. In the first step the application will be drafted. The operator will consult the permitting authority and the application will be submitted. This step is called “background to application”. In the second step “consideration of the application” the application will be verified and there will be an assessment and a statement. In the third step it will be decided if a permit is issued, or if the application is to be rejected. The last two steps of the process, “modification of installation, substantial change”, and “permit review “ describe the situations when an existing permit can be changed. Figure 1 shows these basic steps of permitting.

Figure 1: Basic PROCEDURAL steps

In majority of EU countries the application for integrated environmental permit has to follow some prescribed form. The application form is either compulsory, set by an implementing regulation or recommendable, issued in an official guidance. The proposed integrated permit application form is provided in Annex 1 to this report.

Apart from the application form, it is also useful to elaborate instructions how to fill in the form. These instructions can be a part of the implementing regulation or can be included in a guidance document issued by the relevant Ministry.

The permitting authority can develop instructions how to assess the completeness and correctness of the application. These instructions can be issued as document for internal use  or can as methodological guidance, or as part of implementing regulations in the case there are several permitting authorities. In such a case common instruction supports the uniform approach for assessing applications across the country and in authorities.

The content of integrated permit, setting of permit conditions and permit review process are discussed in the following subchapter.

  - Content of the permit and procedure for defining permit conditions

Although the content of integrated permit can be specified in the environmental permitting law, the permit form and the way how to set permit conditions is recommended to be described in the implementing regulation or methodological guidance. Recommended permit template is provided in Annex 2 of this report. This template contains also examples of permit conditions’ wording.

In case there are more authorities which have to be consulted during preparation of permit conditions or which are responsible for issuing an integrated permit it is advisable to have one permit form and common instructions for setting permit conditions for different environmental aspects.  To ensure that they are commonly used by all participating authorities it is recommended to regulate this on level of implementing regulations

Concerning the aspects to be taken into account while setting permit conditions, it is important that all relevant legislation is considered (e.g. air protection legislation and water protection law). This is important in those permitting regimes, where the integrated permitting legislation sets the framework for permitting, including the permit conditions’ scope, but the detailed technical provisions for protecting environmental media are specified in the environmental media specific regulations. Furthermore procedures on modification or change of an integrated permit are usually outlined in the environmental permitting law. Nevertheless practical instructions on when it is needed to revise and issue completely new permit and when it is sufficient only to register the modification or make minor change in the valid permit are recommended to be issued as methodological guidance for permitting authorities.

Similar instructions are recommended to be elaborated for a permit review process. This can be done in the form of a check list or instructions issued as methodological guidance for permitting or inspection authorities whichever is designated as responsible authority for permit reviews.

  - Exchange of information on BAT

The application of the BAT concept requires systematic way for collecting information about BAT, dissemination of this information and method how to define and assess BAT applicability for an installation.

For collecting and dissemination of information about BAT some EU countries developed national systems for information exchange on BAT. In these cases the main requirements for such a system are prescribed by the environmental permitting law, while detailed division of duties and rights of participants of BAT information exchange system is specified in the implementing regulation, such as government decree. The example of such requirements is provided in the report elaborated under task no. 2.2.2.2. “Recommendations for organization and operation of the BAT/cleaner technologies centres”.

The output of the information exchange on BAT process usually is some kind of BAT guidance. In EU these are either BAT reference documents or BAT conclusions. In case BAT guidance should be used in compulsory way, documents have to be approved and issued as implementing regulations.

The last issue under BAT information exchange system concerns the instructions how to assess BAT for a particular installation. These instructions can be a part of the instructions how to fill in a permit application. In some cases these instructions are issued separately as guidance since they can be used as screening method for preliminary assessment of new investments, upgrades or modifications in existing installations before starting to fill in the application form. Based on this assessment the investor or operator can see whether existing or new installation is in line with BAT and thus can obtain integrated permit with fulfillable conditions.

  - Transitory provisions

The transitory provisions are commonly specified in the environmental permitting law as a date from which the existing and new installations cannot operate without an integrated permit. If the number of IPPC installations in a country exceeds 1000 (taking also into account the capacity of the permitting authorities), it is advisable to elaborate and issue a detailed schedule for bringing the existing installations in compliance with the integrated permitting system requirements.

The existing installations can be divided in several groups (e.g. by sector or by age) and have different deadlines for submission of application or obtaining an integrated permit. The approved schedule can be issued as an implementing legislation to make the agreed deadlines compulsory. An example of such a schedule issued in the UK is available in the Pollution Prevention and Control Regulations from 2000. The proposal of the phase-in schedule for project countries will be part of the report for task 2.1.5.1 “Phase-in schedule for existing LaPA”.

Model forms for integrated permit application and integrated permit template are provided as Annex 1 and Annex 2; model administrative regulations for Designated Administrator responsible for integrated environmental permitting is presented as Annex 3 to this report.

# implementing recommendations for GBR permitting system legislation

This chapter provides recommendations for the specification of GBR permitting system as an option for improving the existing permitting system for medium polluting activities/ installations in countries participating in the project (Armenia, Azerbaijan, Belarus, Georgia, the Republic of Moldova, the Russian Federation and Ukraine).

It focuses on four main issues as follows:

- Providing an overview of key features of GBR permitting system

- Specification of the scope of application of GBRs system

- Ways how to develop GBRs

- Giving the outline of the GBR permitting procedure.

## Key features of GBR permitting system

For medium polluting activities (MPAs), the reduction of environmental impact achieved as a result of implementing BAT at each individual installation (as under integrated permitting) may not be worth the effort spent by the operator to justify and for the regulator to assess the proposed techniques. Therefore the objective is to regulate the sectors of MPAs rather than consider each installation individually. The following features are desirable in a permitting system for MPAs:

- Operators should be encouraged to move away from end-of-pipe techniques for reducing discharges to air or water and adopt integrated operation and maintenance solutions, including effective environmental management techniques.

- The permitting procedure should reduce, compared with integrated permitting, the amount of information the permitting authority has to review and the degree of detailing needed in each case.

- The permitting process should be transparent and easy for the operator and the general public to understand, by reference to published guidance or rules for particular classes of installations.

- To achieve the public acceptance of any system of permitting the basis of the system has to be open and transparent. It is also important that operators can see that they are being treated fairly compare to the others. The process of producing pertinent general rules or guidance must be open to comments by the public and other stakeholders.

- In general a simplified system has to offer the same approach between sectors and should be proportionate to the environmental risk level. It must be seen to retain fairness between operators subject to full integrated permitting and those subject to simplified permitting. This is greatly secured by transparency in setting statutory emission/effluent limits and granting public access to permits.

- Permit conditions should, wherever possible, be consistent with business practices for a given category of installations. For example, monitoring and reporting requirements based on process data (energy or water consumption, materials use, etc.) should be reasonably preferred over pollution measurements, as the latter are much more expensive. At the same time, permit conditions must be clear and the permitting authority must have powers to inspect its compliance.

In the permitting system for MPAs, these criteria are applied through the introduction of General Binding Rules (GBRs), which are described in the following sections.

## Definition of General Binding Rules

GBRs are limit values or other conditions (defined in particular environmental laws, regulations and ordinances) at sector level or wider, that are given with the intention to be used directly to set permit conditions. They provide direct conditions or minimum standards. GBRs are either binding to the relevant environmental permitting authority (permitting authority) or to the operator. However, under certain conditions, some general rules may not be mandatory and deviation will be allowed, although the normal expectation would be that the rules are used directly.

General Binding Rules cover operational aspects of an installation and prescribe certain permit conditions. Under GBR regime, the permitting authority issues permits consistent with specific GBR requirements.

Key advantages of GBRs include:

- Adoption of uniform emission standards (statutory ELVs)

- Simplified application procedure and forms, resulting in reduced bureaucracy

- Transparency, predictability and consistency

- Uniform monitoring requirements, facilitating compliance assurance

- No potential to distort competition within an industrial sector

- Reduced costs for the regulator (although the development of GBRs requires initial resource investment) and the operator. For example, the UK Environment Agency estimates that the assessment of a permit application under a GBR (used in the UK for farms) may take as little as four days.

The legal specification of GBRs in the Industrial Emissions Directive allows using GBRs even for installations under integrated permitting regime.

The Article 6 specifies that: “EU Member States may include requirements for certain categories of installations, combustion plants, waste incineration plants or waste co-incineration plants in general binding rules. Where general binding rules are adopted, the permit may simply include a reference to such rules.”

For the project countries in particular, GBRs have benefits in terms of reduced workload for environmental authorities, emphasis on modern technique-based standards, consistency of regulation (especially useful where the technical capacity of permitting authorities is limited), and preventing opportunities for corruption through reduced discretion for the regulators.

At the same time, GBRs bring a number of disadvantages compared with site-specific integrated permits:

- GBRs are not as flexible as site-specific permits with individual conditions (e.g., they cannot easily take into account local environmental conditions)

- Public participation takes a different form, as permit conditions are not site-specific and the consultation occurs only at the GBR design stage, where the possibility of changes forced by the public is less than for individual permits

- The prescribed techniques are fixed until the GBR is reviewed, and permitting authorities can do little to impose further improvements

- GBRs may not fit well with the implementation of economic instruments of environmental protection, such as emissions trading. Those instruments, to have an incentive impact, require the operator to have some flexibility in establishing operating conditions, whereas GBRs specify conditions precisely.

## Scope of Application of GBRs

There are a number of practical criteria that should be met for the development of GBRs to be feasible:

- A GBR must cover a sufficient number of installations in a given sector for the resources used to develop it to be outweighed by the benefits from reduced effort on individual permit determinations. It is difficult to suggest specific thresholds for appropriate use of a GBR under this criterion, as in each particular country they will depend on the geographical distribution of such installations, their size, the capacity and costs of designing GBRs, etc.

- GBRs can cover all environmental aspects or they can be specific for individual media – e.g. air protection issues or waste issues.

- GBRs can only apply to well-defined categories of installations that use similar, widely accepted technologies that are unlikely to change rapidly. A GBR establishes standard requirements for technologies and techniques to be followed. In case GBRs need to be frequently revised in order to accommodate changes in technology, there is no advantage to their use. At the same time, a GBR may be an effective method for introducing technological improvement in a sector.

- Installations within each category subject to a GBR should have a relatively uniform impact on the environment. If the installations’ environmental impacts are largely site-specific (i.e., depend significantly on local environmental conditions), the imposition of standard conditions would not be feasible.

- It is important that the operators of installations targeted by a GBR are well organised so that their views are coherent and well expressed. GBRs will need to be developed in negotiation between the national environmental authority and the industrial sector’s representatives. An industry (trade) association can be a good partner in this case.

## Example of GBR practice

The Netherlands has extensive experience in the use of GBRs applying to all or some aspects of operation of installations. The rules contain an overall package of provisions and are issued by the national government, but inspection and enforcement is undertaken by local authorities. This type of regulatory approach has been positively received by both competent authorities and industry. It applies to several types of installations, including:

- Construction companies

- Dairy farms

- Crop farms

- Dry cleaning companies

- Petrol stations.

In addition to these, the following types of installations under threshold values of Annex I of IED may be considered for simplified permitting under GBRs:

- Combustion installations with rated thermal input no more than 50 MW

- Furnaces producing small quantities of pig iron, 2.5 tonnes per hour or less

- Ferrous metal foundries melting 4 tonnes per day or less

- Installations for the bulk storage, blending and mixing of cement

- Small scale electrolytic plating baths

- Ceramics manufacturing installations with production capacity below 75 tonnes per day

- Small scale municipal solid waste incinerators (with capacity of 3 t/h or less), and

- Other industrial installations out of the integrated permitting regime.

Furthermore, many other EU countries apply GBRs for large or medium polluting installations. The following table gives overview of sectors and countries where GBRs were used in 2007.

| No. | Sector | Country |
| --- | --- | --- |
| 1 | Intensive Livestock Farming | UK |
| 2 | Intensive Livestock Farming | Netherlands |
| 3 | Intensive Livestock Farming | Portugal |
| 4 | Intensive Livestock Farming | France |
| 5 | Pulp and paper | France |
| 6 | Pulp and paper | Germany |
| 7 | Pulp and paper | Belgium / Walloon region |
| 8 | Foundries | Slovenia |
| 9 | Foundries | Austria |
| 10 | Non-Ferrous metals | Germany |
| 11 | Refineries | Netherlands |
| 12 | Refineries | Belgium / Flanders |
| 13 | Slaughterhouses | Hungary |
| 14 | Slaughterhouses | Spain |
| 15 | Slaughterhouses | Slovakia |

This overview has been gathered within the project financed by EC which aim was to investigate which EU countries are applying GBRs. The results of this project and links to GBRs and the result are available on http://iris.eionet.europa.eu/gbr/.

## Development of GBRs

The Ministry of Environment (or its equivalent), in collaboration with ministries of industry, agriculture, and other concerned sectoral agencies, should identify categories of industrial activities where within each installation the same activities are carried out, or where there are few alternative methods of carrying out these activities and where the best practices are clearly identified.

The environment ministry, perhaps by contracting a specialised institute, would then produce a first draft GBR based on a number of sources:

- EU BREFs, where they are relevant to installations outside the scope of integrated permitting

- Existing industry standards of good practice published either by government bodies or by industry associations (where such standards are seen as appropriate), both domestically and internationally, and

- National statutory emission limit values.

GBRs should be comprehensive and address as far as possible all relevant aspects covered by integrated permitting (see Chapter II). In particular, they should address the sources of environmental impact and the techniques that should be used to minimise it. Generally, sector-specific GBRs should cover the following issues:

- Currently applied processes and techniques

- Current emissions and resource consumption levels

- Production and management techniques to be used in installations subject to that GBR

- Numerical limits for releases of particular substances (ELVs), where appropriate, and

- Self-monitoring and reporting requirements.

The GBR may include an application form tailored to fit the particular type of installations under consideration.

In project countries, in particular, it is important to distinguish between GBR requirements for existing and for new installations. GBR may set out “new plant standards” and incorporate upgrade requirements for existing installations, in which case they would act as a stimulus for improved environmental performance.

Box 1. Example of GBR Conditions: UK Guidance for Small Combustion Units
(<20 MW Thermal Input)

For small gas combustion units less 20 MW T.I. (in aggregate for multiple units) the following conditions should be set, unless there is a reason to set specific conditions (e.g., within an Air Quality Management Area).

- No limit values on air emissions

- For natural gas firing: a minimum monitoring requirement of once per year for NOx (mg/m3); O2 (%); CO (mg/m3), except that for very small units (< 3 MW T.I.) forming part of the aggregation the monitoring may be waived. Performance of such units would normally be managed through the requirement to adequately maintain plant.

- For oil firing as a standby fuel:

- Heavy fuel oil: a fuel sulphur limit of 3% (1% after 31.12.2002)

- Gas oil: a fuel sulphur limit of 0.2% (0.1% after 31.12.2007)

- For oil firing as primary fuel: as for (1), (2), (3) plus additional monitoring requirement of particulates.

- For Coal firing as primary fuel: as for (4) plus a limit on sulphur in the coal burnt of 1% by weight as certified by the supplier.

This note does not apply to any units that are burning waste as a fuel.

Source: UK Environment Agency

The draft GBR should be sent for comments to the stakeholder authorities (the ministry of industry, among others) and discussed with representatives of the industry concerned, and their comments should be taken into account. In fact, it is useful to involve industry representatives in the drafting process already in the early stages.

The production of a GBR should also include public consultation. However, the nature of such consultation is different from that for an individual permit. Comments on a draft GBR (at the national level) would most likely come from environmental NGOs. The draft GBR should be posted on the environment ministry’s website, and a notice to that effect published in a general distribution newspaper as well as in relevant industry journals. It is important that the process be seen as transparent by the general public. After public consultation, the GBR needs to be promulgated in a regulation (secondary legislation).

A key issue is whether a GBR is absolutely binding for the regulator and/or the operator. This should be clear in the statutory document that establishes the GBR. To be absolutely binding, the GBR must address the full range of technologies used within the given category of installations, and local environmental concerns should not be expected to raise a problem.

An alternative approach is to allow for an opt-out to the use of a GBR in favour of a full integrated permit. This might be initiated by the operator (e.g., when alternative techniques are preferred that are not addressed by the GBR) or by the regulator (e.g., to ensure that sensitive local environment is protected). If fully integrated permitting is undertaken, all the advantages (especially cost savings to the operator and the regulator) of a GBR would be lost. So it is not feasible to have a GBR for a category of installations, if their significant share would opt out. However, if opt-outs are allowed, the operator must not be able to seek exemption from individual GBR requirements and would have to follow the full integrated permitting procedure (which may well result in stricter permit conditions).

As techniques improve, the GBRs will need to be reviewed and amended using the same procedure as outlined above. A revised GBR must include an upgrade timetable for installations permitted under the old GBR. There can be no fixed review periods for GBRs, but they should not be revised more frequently than the term of permits issued under those rules (5-7 years). Amending GBRs could require considerable resources both from the national environmental authority and industry, as all respective permits would have to be reviewed as well. This is why this method of regulation is most suitable where techniques are likely to improve only slowly.

## Permitting Procedure under GBRs

The Ministry of Environment in each project country needs to assess if the use of GBRs is compatible with existing permitting procedure. In case it is possible to replace or amend the current requirements by GBRs, and the procedure functions well, then there is no need to change the permitting procedure. The introduction of GBRs will only modify the current procedure and requirements.

If it is not possible to apply GBRs in the current system, then it is proposed to replace the permitting procedure completely by the new one which will enable the implementation of GBRs. In case the GBRs cover more than one environmental media, then the procedure can be similar to the integrated permitting procedure.

In any case it is necessary to make analysis of the impact of introduction of GBRs from the legal, procedural, institutional and financial points of view in order to establish necessary prerequisites for smooth implementation.

### Proposal of new permitting procedure with the use of GBRs

The GBR-based permitting procedure should be developed and adopted by the Ministry of Environment. Its development should be accompanied by consultation with other stakeholder agencies that would take part in the permitting process and with groups representing the regulated industries.

The procedure should designate internal responsibilities and step-by-step actions of permitting authority staff as well as stipulate interactions with the applicant, statutory stakeholders, and the public. A simple standard application form should also be designed.

The developers of a permitting procedure must make sure that it does not come into conflict with any primary or secondary legislation. The permitting procedure should also complement and not contradict existing procedures for environmental assessment, building permit issuance, and compliance assurance (inspection) and enforcement. The basic procedure can be based on the steps outlined below (the description provides also indicative number of days for different tasks).

#### Pre-application Activities

It is important that not too much time be allotted to this stage. However, the operator may ask the permitting authority for a pre-application meeting to discuss any applicable rules and binding limits and issues to be addressed in the application.

#### Application

A permit application under a GBR (which may include a specific application form) serves to justify that the installation complies with all the requirements of the GBR. It should include the main items of an integrated permit application but to a lower degree of detail:

- Identification of the installation;

- Identification of the operator;

- Description of the installation’s activities;

- Operational and management techniques (to show that they conform to the specific GBR requirements);

- Emissions (to demonstrate compliance with statutory limits stipulated in the GBR);

- Environmental impacts (brief description or reference to the findings of an EIA if one has been performed for the installation); and

- Other relevant information.

There may be an administrative fee required to be paid with the application to cover the costs of processing it by the permitting authority.

#### Receipt and Initial Check of Application

The Designated Administrator (DA) at the permitting authority should check that the application has addressed all the required questions and open a Working File. Then the Responsible Official (RO) should look at the basic adequacy of the answers presented. For applications based on a GBR both checks should be fairly quick as the GBR determines a limited number of issues that should be addressed. These checks are intended to ensure only that an application meets at least minimum requirements before the determination process begins. It is in no sense a determination of whether to issue a permit or what conditions ought to apply. The initial check of the application should take no longer than 5 days.

If an application is found not to be valid at this stage, it should be returned immediately. The DA should attach a note to indicate where the application falls short of what is required.

Within 5 days of the application being deemed valid, the RO should decide if any major pieces of additional information are needed to ensure that the environmental quality standard will be complied with. The DA would advise the applicant in writing and give him 10 days to respond. If this information is not received, the application should be refused. Requests for additional information should only be made in exceptional cases, as the application should respond to the requirements clearly stated in the GBR.

#### Consultation

The consultation process in permitting under a GBR regime is generally confined to issues of local environmental quality (consultation with the local authority) and prior compliance record by the applicant (consultation with the environmental inspectorate and local public health authorities). These stakeholder authorities may have information that could help the permitting authority to judge whether the application is truthful and accurate. They may also comment on the past performance of the operator or on possible challenges to the environment in the general vicinity of the installation (e.g., the presence of other significant sources of pollution). However, comments on the technique to be used will not be relevant as they have been decided in the GBR.

Within 10 days from the date the application was deemed valid, the DA should send copies of the application to the stakeholder authorities with a cover letter specifying the inputs that would be helpful and asking to provide their responses within 15 days. There is normally no general public consultation for applications of GBR-governed installations, although eventually the permit itself should be put in the permitting authority’s permit register.

The DA should note all responses from the stakeholder authorities in the Working File and inform the RO. If a body fails to respond, the RO may use his judgment to decide either to seek such a response or proceed without it.

## Assessment of Application and Determination of Permit Conditions

- Determination

For a simple application that is in accordance with the GBR it is unlikely that it would need to be considered by a “permit team” at the permitting authority, as the different cross-media issues will have been addressed during the development of the GBR. However the RO may need to seek advice from other permitting authority colleagues where the application is for an installation in a sensitive location where compliance with the environmental quality standard is or maybe under threat.

For the application’s assessment it is recommended to develop a check-list for compliance with GBRs and local environmental quality standards.

There is no need for the permitting authority to consider the merits of any alternative techniques, as all this work has been done in designing the GBR. This will substantially reduce the permitting authority’s effort compared with integrated permitting. Usually, a GBR-based application should be assessed within 10 days of the receipt of the consultation responses.

- Issuance

Once the application has been considered for compliance with the GBR and there are no serious objections by permitting and stakeholder authorities during consultation, the permit should be written and signed by the RO. The DA would then send the permit to the operator and place a copy of it in the permit register. The effective date would usually be the same as requested in the application. The validity of the permit is recommended to be at least 5 years. The permit should be renewed under a simplified procedure if the original characteristics of the installations have not changed.

As the permit reflects the GBR, it is possible to include some of the rules as conditions or, simply refer to the GBR and thus produce a standard, highly simplified, permit for the sector. The permit should include numerical limits from the GBR (ELVs, limits on the use of water and/or other resources) and contain requirements to monitor and report the actual releases and any accidental discharges beyond these limits. Compliance with GBR-based permit conditions should be verified through regular environmental inspections which would, however, be much less frequent than those of large industrial installations.

- Refusal

If the application’s assessment (using the check-lists mentioned above) results show that further conditions are needed to protect local environmental quality, the RO may decide to refuse the application under the GBR and instruct the operator to submit a full integrated permit application. Such an opt-out by the permitting authority should only be possible if allowed by the applicable regulations and would require an approval by the head of the permitting authority’s permitting department.

If the application fails to show compliance with the GBR, the RO should refuse the application. The criteria for refusal include the following:

- The environmental impact would be unacceptable within the conditions specified in the GBR (a full integrated permitting process may be required)

- The operator’s proposals do not comply with specific GBR requirements, or

- It is apparent that the operator cannot comply with the permit conditions because his inability to demonstrate the appropriate management systems or competence.

If the application is refused, the DA should advise the applicant, noting the details and deadline for appeal, and copy this notice to the permit register and the stakeholder authorities, specifying the reasons for refusal.

- Appeal

The applicable regulations may provide that any person or body, including the applicant for a permit, can make an appeal to the national environmental authority or to an arbitration court either against a refusal to grant a permit or against a decision to grant a permit (on the grounds that the operator cannot comply with local environmental quality standards). However, there can be no appeal against specific conditions that are set by reference to the statutory GBR.

The regulation may specify that an appeal must be made within 30 days of the permitting authority’s decision on the permit and may require it to describe the grounds for the objection and the reasons, considerations, and arguments on which they are based and be accompanied by whatever documents the objector considers necessary.

The appeal can be against any condition issued by a permit; therefore the permit should not enter into force until the appeal is settled. The operator should be advised of this without delay.

## Timeline of the Procedure

It is good practice to set out a period within which the permitting authority will normally determine a valid application. The permitting authority would normally determine a valid GBR-based application within days of its submission. The following table illustrates the timeline for the simplified permitting process under a GBR. However, an appeal of the decision may more than double the length of the process.

| Period | Application review |
| --- | --- |
| 5th day | Initial check of application is completed by the DA and the RO* |
| 15th day | Application is accepted and delivered by DA to stakeholder authorities |
| 30th day | Consultation responses are received from stakeholder authorities |
| 40th day | Assessment of the application is completed by the RO |
| 45th day | Permit or refusal notice is issued |
| 75th day | Possible appeal(s) is received by [national environmental authority] against the decision |
| 105th day | Appeal(s) is determined |
| 110th day | Final decision is issued by the permitting authority to the applicant. End. |

* If significant additional information is needed, and the RO sends to the applicant a request that delays the process by up to 10 days.

## Transition to GBR’s permitting system

Although the simplification of permitting for MPAs will be less resource-intensive than the establishment of an integrated permitting system, it will also require certain legal and institutional adjustments that will take time. These transition issues are addressed in this section.

## Role of Single-Medium Permitting

As large industrial installations gradually convert to integrated permitting, the country’s national environmental authority will have to choose appropriate permitting regimes for the installations that are not covered by the integrated permitting system. While installations with intrinsically low impact can be transferred to a registration scheme fairly quickly, the introduction of GBRs will take time and may not be appropriate for a significant number of installations.

Therefore, for a number of years single-medium permitting will remain the default option for regulating MPAs that are either unsuitable for a GBR or scheduled to be covered by GBR at a later date.

However, once the development of all appropriate GBRs has been completed and a GBR scheme is fully operational, the national environmental authority may consider whether it would be feasible to choose one of the three options for regulation of economic sector in question:

- Incorporate these installations into the integrated permitting system. This may be appropriate for installations that affect more than one environmental medium but would require the development of technical guidance for them, which is a time-consuming and expensive process.

- Develop or adjust national generic statutory ELVs that would directly apply to installations not covered by any other permitting scheme, making them a simplified version of a GBR. However, they would necessarily cover only a limited range of polluting substances and, in the absence of technique and environmental quality considerations in permitting decisions for such installations, would fall short of ensuring a high level of environmental protection.

- Keep a certain number of MPAs under a single-medium permitting scheme. This would be feasible especially for installations that have an impact on only one environmental medium and that are technologically diverse (like medium-sized bricks production). In this case, single-medium permitting should be procedurally simplified, as it would at that point be used exclusively for selected categories of MPAs.

The first two options would mean a gradual phase-out of single-medium permitting in the country, while the third would retain it for rather limited use after the integrated permitting system has been fully established (which may take up to 15 years). Phasing out single-medium permitting may look attractive from the perspective of reducing the number of permitting schemes, but regulating the “remaining” installations under either integrated permitting or statutory ELVs will also have serious drawbacks, as mentioned above. On the other hand, keeping single-medium permitting as a regulatory option is likely to increase the administrative burden on the permitting authorities, as they would have to handle three different permitting processes: full integrated permitting, simplified GBR-based permitting, and single-medium permitting (this institutional issue is addressed below).

Ultimately, a decision on the fate of the single-medium permitting system will be based on the medium-specific impacts and sectoral distribution of the MPAs that were deemed unsuitable for GBR.

## Legal and Institutional Issues

As changes are made to the existing primary environmental legislation concerning the regulation of large pollution sources, it would also be appropriate to provide for simplified permitting regimes for other installations. The applicability criteria for each option (GBRs, and single-medium permits) should be laid out either in a separate law on permitting of MPAs (which must appropriately define this term for the purposes of environmental regulation), or in a section of a law on integrated permitting (if a country chooses to adopt one), and/or in amendments to other environmental laws. The same laws should be used to set transitional periods for the introduction of GBRs, stipulate general requirements for self-monitoring and reporting, and the terms of validity and revision of the respective authorisations.

The competent authority for permitting MPAs should also be designated in the legislation. It is likely to be different for different permitting schemes. For GBR-based and single-medium permitting, the permitting authority should be the same competent environmental authority (regional or, in some cases, national) that is responsible for integrated permitting so as to avoid the existence of parallel permitting authorities. Since GBRs are themselves products of a multi-stakeholder process, decisions on individual GBR-based permits should be made exclusively by permitting authority. Single-medium permitting should also be the prerogative of environmental authorities, as is currently the practice.

As for the organisation within the competent environmental authority itself, the task of handling of different permitting schemes for large and smaller installations favours the establishment of single-medium permitting departments within permitting authority. This would allow environmental authorities to pool human and technical resources and better organise the processing of permit applications. For example, while there may be different responsible officials for handling integrated, GBR-based, and single-medium permits, the support staff (designated administrators) may be shared. In cases where for political reasons the existing single-medium department structure will be kept, one of these departments would have to be made responsible for processing GBR-based permit applications.

## Timing of the Transition

The introduction of a simplified approach to environmental permitting is not likely to require significant expenditures by MPAs except where those enterprises are clearly out of date in their practices. These MPAs must be given sufficient time to enable operators to plan and prepare for any investment that may be required. Large industries have long investment planning cycles, but small industries may need as much time for financial reasons even if the technical issues are simpler.

The introduction of a GBR system will also require that national authorities prepare technical rules for a number of categories of installations. For each category of industrial installations identified to be suitable for GBR regulation (the identification process itself is likely to take several months), the development of a GBR is likely to take between six months and one year. Therefore, the entire process may take between 3 years for the first categories of MPAs and 10 years for the full intended coverage of the system. The priority in developing first GBRs should be given to industry categories with the biggest number of installations (to achieve the biggest reduction of administrative costs upfront) and with the strongest industry associations (to facilitate the GBR development process). Unless the GBRs differentiate the requirements for new and existing installations, existing ones should be given a grace period of up to three years to comply with the GBR requirements, depending on the sector (this grace period should be specified in the GBR itself).

- Implementing legislation recommendations for RegistraTion system

For enterprises with small (insignificant) environmental impact the registration can act as permission to perform their activities. So, the registration can be seen as simplified permit procedure. In this chapter the recommendations for such procedure are given.

  - Key features of registration system

The environmental regulation of installations with insignificant pollution potential is based on a simple registration of these installations’ operations by the registration authority. Such installations have no potential to cause significant pollution and registration can be done by local municipal or environmental authorities. The choice of registration authority will depend on each country’s institutional structure. Detailed recommendations for the registration authorities are elaborated under task 2.1.3.3 of the project.

In order to minimize the administrative burden for operators, the registration authorities should be decentralized. Usually an environmental committee of the city or rural district administration or a city or rural district branch of the central environment ministry is a body given the responsibility to perform registration. The following features are based on principles defined in the Concept for transformation of environmental permitting system under task 2.1.2.1:

- Activities with insignificant impact on environment shall be regulated under simple registration system

- The legislation shall designate administrative body for issuing the registration and setting requirements for operator

- Information and methodological support to the regulatory system shall be provided by the Ministry of Environment – on central level

- If necessary General Binding Rules shall be developed and applied for small polluting installations with insignificant impact on environment

  - Activities with insignificant impact on environment regulated by simple registration

Installations with so-called “intrinsically low impact” must by their very nature be incapable of producing significant pollution without having to rely on active pollution reduction measures. If such installation’s potential for pollution is low, whatever the circumstances, then specific regulation is not likely to add any value.

In case of registration, the information about such installations would be available to the environmental authorities at a minimal administrative cost. By contrast, exempting small polluting installations from regulation under environmental permitting law altogether may bear a risk that environmental authorities are unaware of their existence, which would leave significant scope for uncontrolled pollution.

The environmental permitting law or implementing legislation has to specify clear criteria for classification/determination of relevant installations. The criteria/thresholds can determine the amount and nature of emissions in general or provide specific values for different sectors or activities as well as include capacity thresholds. Example of such criteria is provided further in chapter 4.4. In case the criteria are sector specific, it is necessary to consult the representatives of this industrial sector in order to understand the actual and potential impact from installations and activities in question.

  - Designation of the administrative body for issuing the registration

The administrative body responsible for the registration procedure has to be designated in the environmental permitting law. The law shall contain a list of all main powers and functions of the registration authority. The following responsibilities and tasks are proposed for the registration authority for implementation of the registration system:

- Perform the registration procedure and establish relevant administrative mechanism within the authority

- Ensure the use of electronic registration system in order to enable national database of registered installations

- Cooperate with the superior body in order to have access to information and methodological support,

- Cooperate with the body responsible for GBRs’ development, in case GBRs are applicable to small polluting activities.

Detailed provisions for the registration procedure can be included in the implementing legislation.

  - requirements for operators

The operator should be able to demonstrate to the regulator that, given the nature of the installation’s activities, the criteria of insignificant environmental impact will be met without having to rely on a significant management effort. If the installation depends, for example, on abatement equipment (scrubbers, filters, etc.), it is unlikely that it can be treated as having only a low potential for impact as failure of these could clearly result in significant releases. It should be able to meet the criteria differentiating small polluting installations from medium polluting installations. Following are example criteria (mostly taken from the UK regulations) for determining small polluting installations:

• Air emissions: Releases of any particular substance from the whole installation into the air should not be significant. For example, the US EPA defines “minor” sources of air pollution (which usually do not require a permit) as those that have no toxic air emissions and emit less than 100 tonnes per year of non-toxic air pollutants. Similar criteria are used in the Czech Republic.

• Wastewater discharges: The installation should not release more than 20 m3 of treated wastewater on any one day into surface waters and no direct discharges into groundwater.

• Waste generation: The installation should not generate any hazardous waste or more than 1 tonne of non-hazardous solid waste per day, averaged over a year, and no more than 20 tonnes of solid waste on any one day. As a rule, the simplified permitting regime (registration) cannot be applied to companies having obligations in accordance with the End of Life Vehicles Directive (2000/53/EC) and the Waste Electrical and Electronic Equipment Directive (2012/19/EU).

• Energy consumption: The installation should not consume energy at a rate greater than 1 MW.

• Noise: The noise levels arising from processes and measured at the border of the installations should not exceed the existing noise level (both expressed as LAeq) by more than 3 dB.

• Odour: A low-impact installation should not have the potential to give rise to an offensive odour noticeable outside the premises where the installation is operated.

The above-listed criteria, or similar (they should be relatively simple and not include too many parameters), may be incorporated in environmental law or implementing legislation and used by the competent authority to determine whether a particular installation has an intrinsically low impact.

New small polluting installations should be required to submit an environmental registration form at the same time as they apply for an operating license. Existing small polluting installations that are currently required to obtain environmental permits should be notified by the competent authority that they no longer need to have a permit but have to submit a registration form. Existing installations that presently do not need a permit should be also registered.

The registration should not have a validity limitation, but the operator should be required to notify the competent authority of any changes to the installation’s activities or their cessation. If the installation does not meet some criteria for intrinsically low impact, the official should notify the operator about a need to apply for an environmental permit without which it cannot continue to operate the installation.

  - Information and methodological support of the registration system on central level

The registration system has little requirements for methodological support and information, nevertheless some basic information and guidance is needed. The support shall be provided on a central level by the Ministry of Environment and its regional offices where relevant.

Functioning of the registration system requires the following information and tools:

- Legal provisions for the registration procedure and related issues, criteria for small polluting installations, requirements for operators, duties of registration authorities and possible sanctions

- Electronic database for registration of small polluting installations

- Registration form

- Check list for verification of the filled registration forms.

The registration form for small polluting installations has to be very simple, while making specific reference to the regulation authorising such registration. It should normally include the following:

  - Name and address of the operator;

  - Location of the installation;

  - Brief description of activities carried on it;

  - The nature and amount of any polluting releases from the activities (solid, liquid or gaseous) and a statement that they comply with the criteria for intrinsically low impact;

  - The maximum rate at which energy is used by the activities carried on;

  - A statement that no offensive odour from its activities is present outside the installation; and

  - A statement that noise levels outside the installation arising from the activities do not increase background levels by more than 3 dB Leq.

The example of the Registration form for small polluting activities is presented at Annex 4 to this report.

The methodological support of the registration system will be necessary in case some of the small polluting installations will be required to apply GBRs.

The development of GBRs can be ensured by the same organisation as for the GBRs for medium or big polluting activities or e.g. the BAT support body. Details about developing GBRs are listed above in chapter 3. It is necessary to have the GBRs ready before the start of issuing the registrations. The registration authority needs to know which GBRs are applicable to which type of operation in order to advise to the applicants on the GBRs relevant for their operations.

  - registration procedure

The registration procedure has to be outlined in the environmental permitting law and details can be specified either in the implementing legislation or some procedural steps can be applied based on the administrative code.

A registration officer of the competent environmental or municipal authority should check completeness of the application. If some information is missing, the form should be returned immediately indicating in written where additional information is required. If the form is complete, the official should decide whether it the installation meets all criteria for intrinsically low impact. In some cases the simplified public consultation can be held.

In case of a positive conclusion, the officer should make a respective record in an appropriate database. If the registration is managed by the municipal authority, there should be a procedure to share the information from this database with the relevant environmental authority.

The competent environmental authority (CEA) can choose to occasionally inspect selected registered installations to ensure that they still may be classified as having intrinsically low impact.

# Annex 1. Form for integrated permit application

The application form is elaborated based on

- The recommendations for environmental permitting law (task 2.1.2.3)

- Directive 2010/75/EU of on industrial emissions (part about integrated pollution prevention and control), and

- Experience with the use of application forms in the Czech Republic, Slovakia, England and Wales, Croatia.

Application for integrated permit

A. Data on the company

1. Basic data

| 1.1. | Name of economic entity |  |  |
| --- | --- | --- | --- |
| 1.2. | Legal form of the company |  |  |
| 1.3. | Type of request | New installation |  |
|  |  | Existing installation |  |
|  |  | Substantial modification of installation |  |
|  |  | Shut down of installation |  |
| 1.4. | Address of the economic entity |  |  |
| 1.5. | Postal address if different from 1.4. |  |  |
| 1.6. | E-mail and web address |  |  |
| 1.8. | Economic entity’s company registration number |  |  |
| 1.9. | Contact person, name |  |  |
| 1.10. | Contact person, position |  |  |
| 1.11. | Contact person, phone |  |  |
| 1.12. | Contact person, e-mail |  |  |

2. Data on the installation

| 2.1. | Name of installation |  |
| --- | --- | --- |
| 2.2. | Address of installation |  |
| 2.3. | Address of installation site if different from line 2.2 |  |
| 2.4. | Number of employees |  |
| 2.5. | Date of start-up and shut-down of installation operation, if planned |  |
| 2.6. | Is the installation subject to derogation from BAT guidance? |  |
| 2.7. | Main installation activity pursuant to Annex I | Main unit capacity |
| 2.8. | Other activities pursuant to Annex I | Other units capacities |
| 1. |  |  |
| 2. |  |  |
| 3. |  |  |

3. Additional information on the installation

| 3.1. | Environmental impact assessment carried out |  |
| --- | --- | --- |
|  | NO |  |
|  | YES |  |
|  | Date: |  |
|  | Document no: |  |
| 3.2. | Are there significant transboundary effects on another state? |  |
|  | NO |  |
|  | YES |  |
|  | Document No. |  |

4. Data related to modification of existing integrated environmental protection requirements

| 4.1. | Type of modification proposed and reasons for the modification |  |
| --- | --- | --- |

5. Protected data

Protected data (business secret) have to be marked by a green colour or printed on green paper.

| No. | Protected data | Chapter no and page No. in the Application | Reasons why the data are considered as protected/confidential |
| --- | --- | --- | --- |

B. Environmental management system

| Implemented EMS according ISO 14001 or EMAS and certified/verified | YES | NO |
| --- | --- | --- |
| Implemented EMS according ISO 14001 or EMAS without certification/verification | YES | NO |
| No EMS is implemented | YES | NO |
| List of relevant internal documents related to environmental management |  |  |

C. Data related to the installation and its location

1. Basic data on location

| Local and regional self-government unit |  |
| --- | --- |
| Cadastral municipality |  |
| Cadastral parcels |  |
| Valid documents on physical planning |  |
| List distances in meters from closest settlements, neighbours, water recipients, water surfaces, forests, protected and other sensitive areas |  |

2. Maps and schemes

| No. | Name of map | Mark out | Annex No. |
| --- | --- | --- | --- |
| 1. | Excerpt from the Ecological Network |  |  |
| 2 | Ortho-photo map / wide surrounding | Location of the installation, closest settlements, neighbours, water recipients, water surfaces, forests, protected and other sensitive areas |  |
| 3 | Layout / situation plan of the installation | All emission and discharge points and technological units |  |
| 4 | Block diagram / technological scheme | Technological units according to chapter 3.1 -3.3. with material/energy flows, possibly also all emission points |  |

3. Description of the installation

3.1 Technological unit in which main activity according to national list of main polluting installations is carried out

| Name of the unit |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Name of technological sub-unit | Capacity | Short technical description | Reference code from the layout / block diagram in Annex No. |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

3.2 Technological units in which other activities according to national list of main polluting installations is carried out

| Name of the unit |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Name of technological sub-unit | Capacity | Short technical description | Reference code from the layout / block diagram in Annex No. |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Remark: If more other activities according to national list of main polluting installations are in the facility, use this table appropriately.

3.3 Technological units out of Annex 1 (directly associated activities)

| No. | Name of technological unit | Capacity | Technical description | Reference code from the block diagram in Annex No. |
| --- | --- | --- | --- | --- |

3.4 The main alternatives to the proposed technology, techniques and measures studied by the applicant in outline

| No. | Name of technological unit | Alternative description |
| --- | --- | --- |

4. Reference codes of emission points (prefix A for air; V for water (receiving area); S for emissions to soil, K for public sewage system, O for odour) shown in the layout/block diagram of the installation

| Code | Emission point | Coordinates* |  | Description | Annex No. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | X | Y |  |  |  |

* According to the National Coordinate Reference System

D. List of raw materials, secondary raw materials and other substances and energy consumed or produced during installation operation

1. Raw materials, secondary raw materials and other substances used in the installation

1.1. List of raw materials, supplementary materials and other substances without dangerous substances

| No. | Technological sub-unit | Raw materials, secondary raw materials and other substances | Description and characteristics | Annual consumption (t) | Annual consumption per unit of production (t/unit) |
| --- | --- | --- | --- | --- | --- |

1.2. List of dangerous substances / chemicals

| Name | Composition | CAS | Characteristics | Purpose | Annual consumption (t) | The total storage quantity (t) |
| --- | --- | --- | --- | --- | --- | --- |

1.3. Water

| 1.3.1 |  | Water intake source and the official code of the connection point | Use in installation operations | Consumption of technological and drinking water |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  |  |  |  | Ø (l/s) | max (l/s) | m3/year | Consumption per unit of production (m3/unit) |  |
| 1.3.2 | Intake description, consumption of surface water, ground water and used water for reuse, input water quality, treatment of water taken in |  |  |  |  |  |  |

1.4. Storage of raw materials and other substances

| No. | Storage areas, temporary storage, handling raw materials, products and waste | Capacity | Technical description | Reference code from the block diagram/ layout in Annex No. |
| --- | --- | --- | --- | --- |

1.5. Description of existing and planned methods for minimising consumption and better use of raw materials, secondary raw materials, other substances and water

2. Products and semi-products produced in the installation

2.1. Products and semi-products

| No. | Installation | Product and semi-product | Product and semi-product description | Registry number of substance (CAS) | Production (t/year) |
| --- | --- | --- | --- | --- | --- |

3. Energy consumed or produced in the installation

3.1. Fuel and energy input

| 3.1.1. | Fuel and energy input | Consumption unit/year | Thermal value (GJ/unit) | Transformed into GJ |
| --- | --- | --- | --- | --- |
| 3.1.2. | Fuel: |  |  |  |
| 3.1.3. | Secondary energy |  |  |  |
| 3.1.4. | Renewable resources |  |  |  |
| 3.1.5. | Bought thermal energy |  |  |  |
| 3.1.6. | Bought electric energy |  |  |  |
| 3.1.7. | Other fuels |  |  |  |
| 3.1.8. | Total input energy and fuel quantities in GJ |  |  |  |

3.2. Energy manufactured in the installation

|  |  | Unit | Amount |
| --- | --- | --- | --- |
| 3.2.1. | Installed electrical power | MW |  |
| 3.2.2. | Installed thermal power | MW |  |
| 3.2.3. | Production of electrical energy | MWh |  |
| 3.2.4. |  | GJ |  |
| 3.2.5. | Production of thermal energy | GJ |  |
| 3.2.6. | Sale of thermal energy | GJ |  |
| 3.2.7. | Sale of produced electrical energy | MWh |  |
| 3.2.8. |  | GJ |  |

3.3. Energy consumption

|  |  | Unit | Amount |
| --- | --- | --- | --- |
| 3.3.2. | Total bought and produced energy | GJ |  |
| 3.3.3. | Total sold energy | GJ |  |
| 3.3.4. | Total energy consumption | GJ |  |
| 3.3.5. | Total energy consumption for heating and hot water from the heating system | GJ |  |
| 3.3.6. | Total energy consumption for technological and other processes | GJ |  |

3.4. Energy consumption per unit of production

| No. | Product | Unit | Energy consumption/ unit of production |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Electrical energy |  | Thermal energy GJ/unit | Total energy GJ/unit |
|  |  |  | KWh/unit | GJ/unit |  |  |

3.5. Description of existing and planned methods for improving energy efficiency

E. Description of type and quantity of predicted emissions from the installation into any media and establishment of significant consequences of emissions on the environment and human health

1. Emissions to air

1.1. List of sources and points of emission into the air, including measures for preventing emissions (listed according to the technological unit according to national list of main polluting installations or directly associated activities)

| Technological unit or directly associated activity | Production level | Emission point No. | Emission point description | Reference code from the layout /block diagram in Annex No. | Emission point parameters |  | Coordinates on scheme |  | Monitoring point and method | Gas flow characteristics |  |  | CAS | Substance | Method for reducing emissions | Emission data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | height, m | diameter, m | Х, m | Y, m |  | volume flow rate m3/s | velocity, m/с | temperature, 0С |  |  |  | mg/m³ | g/s | kg/h | t/year | g/production unit |

1.2. Description of existing and planned methods for preventing/reducing emissions, their efficiency and environmental impact

1.3. List of control technologies or equipment

| Technological unit or directly associated activity | Emission point No. | Reference code from the layout /block diagram in Annex No. | CAS | Substance | Before control |  | Control efficiency, % | After control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | volume flow rate, m3/s | concentration, mg/m3 |  | volume flow rate, m3/s | concentration, mg/m3 |

1.4. List of uncontrolled technological emissions

| Technological unit or directly associated activity | Emission point No. | Reference code from the layout /block diagram in Annex No. | CAS | Substance | Concentration, mg/m3 | Frequency of emissions per year | Period, minutes | Emission data |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | g/s | kg/h | t/year |

2. Emissions to water

2.1. Point of discharge into the natural recipient/surface waters

| 2.1.1. | Name of the receiving area (river, lake, sea) |  |
| --- | --- | --- |
| 2.1.2. | Position of the point of discharge in relation to the receiving area, Provide short description of technical solution of discharge point |  |
| 2.1.3. | Zone of sanitary protection of the drinking water well/source |  |

2.1.1. List of water pollution indicators

| Reference code from the block diagram/layout in Annex No. | Site of waste water generation and type of water | Total daily quantity (m³/day), total yearly quantity (m³/year), and flow (m³/h | Pollutants | Before treatment |  | After treatment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Treatment method | Concentration (mg/l) | Concentration (mg/l) | Annual emissions (t) | Emission/ product unit (mg/l ∙unit) |

2.2. Point of discharge into the public or internal/private sewage system

| 2.2.1. | Name of the receiving sewage system |  |
| --- | --- | --- |
| 2.2.2. | Position of the point of discharge in relation to the sewage system |  |

2.2.1. List of water pollution indicators

| Reference code from the block diagram/layout in Annex No. | Site of waste water generation and type of water | Total daily quantity (m³/day), total yearly quantity (m³/year), and flow (m³/h | Pollutants | Before treatment |  | After treatment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Treatment method | Concentration (mg/l) | Concentration (mg/l) | Annual emissions (t) | Emission/ product unit (mg/l ∙unit) |

2.3 Description of existing and planned methods for preventing/reducing emissions

3. Emissions to soil

3.1. Type and characteristic of emissions to soil

| Code of point of emission to soil | Sites of generation of emissions to soil | Type of emission | Pollutants | Total daily quantity (unit) | Before treatment (unit) | After treatment (unit) | Annual load to soil (unit/year) |
| --- | --- | --- | --- | --- | --- | --- | --- |

3.1.1. Description of existing and planned methods for preventing/reducing emissions to soil

| No. | Description of existing and planned methods for preventing/reducing emissions to soil |
| --- | --- |

3.2. Emission to soil related to agricultural activities

3.2.1. Manure

| Type of manure produced |  |
| --- | --- |

3.2.1.1. Content of manure

| Parameter | Amount |  |
| --- | --- | --- |
| pH | - |  |
|  | % | kg/t |
| Dry matter content |  |  |
| Content of total N |  |  |
| Content of ammonia N (NH4) |  |  |
| Phosphorus content (P2O5) |  |  |
| Potassium content (K2O) |  |  |
| pH |  |  |

3.2.2. Application to land

| Type of manure | Yearly amount of produced manure (t) | Total amount of N in manure (kg) | Available land for land spreading (ha) | Applied amount of N to land (kg/ha) |
| --- | --- | --- | --- | --- |

3.2.3 List of parcels and contracts for manure application

|  | Annex No. |
| --- | --- |
| List of available parcels and their surfaces |  |
| Contract with other entity/person for application of manure ( in case of insufficient land area) |  |
| Contract with other entity for offsite manure treatment |  |

3.2.4. Description of existing and planned methods for preventing/reducing emissions to soil

4. Waste management

4.1. Name and quantity of produced hazardous waste

| Key waste number | Waste name | Annual quantity of produced waste (t) | Annual quantity of recovered waste (t) | Waste recovery procedure | Annual quantity of disposed waste (t) | Waste disposal procedure | Waste storage on the location reference– code from layout/block diagram |
| --- | --- | --- | --- | --- | --- | --- | --- |

4.2 Description of existing and planned methods for waste production prevention

5. Noise

| 5.1. | Noise sources | Source description |  | Level of sound pressure at source LWA (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  |  |  |  |  |  |  |
| 5.2. | Value of equivalent noise level LAeq in dB in monitored areas |  |  |  |  |  |  |
| No. | Measurement location |  | Day |  |  | Night |  |
|  |  |  | Highest permitted value |  | Measured value | Highest permitted value | Measured value |

6. Vibrations

| 6.1. | Vibration source | Description of vibration source |  | Values of assessed vibration acceleration, aweq,T (msˉ²) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. |  |  |  |  |  |  |
| 6.2. | Values of assessed vibration acceleration which is caused by the installation in monitored area, aweq,T (msˉ²) |  |  |  |  |  |
| No. | Measurement location | Day |  |  | Night |  |
|  |  | Highest permitted value | Measured value |  | Highest permitted value | Measured value |

F. Description and characteristics of the environment at the installation site

1. Characteristics of wider surrounding area

| Have the concentrations of significant substances which are emitted into air, water and soil (including ground water) been measured/ or modelled in the environment and the level of noise and vibrations determined? Specify report reference number |  |  |
| --- | --- | --- |
| Substance | Description of findings | Reference to the report/study |

2. Previous pollution and measurements for improving the environmental status

| No. | Description | Document No. |
| --- | --- | --- |

G. Description and characteristics of existing or planned measures and equipment used for supervising the installation and environmental emissions

1. Existing system of measures and technical equipment for supervising environmental emissions

| 1.1. | Supervised emission |  |
| --- | --- | --- |
| 1.1.1. | Emission point |  |
| 1..13. | Measurement/sampling site |  |
| 1.1.4. | Measurement /sampling methodology |  |
| 1.1.5. | Measurement frequency |  |
| 1.1.6. | Measurement/sampling conditions |  |
| 1.1.7. | Monitored parameters |  |
| 1.1.8. | Analytical methodology |  |
| 1.1.10. | Entity which performs measurement/sampling |  |
| 1.1.11. | Organisation which performs analysis/laboratory |  |
| 1.1.12. | Authorisation/accreditation for measurements or laboratory authorisation/accreditation |  |
| 1.1.13. | Evaluation of measurement results |  |
| 1.1.14. | Method of registering and storing data |  |
| 1.1.15. | Planned supervision changes |  |
| 1.1.16. | Is the environmental status supervised? |  |

2. Planned system and technical equipment for the supervision of the installation and environmental emissions

Note: use table from item 1 above appropriately

3. Environmental monitoring

3.1. Environmental components that are monitored

Note: use table from item 1 above appropriately

4. Additional indicators/parameters which are being controlled by the operator

| No. | Indicator/parameter | Short description |
| --- | --- | --- |

H. Detailed analysis of the installation in relation to best available techniques (BAT)

1. List of used BAT guidance (e.g. BAT reference documents (BREFs)/ BAT conclusion)

| BREF Document/BAT conclusion | Abbreviation | Issued (date) |
| --- | --- | --- |
| For main activity according to national list of main polluting installations |  |  |
| For other activities according to national list of main polluting installations |  |  |

2. General comparison with BAT

| Chapter in BREF document/BAT conclusion | No. of BAT technique | BAT according to the BREF document/ BAT conclusion (description) | Current/planned application in installation | Compliance |  | Justification (explanation) of non-compliance. Propose a plan for undertaking measures and time framework for the compliance with BAT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | YES | NO |  |

3. Analysis of emission parameters of the installation in relation to BAT

3.1. Emission to air

| Chapter in BREF document/ BAT conclusion | No. of BAT technique | BAT associated ELVs | Achieved/ planned emission limit values | Compliance |  | ELV according to national legislation | Justification (explanation) of the difference between the range of emissions while using BAT and achieved emissions. Propose a plan for undertaking measures and time framework for the achievement of levels equal to the ones achieved while using BAT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | YES | NO |  |  |

3.2. Emission to water

Note: use table from item 3.1 above appropriately

3.3. Emission to soil

Note: use table from item 3.1 above appropriately

I. List of measures to be undertaken after shutdown of the installation, for the purpose of preventing any risks of pollution or avoiding threats for human health and remediation of the installation location

List of measures to be undertaken after shutdown of the installation Description of the proposed decommissioning programme for the installation or proposal for the preparation of such a programme

Results of the location examination in relation to existing pollution of soil and ground water from the installation, or proposal for the performance of such examination, and proposed timeframe (Baseline report list as an Annex)

J. Identification of participants in the process and other entities for which the economic entity which manages the installation knows that they might be exposed to adverse effects if the existing or new installation has transboundary effects

Fill in only in case of transboundary effect!

List of participants

K. Statement

I hereby confirm having prepared this request for the issuing of a unique/ modified unique permit.

I hereby confirm the accuracy, correctness and completeness of data.

The body which issues permits or local administration bodies are allowed to submit copies of this request or parts thereof to other persons.

Signature:                                          Date: _________________

(Company representative)

Signatory’s name: __________________

Position in the company: __________________

Company seal:

Signature:                                                                                   Date: _________________

(Author of the application)

Signatory’s name: __________________

Position in the company: __________________

Company seal:

L. Abbreviations and symbols

| Abbreviation/ symbol | Description |
| --- | --- |

M. Annexes

|  | List of annexes |  |  |  | Annex No. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Non-technical summary |  |  |  |  |  |
|  | Confirmation from the court register for legal persons, confirmation from the register for physical persons for non-legal persons (e.g. farmers) |  |  |  |  |  |
|  | Confirmation from the cadastre (land registry books) for the area where the installation, for which the permit is being issued, is/will be located |  |  |  |  |  |
|  | List of basic data on all operational permits in the following format: No.; Name of the permit; Date of issue; Number of permit; Not issued. |  |  |
|  | Decisions and opinions on environmental components issued prior to submitting the application |  |  |  |  |  |
|  | Type of approval, permit, decision, etc., body competent for issuing | Date of issuing | Valid until | Document number |  |  |
|  | Excerpt from the Ecological Network |  |  |  |  |  |
|  | Orto-photo map showing the location of the installation and the surrounding area |  |  |  |  |  |
|  | Layout /block diagram of the installation with marked buildings and emission points and /or block diagram of the process with marked emission points |  |  |  |  |  |
|  | Block diagram / technological scheme |  |  |  |  |  |
|  | Situation (layout) of internal waste water sewage/drainage system (showing all water buildings for drainage and waste water treatment and location of all discharge points) |  |  |  |  |  |
|  | Plan for operation and maintenance of waste water drainage and treatment buildings |  |  |  |  |  |
|  | Operational plan for intervention measures in case of extraordinary and sudden water pollution |  |  |  |  |  |
|  | Data on impermeability of internal water drainage system and accompanying buildings and facilities. |  |  |  |  |  |
|  | If applicable, contract with the utility company on acceptance of appropriate waste water discharges, |  |  |  |  |  |
|  | Contract with other entity/person for application of manure |  |  |  |  |  |
|  | Contract with other entity for off-site manure treatment |  |  |  |  |  |
|  | Other enclosed documents have been prepared for the purpose of submitting the request |  |  |  |  |  |

N. Proposal of integrated permit conditions

1. Emission limit values

1.1. Emissions to air

| Emission limits for emissions to air |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emission point indication | Substance or other relevant indicator | Emission limit | Unit | Reference conditions | Notes |

1.2. Emissions to water

| Effluent emission limits for releases to water and sea |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emission point indication | Substance or other relevant indicator | Emission limit | Unit | Reference conditions | Notes |

1.3. Emissions to public drainage

| Effluent emission limits for releases to drainage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emission point indication | Substance or other relevant indicator | Emission limit | Unit | Reference conditions | Notes |

1.4. Noise emissions

| Emission limits for outside noise |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emission point indication | Substance or other relevant indicator | Emission limit | Unit | Reference conditions | Notes |

1.5. Emissions to soil

| Emission limits for releases to soil |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Emission point indication | Substance or other relevant indicator | Emission limit | Unit | Reference conditions | Notes |

2. Emission monitoring requirements specifying measurement methodology, frequency and evaluation procedure

| Emission point indication | Substance | Monitoring frequency | Monitoring method |
| --- | --- | --- | --- |

3. Waste management (list of hazardous waste generated on site,)

4. Process techniques (with defined normal and other than normal operating conditions such as start-up and shut-down operations, leaks, malfunctions, momentary stoppages and definitive cessation of operation

5. Implemented prevention and control techniques (including regular maintenance and surveillance of measures taken to prevent emissions to soil, water, measures concerning monitoring and management of waste generated and monitoring of hazardous substances

| No. | Technique |
| --- | --- |
| 1 |  |
| 2 |  |

6.	Improvement programme (planned prevention and control techniques in order to comply with BAT guidance or improve the current environmental performance)

| Reference | Requirement | Chapter in BAT guidance | Date |
| --- | --- | --- | --- |

7. Measures in the case of installation decommissioning

8. Condition outside the installation (if required, e.g. nature protection)

Guidelines on the extent and manner of completion of the new application form for integrated permit

Definition of an installation

‘Installation’ means a stationary technical unit within which one or more activities listed in Annex I of the law on Environmental Permitting are carried out, and any other directly associated activities on the same site which have a technical connection with the activities listed in the Annex and which could have an effect on emissions and pollution.

General comment on the completion of the application

1. With respect to all numerical figures contained in the application, their source should be always indicated (company bookkeeping, company information system, records of analyses, reports on measurement) and the manner of ascertainment of values (measured, ascertained by calculation, etc.).

2. The operator of the installation shall complete the application first in electronic format. This format will be used by the operator subsequently as the basis for the preparation of the application’s hard copy. The operator shall complete the application to the extent corresponding to the nature of the installation that is to be permitted. The operator of the installation that has been already previously operated shall complete the required data pertaining to a given installation for the last closed period of one year. The operator of the installation which is prior to the commencement of future construction or is already under construction, shall complete the data based on projected values or anticipated data.

4. When preparing individual chapters of the application, the comments on selected chapters of the application given below should be followed. The numbering of chapters, subchapters and annexes to the application cannot be changed.

5. The data required in individual chapters of the application form should be completed on consistent basis, i.e. information submitted in chapters C to E should be mutually consistent and shall provide justification for the BAT comparison in chapter I and for binding conditions proposed in chapter N.

Comments on the completion of selected chapters of the application

The numbering of comments is consistent with the reference numbers of individual chapters/points in the new application form.

A. Data on the company

2.6 Fill in “YES” only if this application contains request for derogation from BAT guidance or if already approved derogation is still valid.

2.7 Main installation activity pursuant to Annex I and 2.13 other activities pursuant to Annex I

To clearly define the main activity in case of a plant with more activities according to national list of main polluting installations is very important, since the review period of the permit should be based on the update of the BAT guidance for the main activity.

Example: In a sugar mill:

- The main activity is sugar production (Activity 6.4. (b) Treatment and processing, other than exclusively packaging, of the following raw materials, whether previously processed or unprocessed, intended for the production of food or feed from: (ii) only vegetable raw materials with a finished product production capacity greater than 300 tonnes per day or 600 tonnes per day where the installation operates for a period of no more than 90 consecutive days in any year)

- The other activities are (for example) - energy production (Activity 1.1. Combustion of fuels in installations with a total rated thermal input of 50 MW or more) - and lime production (Activity 3.1. Production of cement, lime and magnesium oxide: (b) production of lime in kilns with a production capacity exceeding 50 tonnes per day).

In an installation with just one activity is this activity always considered as the main one?

5. Protected data - protected data have to be marked by a green marker or printed on light green paper. It is necessary to provide justification why some data should be protected. Any data about pollution and environment status cannot be protected. The justification should be in a separate document sent together with the application.

B. Environmental management system

Put codes of internal documentation which is relevant to the environmental protection, as for example:

- Environmental policy

- Environmental aspects and their monitoring

- Environmental management programmes

- Accidental plan, emergency preparedness and response

- Evacuation plan

- Safety report.

C. Data related to the installation and its location

2. Maps and schemes

Points 1, 2 and 3 (Excerpt from the Ecological Network, Ortho-photo map / wide surrounding and Layout / situation plan of the installation) are obligatory. Point 4 (Block diagram / technological scheme) list only in case of complex technological installation (e.g. not in case of chicken/pig farms).

3. Description of the installation

3.1 Technological unit where main activity according to national list of main polluting installations is carried out

The main activity and its total capacity are defined under point 2.12. Here you are asked to divide this unit into sub-units and define the partial capacities. Remark: Sub-unit of the main unit is part of the technology which creates indivisible part of the unit. Technological unit where the main activity according to national list of main polluting installations is carried out is subject of comparison with BAT.

3.2 Technological units where other activities according to national list of main polluting installations is carried out

The other activities and their total capacities are defined under point 2.13. Here you are asked to divide these units into sub-units and define the partial capacity. Remark: Sub-unit of the main unit is part of the technology which creates indivisible part of the unit. If more other activities according to national list of main polluting installations are in the installation, use this table appropriately. Technological units with other activities according to national list of main polluting installations are subject of comparison with BAT.

3.3 Technological units out of Annex 1 (directly associated activities)

Here list all other technological units which are not subject of previous points 3.1 and 3.2., and are operated by the same operator on the same site, e.g. production lines, packaging lines, waste water treatment plant, boiler room (if the total capacity of boilers is under 50 MW), etc.

Remark: Chapters 3.1 – 3.3 must sufficiently describe the installation from the technical point of view. These chapters will be transformed into the integrated permit. No Technical-Technological Solution will be enclosed to the permit anymore. It means that chapters 2.1 – 2.3 must be brief, but comprehensive.

3.4 The main alternatives to the proposed technology, techniques and measures studied by the applicant in outline

Relevant only for new or reconstructed installation for technological units according to national list of main polluting installations, if alternative solution was assessed.

4. Reference codes of emission points (prefix A for air; V for water (receiving area); S for emissions to soil, K for public sewage system) shown in the layout/block diagram of the installation

A - air – all emission points according to the air law (including raw material storage house, if they are air emission points at the same time)

V - water - all points of discharging the waste/rain waters into the natural recipients

S - soil – pit holes for waste/rain waters infiltration and any other soil emission points

K – water - all points of discharging the waste/rain waters into the public sewage system

D. List of raw materials, secondary raw materials and other substances and energy consumed or produced during installation operation

1.1. List of raw materials, supplementary materials and other substances without dangerous substances

Fill in only bulk materials, which are used in amount more than 1 t per year.

1.2. List of dangerous substances / chemicals

Fill in materials with dangerous characteristics according to law on chemicals, which are used in the amount more than 100 kg per year. Fill in all highly toxic substances regardless the amount used. Put HG, H and P phrase according to Regulation EC No 1272/2008 on classification, labelling and packaging of substances and mixtures.

1.4. Storage of raw materials and other substances

Do not list communal waste storages as well as individual bins.

2. Products and semi-products produced in the installation

It is important to define the “main product” or “main products” for every technological unit according to annex 1. The amount of the “main product/s” will be used for calculation of Annual consumption per unit of production in Table D 1.1, D 1.3 and D 3.4. (These figures serve for BAT comparison.)

Semi-product means material, which is produced in the installation, is inter-stored there and consequently utilised. Describe dangerous characteristics of products and semi-products, if applicable.

3. Energy consumed or produced in the installation

3.1. Fuel and energy input – if more fuels are utilised (Natural gas, Brown coal, Black coal, Coax, Other solid fuels, Crude oil (fuel oil), Gas oil, Heating oil, Other gases, Diesel fuel), add new rows,

List just fuel used for stationary units, not for mobile units (vehicles).

For thermal value use figures from the national register.

In the table 3.3 calculate the values from previous tables 3.1 and 3.2 as follows:

3.3.2. Total bought and produced energy = (3.1.5. + 3.1.6.) + (3.2.4.+3.2.5.)

3.3.3. Total sold energy = 3.2.6. + 3.2.8.

3.3.4. Total energy consumption = (3.1.5. + 3.2.5. - 3.2.6.) + (3.1.6. + 3.2.4. - 3.2.8.) + (3.3.5. + 3.3.6.)

3.4 Energy consumption for unit of production – calculate only for main products, and take into consideration only energy for technological purposes on relevant technological unit – figures 3.3.6. (These figures serve for BAT comparison.)

General comment to chapter D: It is not sufficient to produce the material and energy inventory check just once. It is important to make the material/energy balances/audits regularly and to use the results for preventive measures proposal (cleaner production approach). This is one of the basic Bat’s.

E. Description of type and quantity of predicted emissions from the installation into any media and establishment of significant consequences of emissions on the environment and human health

For all emissions mentioned in this application it is necessary to state periods in which sampling is performed and for which averages are calculated, as well as emission variations, e.g. where data are available – half hourly averages, daily averages, monthly averages, mass emissions and emissions per tonne of product.

Consequence of pollutant emission fill only in case, that corresponding study was undertaken.

1. Air pollution

Description of all air pollution sources and points which were appointed under point C3.

2. Water pollution

Description of all water pollution sources and points which were appointed under point C3 including discharged water quality. Water quality before the treatment - fill in only if observed.

2.1.1 and 2.2.1 Type of water = sanitary, technological or storm water

3. Soil pollution

Fill in only if relevant.

3.1 and 3.1.1 fill in for industrial soil pollution sources, not for agriculture (landspreading of manure, fertilisation).

3.2. Fill in only for agricultural activities, if applicable

3.2.1.1. Fill in only if such analysis were carried out.

3.2.2. Fill in the case, if the operator of the installation ensures application to land.

3.2.3 List of available parcels and their surfaces – if the operator is manure producer, at least one row must be filled in (List of parcels or Contract/s) or both.

4. Waste management

Fill in only for hazardous waste.

6. Vibrations

Fill in only if specific study was undertaken.

F. Description and characteristics of the environment at the installation site

Fill in only if specific study was undertaken either by operator or other institution.

G. Description and characteristics of existing or planned measures and equipment used for supervising the installation and environmental emissions

Table 1.

1.10.	Entity which performed measurement/sampling in last year

1.11.	Organisation which performed analysis/laboratory in last year

1.12.	Authorisation/accreditation for measurements or laboratory authorisation/accreditation which provided services in last year

4. Additional indicators which are being controlled by the operator – fill in just if such additional indicators exist and are relevant

H. Detailed analysis of the installation in relation to best available techniques (BAT)

1. List all BAT guidance (e.g. BREF documents and BAT conclusions), which are relevant for main and other activities according to national list of main polluting installations.

2. Use this BAT guidance for general comparison with BAT for all activities according to national list of main polluting installations (means comparison of techniques, not emission parameters).

Provide the comparison just in few main points. Use only comparison points, which can be:

- Used for binding conditions formulation,

- Proved, monitored and measured,

- Inspected.

3. Analyse emission parameters of the installation in relation to data contained in all BAT guidance /conclusions, and if does not exist, then with BREF documents. Analyse all emission parameters to the air, water and soil. If you find non-compliance with data from BAT guidance/conclusion, then justify the derogation in detail in an additional annex and propose measures and implementation timetable to achieve compliance with BAT associated emission levels.

I. List of measures which should be undertaken after shutdown of the installation, for the purpose of avoiding all pollution threats or avoiding threats to human health and remediation of the installation site

- If the ceasing of operation is not planned in near future, describe just briefly.

- If the purpose of the application is ceasing of operation, be very detailed.

- If the location examination and/or baseline report has been prepared, put here a plan for returning into the original state.

J. Identifying participants in the process and other entities for which the economic entity which manages the installation knows that they might be exposed to significant adverse effects if the existing or new installation would have transboundary effects

Group C.

To be filled in just in case of transboundary effect.

K. Statement

Statement must be signed by both operators and authorised consulting organisation representatives.

L. Abbreviations and symbols

List all abbreviations and symbols which are used in the application and annexes.

M. Annexes

The first annex is Non-technical summary. This annex will be used for the purpose of informing the public during the public participation period. It should be brief and comprehensive, and should contain summary of data mentioned under items A to J. The Non-technical summary should not have more than 5-10 pages of text + 2 maps.

The recommended content of the Non-technical summary is following.

1. Name, location, operator and owner of the installation

2. Short description of the installation, overall activities and main products

3. Name, code and capacity of the main installation activity pursuant to Annex I and all other activities pursuant to Annex I. Description of every activity with emphasis on environmental impact and use of resources and emission production.

3.1. Use of energy and water- annual quantities

3.2. Main raw materials and hazardous substances.

3.3. Used techniques and comparison with BAT

3.4. Significant air, water and soil emissions (concentrations and annual quantities) and impact on air, water and soil quality and other environmental components

3.5. Production of hazardous waste and its treatment

4. Planning for the future: measures for environmental impact decreasing, reconstruction, extension, etc.

List of attachments:

1. Ortho-photo map / wide surrounding

2. Layout / situation plan of the installation

3. Block diagram / technological scheme

N. Proposal of requirements for obtaining the permit – optional

The content of this chapter corresponds to information which will be listed in the integrated permit.

This chapter, if filled in properly, represents the wish of the operator how the binding conditions in the permit should look like.

This can be accepted by the regulator, or it becomes a base for negotiation.

# Annex 2. Integrated permit template

Integrated permit template

The purpose of this document is to present the structure and content of an integrated permit. It has been compiled following a review of best practice in permitting, with the emphasis on practical conditions taken from Czech, Croatian, Dutch and UK permits issued under the integrated pollution prevention and control regime. The integrated permit template is aimed at relevant permitting Authorities from project countries who are or will be responsible for issuing integrated permits.

The template is structured in such a way that it addresses all of the requirements of the Industrial Emissions Directive 2010/75/EU. At the same time the template is based on the recommendations for the environmental permitting law (task 2.1.2.3). References to legislative provisions required by national legislation should be inserted at appropriate points in the document template. In particular all three environmental media are addressed along with emissions and environmental monitoring. Specific sections and conditions are included which refer to specific requirements on energy efficiency, raw materials, noise etc.

The template contains the following main chapters:

- Description of the installation and connected activities and its location

- Operating conditions including emissions and other requirements to protect environment, reporting and improvement programme

- Expiring permits

- Justification

- Information about appeal.

Permit writers are strongly recommended to follow the structure of the template as it covers all relevant requirements of the legislation. Every section of the template contains examples of conditions that could be set for an installation. Permit writers must exercise their professional judgement in choosing which one of the examples is most appropriate for this installation they are permitting and they should insert any other relevant condition relevant for the permitted installation.

AUTHORITY (name)

Department of Environmental Permitting

…… (address and contact)

Application for integrated permit  No.: …… dated ……

Evidence number: ……

Dealt by …… (name/contact)

Place and date ……

D E C I S I O N

of the Authority  ……, Department of Environmental Permitting, on the application for integrated permit according to § … Act No. ….., on environmental permitting (further on only ‘Act on EP’).

Statement:

The Authority ……, Department of Environmental Permitting (further just ‘Authority’), as a relevant state authority according to the subject and place in line with § … Act No. ….., on ………., after carrying the administrative procedure according to the Act No. …….., on administrative procedure (Administrative Act), issues

the integrated permit

according to § …. the Act on EP

to the installation operator: …… (data on the legal/natural person)

For installation operation:

Description of the installation and connected activities:

Installation name: ……

Installation category: …… (according to national list of main polluting installations of the Act on Environmental Permitting)

Description of the technical and technological unit stated in national list of main polluting installations of the Act on Environmental Permitting: ……

Description of the technical and technological unit not stated in national list of main polluting installations of the Act on Environmental Permitting: ……

Description of the connected activities: ……

Description of the installation location:

Region: ……

Municipality: ……

Land number (lot of land number):…….

- Operating conditions

The Authority for the operator of the installation …… (data on the legal/natural person) sets the following binding conditions for the operation according to § …of the Act on EP:

1.	Emission limits for air and water protection and related monitoring

1.1.	Air protection

1.1.1	The emission concentrations of the substances in Table 1.1.1 in the gas released from the emission points specified in Table 1.1.1 are not to be exceeded.

Table 1.1.1.

| Emission point No. | Emission point description | Substance | Emission limit values (incl. reference period) measured in | Date when to fulfil the limit | Monitoring frequency | Monitoring method |
| --- | --- | --- | --- | --- | --- | --- |
| Example: A 1 | Incinerator line 1 (60 m stack) | Particulate matter | 30 mg/m3 as a half hour average | From the date of permit issuing | 1/2h average and daily average; continuous measurement | Insert reference to legislation or method |

Note: Emission limit value’ (ELV) means the mass, expressed in terms of certain specific parameters, concentration and/or level of an emission, which may not be exceeded during one or more periods of time; ELV may be supplemented or replaced by equivalent parameters or technical measures ensuring an equivalent level of environmental protection for example:

- threshold (solvent consumption threshold in tonnes/year),

- limit for VOC (in percentage of solvent input),

- emission factors (in kg of pollutants per tonne of production)

    - Additional air protection conditions – if needed

  - Water protection

    - Water intake

1.2.1.1	Underground water source: definition…. and maximum water intake limits:…. (for example Q max (l/s), Qmonthly (m-3), Qyearly (m-3)

      - Surface water source definition of the intake place ……and maximum water intake limits:……………

    - Waste water/ Sewerage

      - Technological water conditions - the emission concentrations of the substances in Table 1.2.2 in the effluent released from the emission points specified in Table 1.2.2 are not to be exceeded.

Table 1.2.2

| Emission point No. | Emission point description | Substance | Emission limit values (including reference period) | Date when to fulfil the limit | Monitoring frequency | Monitoring method |
| --- | --- | --- | --- | --- | --- | --- |
| Example: W 1 | Discharge point to River No.: | Suspended solids | 100 mg/l | From the date of permit issuing | Flow weighted daily average | Insert reference to legislation or method |

      - The discharge temperature of the effluent water shall not exceed …oC above the temperature of the surface water. The maximum temperature of the effluent is not to exceed …oC.

1.2.3 Rain waters and water from hard surface – surface runoff?

1.2.3.1 Rain waters and waters from hard surfaces are collected separately to rain drainage which flow to retention tanks and further to biological ponds.

1.2.3.2	Additional water protection conditions – if needed

1.3	Odour

1.3.1	The Operator shall take measures to prevent or where that is not practicable to reduce odorous emissions from the Permitted Installation, in particular by:

-	limiting the use of odorous materials

-	restricting odorous activities

-	controlling the storage conditions of odorous materials

-	controlling processing parameters to minimise the generation of odour

-	timely monitoring, inspection and maintenance

-	employing an approved odour management plan.

Note:	An odour management plan may be appropriate where there is a need to set out responses to be taken in the event of an odour “incident”, or to cover odour management as a whole.

1.3.2	All emissions to air from the installation should be free from offensive odour as perceived by an authorized officer of the enforcing authority at or outside of the installation boundary.

2.	Emissions to land and related monitoring

  - No emissions of polluted rainwater or other substances from the Permitted installation should be made to land

  - .Leakage test of the waste water tanks and the safety tank under storage tanks 1x year.

  - All equipment which is used for gathering, usage, storage of dangerous substances, must be in such design and stage so any unwanted release of these substances into soil, underground water, surface water or unwanted mixing with waste or rain water will be not possible.

Note: if emissions to land are to be limited, use condition and table 1.2.2 as a template.

3.	Site and Roads

3.1	All process vessels should be located on a liquid tight floor so that any liquids can flow to the sewer system/catch pit. The sewer system should be provided with flame arresters.

  - The area where process equipment is located should be provided with an inflammable and liquid-tight surface.

  - In the immediate neighbourhood of the area where process equipment, tanks etc. are located there should be no inflammable vegetation. The presence of inflammable objects or materials that are not necessary for the process is not permitted.

4.	Prevention of accidents and minimisation of their effects

  - The Operator shall maintain and implement an accident management plan.

  - The accident management plan should be reviewed at least every 2 years or as soon as practicable after an accident, whichever is the earlier. The permitting/enforcing authority should be notified of the results of the review within 3 months of its completion.

5.	Management

Clear instructions for the operating personnel should be present in the control room. In these instructions, procedures to be followed should be indicated for the following conditions:

a.	the start-up of the installation;

b.	the operation of the installation;

c.	the shutdown of the installation;

d.	disturbances and/or emergencies in the installation under consideration or in another installation which can have an effect on the installation under consideration.

5.2	The Operator shall maintain a record of the skills and training requirements for all staff whose tasks in relation to the Permitted Installation may have an impact on the environment, and shall keep records of all relevant training.

6.	Incidents and Complaints

6.1	The Operator shall maintain and implement written procedures for:

- taking prompt remedial action, investigating and reporting actual or potential non-compliance with operating procedures or emission limits;

- investigating incidents, (including any malfunction, breakdown or failure of plant, equipment or techniques, down time, any short term and long term remedial measures and near misses) and prompt implementation of appropriate actions; and

- ensuring that detailed records are made of all such actions and investigations.

7.	Efficient use of raw and supplementary materials and water

  - The Operator shall carry out periodic waste minimisation audits and water use efficiency audits. If such an audit has not been carried out in the 2 years since the validity of this Permit, then the first such audit shall take place within 2 years of its issue.

8.	Energy Efficiency

8.1	The Operator shall produce a report on the energy consumed at the Permitted Installation over the previous calendar year, by 31 January each year.

8.2	The Operator shall maintain and update annually an energy management system which shall include, in particular, the monitoring of energy flows and targeting of areas for improving energy efficiency.

9.	Waste storage, handling, recovery or disposal

9.1	The Operator shall design, maintain and operate all facilities for the storage and handling of waste on site such that there are no releases to water or land during normal operation, and that emissions to air and the risk of accidental release to water or land are minimised.

9.2	The Operator is allowed on the site to generate the hazardous waste contained in the following list:

- List of hazardous waste

9.3	The above listed hazardous waste has to be sorted, gathered and handed over to the authorized company for treatment, recovery and/or disposal.

10.	Noise and Vibration

10.1	The equivalent sound pressure level (LAeq) produced by the installation as well as by the operations must not be higher than ….. dB(A) at the boundary of the installation.

10.2	Equipment and machinery must be arranged in such a way that it does not cause noticeable vibrations in areas not within the installation.

11.	Closure and Decommissioning

11.1	The Operator shall maintain and operate the Permitted Installation so as to prevent or minimise any pollution risk, including the generation of waste, on closure and decommissioning in particular by:

- taking into account the construction plan of new parts of the company or installation

- the maintenance of a record of any events which have, or might have, impacted on the condition of the site along with any further investigation or remediation work carried out

- the maintenance of a site closure plan to demonstrate that the installation can be decommissioned avoiding any pollution risk and returning the site of operation to a satisfactory state.

11.2	The Site Closure Plan should be implemented on final cessation or decommissioning of the Permitted activities or part thereof.

12.	Reporting on assessment of the compliance with permit conditions

12.1	The operator is obliged in accordance with relevant legal regulations to:

- Carry out inventory of the sources of air pollution and prepare overall operational inventory for the calendar year and hand it over to the Environmental Inspection by ….., on a yearly basis.

- Carry out continuous inventory of waste and the way of handling them.

- Submit the monitoring report about compliance with the integrated permit conditions to the Authority by 31.3., on a yearly basis.

- To inform the planned change of installation to the Authority.

Though monitoring is mentioned several times, it might be useful to devote a special paragraph to self-monitoring. Perhaps something general saying that self-monitoring programmes should be implemented and updated. Procedures, instructions, methods of measurements, record books should be checked and corrected when needed… Self-Monitoring BREF should be also mentioned.

13. On site monitoring

13. 1 The Operator shall maintain and implement an emissions monitoring programme which ensures that emissions are monitored from the specified points, for the parameters listed in, at the frequencies and using the methods described in Tables 1.1.1 and 1.2.2, (amend table numbers as necessary), unless otherwise agreed in writing, and he shall ensure that the results of such monitoring are assessed. The programme shall ensure that monitoring is carried out under an appropriate range of operating conditions. This program is part of the Monitoring Program listed further.

13.2	The Operator shall maintain the Monitoring Programme (MP) submitted which is annex to this permit, and shall carry out regular reviews of it. The results of such reviews and any changes made to the MP should be reported to the permitting authority within [1 month] of the review or change.

14.	Improvement programme

  - The Operator shall complete the requirements specified in Table 14.1 by the date specified in that Table.

Table 14.1

| Reference | Requirement | Date |
| --- | --- | --- |

II.	Expiring permits

The Authority in compliance with § … of the Act on EP cancels enforceable decisions, statements, opinions and approvals:

| Number of the permit | Date of the permit | Permitting authority |
| --- | --- | --- |

This integrated permit substitutes, in compliance with § …. of the Act on EP, decisions, statements, opinions and approvals which were issued based on the specific laws:

……

Responsibilities resulting from the regulations of specific legislation pieces and administrative acts that are not covered by this integrated permit shall remain, in compliance with valid § …. of the Act on EP, unchanged.

- Justification

Permitting process

The Authority received on …… (date) the application for an integrated permit according to § …of the Act on EP from the installation operator …… (name of the legal/natural person) for the installation operation …… Since this date the administrative procedure for issuing an integrated permit has started in compliance with § …of the Act No. ….., on administrative procedure (Administrative Act), and in compliance with the regulations of the Act on EP.

On (date) …… the application for issuing of an integrated permit was send to participants in the procedure, relevant state administrative authorities and at the same time the non-technical summary of this application was published on the notice board of the public administration …… and on the notice board of the municipality …… for a period of 30 days.

There were statements submitted to the application for issuing of an integrated permit.

Handling with comments on the application for issuing of the integrated permit contained in the statements on this application.

Participant in the procedure

| Comment of the participant in the procedure | How was it handled |
| --- | --- |

Comparison with the Best Available Techniques

The Authority during setting the binding conditions of the installation operation on the basis of the data stated in the application for integrated permit and statements received on this application considered also the aspects for BAT identification stated in the Annex … to the Act on EP with regard to technical characteristics of the installation, its location and local conditions of the environment:

1.	Usage of the low waste technologies

…… (fulfilment of the aspect)

2.	Usage of less dangerous substances

…… (fulfilment of the aspect)

3.	Support of regeneration and recycling of substances which arise or are used within the technological process, optionally regeneration or recycling of waste

…… (fulfilment of the aspect)

4.	Comparable processes, installations or operational methods which were already successfully tested in an industrial scale

…… (fulfilment of the aspect)

5.	Nature, effects and amount of relevant emissions

…… (fulfilment of the aspect)

6.	Time needed for BAT implementation

…… (fulfilment of the aspect)

7.	Consumption and type of raw materials (including water) used within the technological process and their energetic efficiency

…… (fulfilment of the aspect)

8.	Requirement on prevention or minimization of the whole emissions impact onto the environment and risks connected with them

…… (fulfilment of the aspect)

9.	Requirement on accident prevention and minimization of their effects for the environment

…… (fulfilment of the aspect)

The Authority during the assessment of the application for integrated permit based its decision on data included in the application and on the oral discussion with the aim to ensure the high level protection of the environment on the basis of BAT in compliance with relevant regulations of the Act on EP.

Administrative fee in the sense of the Act No. ….., on administrative fees, at the level of …. was paid in cash at the cash desk of the Authority on (date) …… /credit transfer to the account of the Authority No. …… on (date)…….

On the basis of the above mentioned facts the Authority decided as it is stated in the statement of this decision.

- Information about appealing

It is possible to appeal against this decision to the Ministry of Environment by a submission done at the Authority …… (address), and it just be within the period of 15 days from the delivery of this decision.

Name, surname and function of the relevant person from the Authority and their signature

The official stamp

Annexes:

……

List of receivers:

Participants in the procedure

……

State authorities concerned:

…….

# Annex 3. Model administrative regulations for Designated Administrator responsible for integrated environmental permitting

Пояснительная записка
к Модельному административному регламенту исполнения уполномоченным органом государственной функции по осуществлению выдачи комплексных экологических разрешений предприятиям и организациям

Модельный административный регламент исполнения уполномоченным органом государственной функции по осуществлению выдачи комплексных экологических разрешений предприятиям и организациям разработан на основе проекта российского федерального закона «О внесении изменений в отдельные законодательные акты Российской Федерации в части совершенствования нормирования в области охраны окружающей среды и введения мер экономического стимулирования хозяйствующих субъектов для внедрения наилучших технологий» (далее – законопроект ТНОС).

Административные регламенты служат руководством по исполнению государственными органами исполнительной власти своих функций в соответствии с федеральным либо региональным законодательством. Поэтому все недостатки соответствующих законов (в частности, законопроекта ТНОС) отражаются и в административных регламентах.

Законопроект ТНОС в своем нынешнем виде явился плодом компромисса между Правительством РФ, представленном Министерством природных ресурсов и экологии (МПРиЭ) и Министерством экономического развития (МЭР), с одной стороны, и промышленниками (Российский союз промышленников и предпринимателей (РСПП) и Торгово-промышленная палата Российской Федерации (ТПП), с другой стороны. В последнее время доработка законопроекта ТНОС осуществлялась с участием Рабочей группы по совершенствованию нормирования в области охраны окружающей среды и введения мер экономического стимулирования хозяйствующих субъектов для внедрения наилучших технологий (далее – Рабочая группа), созданной по приказу Министра природных ресурсов и экологии №320 от 03.09.2013 г. В Рабочую группу вошли представители Министерств, РСПП, ТПП, общественных и научных организаций. Автор модельного регламента, эксперт проекта Air-Q-Gov М.В. Бегак, также был включен в эту группу.

Поскольку законопроект ТНОС в процессе согласований значительно отклонился от первоначальной версии, гармонизированной с Директивой ЕС о промышленном загрязнении (IED), то и Регламент вынужденно включил эти отклонения. На дату подготовки Регламента использовалась доступная в то время версия законопроекта от 27.08.2013 г. В настоящее время, появилась новая версия законопроекта, которая будет представлена в Государственную Думу РФ на второе чтение (20.06.2014 г.). Поэтому в модельный административный регламент вносятся замечания, которые отражены в настоящей пояснительной записке. Кроме того, в записке отражены те отличия от IED, которые существуют в российском законопроекте ТНОС.

Так, например, изменен пункт 6.1 Регламента, описывающий содержание заявки на комплексное экологическое разрешение. В него, в соответствии с IED, включаются данные о сырье и вспомогательных материалов, а также энергии, используемой установкой. В соответствии с российским законодательством, а также законодательством Украины и Беларуси, туда включены требования о представлении в заявки информация о положительном заключении государственной экологической экспертизы документации, обосновывающей строительство, реконструкцию установки и информации о положительном заключении экспертизы промышленной безопасности (если требуется).

В связи с этим в заявке не предусмотрено требуемое IED описание «основных альтернатив предлагаемым технологиям, методам и мерам, изученные заявителем в общих чертах», поскольку альтернативы уже были рассмотрены на стадии государственной экологической экспертизы.

В остальном, требования законопроекта ТНОС и модельного регламента в части содержания заявки на комплексное экологическое разрешение соответствуют статье 12 IED.

Обновленной версией законопроекта не предусмотрено оснований для отказа в выдаче комплексных экологических разрешений за исключением недостоверности и неточности материалов заявки. Это основание не является фатальным, и заявка может быть снова рассмотрена после устранения недочётов. Поэтому пункт 7 Регламента изменён.

Порядок выдачи комплексных экологических разрешений, внесения в них изменений, их переоформления и пересмотра законопроектом не определен и будет, впоследствии, установлен Правительством. Содержащиеся в предлагаемом Регламенте нормы соответствуют общей практике выдачи частных экологических разрешений.

Учитывая замечание рецензента, в пункте 15.6 увеличено время рассмотрения материалов заявки с 60 до 90 рабочих дней, в том случае, если требуется предоставление Заявителем дополнительных материалов. Также включено положение о необходимости консультаций с Заявителем.

В соответствии с российским законодательством вопросы общественного обсуждения проектов хозяйственной деятельности относятся к компетенции органов местного самоуправления. Поэтому в пункте 15.6 указано, что общественные слушания по заявке на комплексное экологическое разрешение производятся по инициативе населения, представительного органа муниципального образования или главы муниципального образования. Органы государственной власти обязаны оказывать содействие органам местного самоуправления в организации и проведении слушаний. Материалы заявки на комплексное экологическое разрешение находятся в открытом доступе, как указано в п. 6.2 Регламента.

Регламент рассматривает процедуры выдачи комплексного экологического разрешения, выдачи временного комплексного экологического разрешения на период выполнения согласованной с контролирующим органом программы повышения экологической эффективности, приостановления и возобновления действия комплексного экологического разрешения, а также переоформления и пересмотра разрешения.

Предусмотрена также процедура продления комплексного экологического разрешения в том случае, если все технологические процессы остались неизменными, а срок разрешения истек. Эта процедура, по рекомендации рецензента, осуществляется аналогично процедуре пересмотра разрешения.

В связи с появлением в законопроекте ТНОС оснований для отзыва комплексного экологического разрешения

Приостановление действия комплексного экологического разрешения происходит на основании данных государственного экологического надзора о выявлении одного из перечисленных фактов:

- неоднократное (более двух раз) превышение (более чем в три раза) технологических нормативов выбросов и сбросов или нормативов допустимого сброса (выброса) для хозяйствующего субъекта, получившего временное комплексное экологическое разрешение;

- отсутствие производственного лабораторного контроля за качеством и количеством выбросов и сбросов веществ, указанных в комплексном экологическом разрешении;

- невыполнение в установленные сроки плана по снижению сбросов и выбросов загрязняющих веществ либо недостижение запланированной эффективности плана мероприятий по охране окружающей среды или программы повышения экологической эффективности для хозяйствующего субъекта, получившего временное комплексное экологическое разрешение.

При ликвидации перечисленных нарушений действие комплексного экологического разрешения возобновляется. В случае невозможности осуществления лабораторного контроля своими силами, эта процедура может осуществляться сторонней организацией.

Из раздела V, по рекомендации рецензента, исключён раздел, рассматривающий процедуру рассмотрения жалоб, как дублирующий Административный кодекс.

МОДЕЛЬНЫЙ АДМИНИСТРАТИВНЫЙ РЕГЛАМЕНТ
ИСПОЛНЕНИЯ УПОЛНОМОЧЕННЫМ ОРГАНОМ ГОСУДАРСТВЕННОЙ ФУНКЦИИ ПО ОСУЩЕСТВЛЕНИЮ ВЫДАЧИ КОМПЛЕКСНЫХ ЭКОЛОГИЧЕСКИХ РАЗРЕШЕНИЙ ПРЕДПРИЯТИЯМ И ОРГАНИЗАЦИЯМ

- Общие положения

1. Настоящий Административный регламент определяет последовательность и сроки действий (административных процедур) исполнения Уполномоченным органом государственной функции по выдаче комплексных экологических разрешений предприятиям и организациям, на которых, в соответствии с законодательством Государства, распространяется данная мера регулирования воздействия на окружающую среду (далее — Заявитель).

2. Государственная функция по выдаче комплексных экологических разрешений исполняется непосредственно Уполномоченным органом.

3. Содержание комплексного экологического разрешения.

3.1. Комплексное экологическое разрешение содержит:

- технологические нормативы;

- допустимые уровни физических воздействий;

- нормативы образования отходов производства и потребления и лимиты на их размещение;

- требования по обращению с отходами производства и потребления;

- согласованную программу производственного экологического контроля;

- требования о представлении уведомлений о превышении установленных выбросов, сбросов загрязняющих веществ, об авариях, повлекших загрязнение окружающей среды, о временном или постоянном прекращении деятельности;

- срок действия комплексного экологического разрешения.

3.2. Комплексное экологическое разрешение может быть выдано на период реализации плана мероприятий по охране окружающей среды или программы повышения экологической эффективности включающих в себя показатели планируемого снижения объёмов (массы) выбросов, сбросов загрязняющих веществ (временное комплексное экологическое разрешение).

3.3. Временное комплексное экологическое разрешение дополнительно содержит:

- утверждённый план мероприятий по охране окружающей среды или программу повышения экологической эффективности;

- временно разрешённые выбросы, сбросы загрязняющих веществ.

4. Срок действия, условия и порядок продления или пересмотра комплексного экологического разрешения.

4.1. Срок действия комплексного экологического разрешения составляет семь лет.

4.2. Комплексное экологическое разрешение продлевается на семь лет при следующих условиях:

- соблюдения технологических нормативов выбросов и сбросов, нормативов образования отходов производства и потребления и лимитов на их размещение;

- отсутствия задолженности по плате за негативное воздействие на окружающую среду;

- своевременного представления отчетности о выполнении программ производственного экологического контроля, уведомлений о временном прекращении деятельности.

4.3. Комплексное экологическое разрешение пересматривается в случае изменений технологических процессов, замены оборудования, сырья, повлекших увеличение установленных объёмов или масс выбросов, сбросов загрязняющих веществ, допустимых уровней физических воздействий, нормативов образования отходов производства и потребления и лимитов на их размещение.

Пересмотр комплексного экологического разрешения осуществляется в порядке выдачи комплексного экологического разрешения, установленном настоящим Административным регламентом.

4.. Подача заявки на получение комплексного экологического разрешения осуществляется не позднее, чем за три месяца до истечения срока действующего комплексного экологического разрешения или до ввода в эксплуатацию построенного, реконструированного объекта.

5. Требования к порядку информирования о предоставлении государственной услуги.

5.1. Государственная услуга по выдаче комплексных экологических разрешений осуществляется в Управлении разрешительной деятельности Уполномоченного органа, расположенной по адресу: ************

График работы: с 10 до 18 часов ежедневно, кроме субботы и воскресенья.

Телефон для справок по входящей корреспонденции: ********.

Телефон для справок по исходящей корреспонденции: ********.

5.2. В соответствии с законом Государства ***** предоставление государственных услуг Уполномоченного органа осуществляется посредством портала государственных и муниципальных услуг в электронной форме.

Уполномоченный орган является участником межведомственного информационного взаимодействия, осуществляемого в целях предоставления государственных и муниципальных услуг по вопросам обмена документами и информацией, в том числе в электронной форме, между органами, предоставляющими государственные услуги.

Заявитель вправе не представлять документы, получение которых осуществляется в других органах исполнительной власти, их территориальных органах и подведомственных органам исполнительной власти организациях, участвующих в предоставлении государственных услуг, кроме документов личного хранения.

Телефон горячей линии по вопросам предоставления государственных услуг********

5.3. Консультации по процедуре исполнения государственной услуги по выдаче комплексного экологического разрешения могут предоставляться:

- по письменным обращениям;

- по телефону;

- по электронной почте.

Консультации представляются бесплатно.

5.4. При предоставлении консультаций по письменным обращениям ответ на обращение направляется почтой в адрес Заявителя в срок, не превышающий 30 рабочих дней с момента поступления письменного обращения.

5.5. При представлении консультаций по телефону должностные лица Уполномоченного органа обязаны в соответствии с поступившим запросом предоставлять информацию по следующим вопросам:

- информацию о входящих номерах, под которыми зарегистрированы в системе делопроизводства материалы Заявителя;

- информацию о принятии решения по конкретному заявлению и прилагающимся материалам;

- сведения о нормативных правовых актах по вопросам выдачи комплексного экологического разрешения (наименование, номер, дата принятия нормативного правового акта);

- перечень документов, представление которых необходимо для выдачи комплексного экологического разрешения;

- требования к заверению документов, прилагаемых к заявлению;

- место размещения на официальном сайте Уполномоченного органа справочных материалов по вопросам выдачи комплексного экологического разрешения.

Иные вопросы рассматриваются только на основании соответствующего письменного обращения.

5.6. При консультировании по электронной почте ответ на обращение направляется на электронный адрес Заявителя в срок, не превышающий 5 рабочих дней с момента поступления обращения.

6. Правовые основания для предоставления государственной услуги

6.1. Предоставление государственной услуги по выдаче комплексных экологических разрешений осуществляется в соответствии с:

- Конституцией Государства;

- Законом Государства “О комплексном предупреждении и контроле загрязнений окружающей среды”;

- Законом Государства “Об организации представления государственных и муниципальных услуг”;

- Правилами разработки и утверждения административных регламентов предоставления государственных услуг, утвержденного постановлением Правительства Государства;

- Постановлением Правительства Государства “Об утверждении положения об Уполномоченном органе”

- Стандарт предоставления государственной услуги

6. Исчерпывающий перечень документов, необходимых для предоставления государственной услуги.

6.1. Заявитель включает в заявку на получение комплексного экологического разрешения следующие сведения:

- наименование, организационно-правовая форма и адрес (место нахождения) юридического лица или фамилия, имя, отчество (если имеется), место жительства индивидуального предпринимателя;

- код объекта, оказывающего негативное воздействие на окружающую среду;

- вид основной деятельности и объём выпускаемой продукции;

- информацию об использовании сырья, воды, электрической и тепловой энергии;

- данные об авариях и инцидентах, повлекших негативное воздействие на окружающую среду, произошедших за предыдущие семь лет;

- информация о реализации программы повышения экологической эффективности при ее наличии;

- проекты технологических нормативов;

- проекты лимитов на размещение для объектов, оказывающих негативное воздействие на окружающую среду, отнесенных к областям применения наилучших доступных технологий, а также нормативов образования отходов производства и потребления и лимитов на их размещение;

- проект программы производственного экологического контроля;

- информация о положительном заключении государственной экологической экспертизы документации, обосновывающей строительство, реконструкцию объекта хозяйственной и иной деятельности, в случае необходимости ее проведения в соответствии законодательством в области экологической экспертизы;

- информация о положительном заключении экспертизы промышленной безопасности в случае необходимости ее проведения в соответствии с законодательством в области промышленной безопасности опасных производственных объектов;

- иная информация, которую заявитель считает необходимым представить.

6.2. Электронный образ заявки на комплексное экологическое разрешение включается в информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

6.3. Заявитель, получающий временное комплексное экологическое разрешение, дополнительно к заявке представляет следующие документы:

- предлагаемые временно разрешённые выбросы, сбросы загрязняющих веществ с указанием объёмов или масс выбросов, сбросов загрязняющих веществ, образования отходов производства и потребления и их размещения на текущий момент, на период реализации плана мероприятий по охране окружающей среды или программы повышения экологической эффективности и после его выполнения;

- копия положительного заключения государственной экологической экспертизы документации, обосновывающей строительство, реконструкцию и техническое перевооружение объекта хозяйственной и иной деятельности, в случае необходимости ее проведения в соответствии законодательством в области экологической экспертизы.

6.4. Запрещается требовать от Заявителя иные документы, не предусмотренные настоящим Регламентом и осуществлять действия, не предусмотренные настоящим Регламентом.

Запрещается требовать от Заявителя документы и материалы, получаемые по межведомственному информационному взаимодействию в других государственных органах, органах местного самоуправления и у иных должностных лиц.

7. Исчерпывающий перечень оснований для отказа в приеме документов и в предоставлении государственной услуги.

7.1. В приёме документов отказывается, если документы, указанные в пунктах 6.1, 6.2 настоящего Регламента представлены Заявителем не в полном объёме (некомплектность материалов Заявителя).

7.2. Основаниями для отказа в выдаче комплексного экологического разрешения являются:

а) наличие в составе материалов Заявителя искаженных сведений или недостоверной информации;

б) наличие более пяти аварий и инцидентов, повлекших негативное воздействие на окружающую среду, произошедших за предыдущие семь лет.

7.3. Основаниями для отказа в выдаче временного комплексного экологического разрешения являются:

а) наличие в составе материалов Заявителя искаженных сведений или недостоверной информации;

б) наличие более пяти аварий и инцидентов, повлекших негативное воздействие на окружающую среду, произошедших за предыдущие семь лет;

в) невыполнение Заявителем в установленные сроки плана мероприятий по охране окружающей среды или программы повышения экологической эффективности.

8. За выдачу, продление и пересмотр комплексного экологического разрешения уплачивается государственная пошлина в размере *****. Государственная пошлина уплачивается до выдачи, продления и пересмотра комплексного экологического разрешения.

За приостановление действия комплексного экологического разрешения, возобновление действия разрешения и переоформление разрешения плата не взимается.

9. Пересмотр комплексного экологического разрешения и внесение изменений в комплексное экологическое разрешение осуществляется по инициативе юридического лица и индивидуального предпринимателя, получившего комплексное экологическое разрешение.

10. В случаях смены юридического лица и индивидуального предпринимателя, осуществляющего хозяйственную деятельность на объекте, реорганизации юридического лица в форме преобразования, изменения его наименования, адреса места нахождения, а также в случаях изменения места жительства, фамилии, имени и (в случае, если имеется) отчества индивидуального предпринимателя, реквизитов документа, удостоверяющего его личность, комплексное экологическое разрешение подлежит переоформлению одновременно с актуализацией сведений государственного учета объектов, оказывающих негативное воздействие на окружающую среду.

11. Показатели доступности и качества государственных услуг

Основными показателями доступности и качества государственной услуги являются количество жалоб от Заявителей о нарушениях сроков предоставления государственной услуги, предусмотренных настоящим Регламентом, а также количество судебных исков по обжалованию решений и действий (бездействия) Уполномоченного органа и его должностных лиц, предоставляющих государственную услугу.

12. Предоставление государственной услуги в электронной форме

12.1. Все предусмотренные настоящим Регламентом документы, необходимые для выдачи комплексного экологического разрешения либо временного разрешения могут быть поданы Заявителем в форме электронных документов в порядке, установленном Правительством Государства. В этом случае все уведомления о ходе предоставления государственной услуги, а также итоговые документы, направляются в электронной форме, если иное не указано Заявителем в заявлении. В этом случае комплексное экологическое разрешение по запросу Заявителя предоставляется ему также и на бумажном носителе.

12.2. Для получения комплексного экологического разрешения в электронной форме Заявитель направляет соответствующее заявление в электронной форме путем заполнения формы заявления, размещенной на сайте Уполномоченного органа в сети Интернет в разделе “Оказание государственных услуг в электронном виде” (далее — заявление в электронной форме), также можно направить заявление в электронной форме с использованием государственной информационной системы “Портал государственных услуг Государства”.

12.3. Рассмотрение заявления, полученного в электронной форме, осуществляется в том же порядке, что и рассмотрение заявлений, полученных лично от заявителей.

12.4. В заявлении, направленном в электронной форме, указывается один из следующих способов получения разрешения:

а) в виде бумажного документа, который заявитель получает непосредственно при личном обращении;

б) в виде электронного образа документа, который направляется заявителю посредством электронной почты.

13. Предоставление государственной услуги осуществляется с использованием системы межведомственного электронного взаимодействия.

Уполномоченный орган в установленном порядке и в сроки с использованием единой системы межведомственного электронного взаимодействия получает от уполномоченного органа исполнительной власти следующие документы о заявителе:

а) выписка из Единого реестра юридических лиц – для юридического лица;

б) выписку из Единого государственного реестра индивидуальных предпринимателей – для индивидуального предпринимателя;

в) копию свидетельства о постановке на учет в налоговом органе.

- Состав, последовательность и сроки выполнения административных процедур, требования к порядку их выполнения

14. В рамках исполнения государственной услуги по выдаче комплексных экологических разрешений осуществляются следующие административные процедуры:

- выдача комплексного экологического разрешения;

- выдача временного комплексного экологического разрешения;

- приостановление действия комплексного экологического разрешения;

- возобновление действия комплексного экологического разрешения;

- переоформление комплексного экологического разрешения;

- пересмотр комплексного экологического разрешения;

- продление комплексного экологического разрешения.

15. Административная процедура “Выдача комплексного экологического разрешения”.

15.1. Административная процедура “Выдача комплексного экологического разрешения” включает следующие административные действия:

- приём материалов (заявки) Заявителя;

- назначение ответственного должностного лица;

- рассмотрение материалов Заявителя, подготовку проекта разрешения и последующее издание приказа о выдаче комплексного экологического разрешения или об отказе в выдаче разрешения;

- оформление комплексного экологического разрешения;

- внесение информации о комплексном экологическом разрешении в соответствующий информационный ресурс (журнал, банк данных), включая информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

15.2. Основанием для начала административного действия является поступление в Уполномоченный орган Заявления и прилагающихся материалов в соответствии с пунктами 6.1 и 6.2 Регламента. Регистрация материалов Заявителя осуществляется в течение 1 рабочего дня. Результатом действия являются зарегистрированные в установленном порядке входящие материалы Заявителя.

15.3. Один экземпляр заявления и описи, подготовленной Заявителем, прилагающихся материалов с отметкой о дате приема направляется (вручается, возвращается) Заявителю. Зарегистрированные материалы Заявителя передаются начальнику Управления разрешительной деятельности Уполномоченного органа, уполномоченного на исполнения государственной услуги (далее – уполномоченное подразделение), для назначения ответственного исполнителя для их рассмотрения.

15.4. Начальник уполномоченного подразделения в течение 1 рабочего дня принимает решение о назначении ответственного должностного лица (далее – ответственный исполнитель) с учетом его должностных обязанностей и ставит резолюцию о назначении ответственного исполнителя на материалах Заявителя.

15.5. Ответственный исполнитель в течение 5 рабочих дней рассматривает материалы Заявителя на соответствие перечню документов, указанных в пунктах 6.1 и 6.2 Регламента, и в случае выявления некомплектности материалов Заявителя готовит проект письма Заявителю о приостановлении рассмотрения материалов по причине их некомплектности, которое передается на подпись руководителю уполномоченного подразделения или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом.

15.6. При установлении комплектности материалов Заявителя ответственный исполнитель, в течение не более 60 рабочих дней рассматривает материалы Заявителя. Во время рассмотрения заявки проводятся консультации с Заявителем. При необходимости получения дополнительной информации от Заявителя срок рассмотрения материалов может быть увеличен до 90 рабочих дней. В случае инициативы населения, представительного органа муниципального образования или главы муниципального образования на проведение публичных слушаний ответственный исполнитель оказывает содействие в организации и проведении публичных слушаний по заявке.

В том случае, если:

- в составе материалов Заявителя нет искаженных сведений или недостоверной информации;

- проект технологических нормативов сбросов и выбросов, использования сырья и энергии соответствуют параметрам НДТ;

- нормативы образования отходов производства и потребления соответствуют лимитам на их размещение;

- у Заявителя за предыдущие семь лет произошло менее пяти аварий и инцидентов, повлекших негативное воздействие на окружающую среду,

Ответственный исполнитель готовит решение о выдаче комплексного экологического разрешения. Подготовленный проект приказа Уполномоченного органа оформляется в соответствии с инструкцией по делопроизводству, визируется в установленном порядке и направляется на подпись руководителю Уполномоченного органа или лицу, его замещающему, в соответствии с должностным регламентом.

15.7. Основанием для оформления комплексного экологического разрешения является подписанный в установленном порядке приказ Уполномоченного органа о выдаче разрешения.

15.8. Должностное лицо Уполномоченного органа, ответственное за выполнение административного действия, в течение не более 5 рабочих дней с даты регистрации приказа Уполномоченного органа о выдаче комплексного экологического разрешения, оформляет комплексное экологическое разрешение.

Разрешение подписывается руководителем Уполномоченного органа или лицом, его замещающим, и заверяется оттиском гербовой печати.

Разрешения оформляются в двух экземплярах, один из которых выдается Заявителю, а второй хранится в уполномоченном подразделении Уполномоченного органа в течение семи лет.

Подписанное комплексное экологическое разрешение направляется в канцелярию Уполномоченного органа, сотрудник которого вносит информацию о подписании разрешения в базу данных входящих документов и информирует по телефону Заявителя о времени выдачи комплексного экологического разрешения в день поступления подписанного разрешения. Оформленное разрешение выдается Заявителю либо его представителю по доверенности сотрудником канцелярии, осуществляющим регистрацию входящей (исходящей) документации либо отправляется по почте по заявлению Заявителя. По заявлению Заявителя электронный образ разрешения направляется Заявителю посредством электронной почты.

Электронный образ комплексного экологического разрешения включается в информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

15.9. Отказ в предоставлении комплексного экологического разрешения оформляется приказом руководителя Уполномоченного органа и регистрируется канцелярией в том же порядке, что и приказ о выдаче разрешения. Один экземпляр приказа хранится в уполномоченном подразделении Уполномоченного органа в течение семи лет. Второй экземпляр приказа выдается Заявителю либо его представителю по доверенности сотрудником канцелярии, осуществляющим регистрацию входящей (исходящей) документации либо, по заявлению Заявителя, отправляется по почте.

По заявлению Заявителя электронный образ приказа направляется Заявителю посредством электронной почты.

16. Административная процедура “Выдача временного комплексного экологического разрешения”.

16.1. Административная процедура “Выдача временного комплексного экологического разрешения” включает следующие административные действия:

- приём материалов (заявки) Заявителя;

- назначение ответственного должностного лица;

- рассмотрение материалов Заявителя, подготовку проекта временного разрешения и последующее издание приказа о выдаче временного комплексного экологического разрешения или об отказе в выдаче разрешения;

- оформление временного комплексного экологического разрешения;

- внесение информации о временном комплексном экологическом разрешении в соответствующий информационный ресурс (журнал, банк данных), включая информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

16.2. Основанием для начала административного действия является поступление в Уполномоченный орган Заявления и прилагающихся материалов в соответствии с пунктами 6.1 и 6.2 Регламента. Регистрация материалов Заявителя осуществляется в течение 1 рабочего дня. Результатом действия являются зарегистрированные в установленном порядке входящие материалы Заявителя.

16.3. Один экземпляр заявления и описи, подготовленной Заявителем, прилагающихся материалов с отметкой о дате приема направляется (вручается, возвращается) Заявителю. Зарегистрированные материалы Заявителя передаются начальнику уполномоченном подразделении Уполномоченного органа для назначения ответственного исполнителя для их рассмотрения.

16.4. Начальник уполномоченного подразделения в течение 1 рабочего дня принимает решение о назначении ответственного должностного лица (далее – ответственный исполнитель) с учетом его должностных обязанностей и ставит резолюцию о назначении ответственного исполнителя на материалах Заявителя.

16.5. Ответственный исполнитель в течение 5 рабочих дней рассматривает материалы Заявителя на соответствие перечню документов, указанных в пунктах 6.1 и 6.2 Регламента, и в случае выявления некомплектности материалов Заявителя готовит проект письма Заявителю о приостановлении рассмотрения материалов по причине их некомплектности, которое передается на подпись руководителю уполномоченного подразделения или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом.

16.6. При установлении комплектности материалов Заявителя ответственный исполнитель, в течение не более 60 рабочих дней рассматривает материалы Заявителя. Во время рассмотрения заявки проводятся консультации с Заявителем. При необходимости получения дополнительной информации от Заявителя срок рассмотрения материалов может быть увеличен до 90 рабочих дней. В случае инициативы населения, представительного органа муниципального образования или главы муниципального образования на проведение публичных слушаний ответственный исполнитель оказывает содействие в организации и проведении публичных слушаний по заявке.

В том случае, если:

- в составе материалов Заявителя нет искаженных сведений или недостоверной информации,

- период реализации плана мероприятий по охране окружающей среды или программы повышения экологической эффективности не превышает семи лет,

- после реализации плана мероприятий по охране окружающей среды или программы повышения экологической эффективности выбросы и сбросы соответствуют установленным технологическим нормативам НДТ, а нормативы образования отходов производства и потребления соответствуют лимитам на их размещение,

- у Заявителя за предыдущие семь лет произошло менее пяти аварий и инцидентов, повлекших негативное воздействие на окружающую среду,

ответственный исполнитель готовит решение о выдаче временного комплексного экологического разрешения на срок реализации плана мероприятий по охране окружающей среды или программы повышения экологической эффективности. Подготовленный проект приказа Уполномоченного органа оформляется в соответствии с инструкцией по делопроизводству, визируется в установленном порядке и направляется на подпись руководителю Уполномоченного органа или лицу, его замещающему, в соответствии с должностным регламентом.

16.7. Основанием для оформления временного комплексного экологического разрешения является подписанный в установленном порядке приказ Уполномоченного органа о выдаче комплексного экологического разрешения.

16.8. Должностное лицо Уполномоченного органа, ответственное за выполнение административного действия, в течение не более 5 рабочих дней с даты регистрации приказа руководителя Уполномоченного органа о выдаче временного комплексного экологического разрешения, оформляет временное комплексное экологическое разрешение.

16.9. Разрешение подписывается руководителем Уполномоченного органа или лицом, его замещающим, и заверяется оттиском гербовой печати.

Разрешения оформляются в двух экземплярах, один из которых выдается Заявителю, а второй хранится в уполномоченном подразделении Уполномоченного органа в течение семи лет.

16.10. Подписанное временное комплексное экологическое разрешение направляется в канцелярию Уполномоченного органа, сотрудник которого вносит информацию о подписании разрешения в базу данных входящих документов и информирует по телефону Заявителя о времени выдачи временного комплексного экологического разрешения в день поступления подписанного разрешения. Оформленное разрешение выдается Заявителю либо его представителю по доверенности сотрудником канцелярии, осуществляющим регистрацию входящей (исходящей) документации либо отправляется по почте по заявлению Заявителя. По заявлению Заявителя электронный образ разрешения направляется Заявителю посредством электронной почты.

Электронный образ временного комплексного экологического разрешения включается в информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

16.11. Отказ в предоставлении временного комплексного экологического разрешения оформляется приказом руководителя Уполномоченного органа и регистрируется канцелярией в том же порядке, что и приказ о выдаче разрешения. Один экземпляр приказа хранится в уполномоченном подразделении Уполномоченного органа в течение семи лет. Второй экземпляр приказа выдается Заявителю либо его представителю по доверенности сотрудником канцелярии, осуществляющим регистрацию входящей (исходящей) документации. По заявлению Заявителя электронный образ приказа направляется Заявителю посредством электронной почты.

17. Административная процедура “Приостановление действия комплексного экологического разрешения”

17.1. Административная процедура “Приостановление действия комплексного экологического разрешения” включает следующие административные действия:

- получение информации и документированных фактов о выявлении оснований для приостановления действия комплексного экологического разрешения;

- назначение ответственного должностного лица;

- рассмотрение представленной информации государственного экологического надзора, содержащей основания для приостановления действия комплексного экологического разрешения и оценку соответствия информации перечню оснований для приостановления действия разрешения, установленному Постановлением Правительства Государства;

- подготовку и оформление в установленном порядке проекта приказа и издание приказа Уполномоченного органа о приостановлении действия комплексного экологического разрешения;

- подготовку и оформление письма хозяйствующему субъекту — владельцу разрешения о приостановлении действия комплексного экологического разрешения;

- направление письма о приостановлении действия разрешения хозяйствующему субъекту — владельцу разрешения о приостановлении действия комплексного экологического разрешения;

- внесение информации о приостановлении действия разрешения в соответствующий информационный ресурс (журнал, банк данных), включая информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

17.2. Основанием для начала исполнения Уполномоченным органом административной процедуры по приостановлению действия комплексного экологического разрешения является получение информации по результатам государственного экологического надзора о выявлении одного из перечисленных фактов:

- неоднократное (более двух раз) превышение (более чем в три раза) технологических нормативов выбросов и сбросов или нормативов допустимого сброса (выброса) для хозяйствующего субъекта, получившего временное комплексное экологическое разрешение;

- отсутствие производственного лабораторного контроля качества и количества выбросов и сбросов веществ, указанных в комплексном экологическом разрешении;

- невыполнение в установленные сроки плана по снижению сбросов и выбросов загрязняющих веществ либо недостижение запланированной эффективности плана мероприятий по охране окружающей среды или программы повышения экологической эффективности для хозяйствующего субъекта, получившего временное комплексное экологическое разрешение.

17.3. При получении в ходе государственного экологического надзора информации, содержащей основания для приостановления действия комплексного экологического разрешения, начальник уполномоченном подразделении Уполномоченного органа принимает решение о назначении должностного лица, ответственного за исполнение указанной административной процедуры (далее — ответственный исполнитель). Срок регистрации и назначения ответственного исполнителя — не более 1 рабочего дня.

17.4. Ответственный исполнитель в срок, не превышающий 5 рабочих дней, осуществляет рассмотрение представленной информации и оценку соответствия этих фактов основаниям для приостановления действия комплексного экологического разрешения, указанным в пункте 17.2 Регламента.

17.5. При подтверждении наличия оснований для приостановления действия разрешения, указанных в пункте 17.2 Регламента, ответственный исполнитель готовит проект приказа руководителя Уполномоченного органа о приостановлении действия комплексного экологического разрешения. Подготовленный проект приказа оформляется в соответствии с инструкцией по делопроизводству, визируется в установленном порядке и направляется на подпись руководителю Уполномоченного органа или лицу, его замещающему, в соответствии с должностным регламентом. Срок подготовки и издания приказа составляет не более 2 рабочих дней.

17.6. Основанием для приостановления действия комплексного экологического разрешения является подписанный руководителем Уполномоченного органа или лицом, его замещающим, приказ руководителя Уполномоченного органа о приостановлении действия комплексного экологического разрешения.

17.7. Ответственный исполнитель в течение не более 1 рабочего дня с даты издания приказа руководителя Уполномоченного органа о приостановлении действия комплексного экологического разрешения готовит письмо хозяйствующему субъекту комплексного экологического разрешения (владельцу разрешения) с информацией о приостановлении действия комплексного экологического разрешения с приложением соответствующего приказа руководителя Уполномоченного органа, направляет письмо на визирование начальнику уполномоченного подразделения Уполномоченного органа и на подпись руководителю Уполномоченного органа или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом. Подписанное письмо с прилагающимся приказом направляется в канцелярию либо соответствующее структурное подразделение, сотрудник которого регистрирует письмо и вносит информацию в базу данных входящей (исходящей) документации. Письмо направляется в адрес хозяйствующего субъекта - владельца разрешения, почтовым отправлением. Электронные образы письма и приказа отправляются владельцу разрешения электронной почтой.

17.8. После подписания, регистрации и направления хозяйствующему субъекту письма о приостановлении действия комплексного экологического разрешения информация о приостановлении действия разрешения вносится ответственным исполнителем в соответствующий информационный ресурс (журнал, банк данных), а также размещается в электронно-цифровой форме в информационной системе общего пользования для доступа общественности.

17.9. Срок исполнения Уполномоченным органом административной процедуры по приостановлению действия комплексного экологического разрешения — 10 рабочих дней со дня получения информации, являющейся основанием для приостановления действия разрешения.

18. Административная процедура “Возобновление действия комплексного экологического разрешения”.

18.1. Административная процедура “Возобновление действия комплексного экологического разрешения” включает следующие административные действия:

- получение Уполномоченным органом информации об устранении нарушений, повлекших за собой приостановление действия комплексного экологического разрешения;

- рассмотрение представленной информации о фактах устранения нарушений, повлекших за собой приостановление действия комплексного экологического разрешения, подготовку и оформление в установленном порядке проекта приказа руководителя Уполномоченного органа о возобновлении действия комплексного экологического разрешения;

- подготовку и оформление письма хозяйствующему субъекту - владельцу комплексного экологического разрешения о возобновлении действия разрешения;

- направление письма о возобновлении действия комплексного экологического разрешения хозяйствующему субъекту - владельцу разрешения;

- внесение информации о возобновлении действия комплексного экологического разрешения в соответствующий информационный ресурс (журнал, банк данных) и в информационную систему общего пользования для доступа общественности.

18.2. Основанием для начала административной процедуры по возобновлению действия комплексного экологического разрешения является получение по результатам государственного экологического надзора информации об устранении хозяйствующим субъектом - владельцем комплексного экологического разрешения нарушений, повлекших за собой приостановление действия разрешения.

18.3. При получении информации об устранении нарушений, повлекших за собой приостановление действия комплексного экологического разрешения, начальник уполномоченного подразделения Уполномоченного органа принимает решение о назначении должностного лица, ответственного за исполнение указанной административной процедуры (далее — ответственный исполнитель). Срок регистрации и назначения ответственного исполнителя — не более 1 рабочего дня.

18.4. Ответственный исполнитель в срок, не превышающий 5 рабочих дней, осуществляет рассмотрение представленной информации и оценку соответствия этих фактов условиям комплексного экологического разрешения.

18.5. При подтверждении наличия оснований для возобновления действия разрешения ответственный исполнитель готовит проект приказа руководителя Уполномоченного органа а о возобновлении действия комплексного экологического разрешения. Подготовленный проект приказа оформляется в соответствии с инструкцией по делопроизводству, визируется в установленном порядке и направляется на подпись руководителю Уполномоченного органа или лицу, его замещающему, в соответствии с должностным регламентом. Срок подготовки и издания приказа составляет не более 2 рабочих дней.

18.6. Основанием для возобновления действия комплексного экологического разрешения является подписанный руководителем Уполномоченного органа или лицом, его замещающим, приказ руководителя Уполномоченного органа о возобновлении действия комплексного экологического разрешения.

18.7. Ответственный исполнитель в течение не более 1 рабочего дня с даты издания приказа Уполномоченного органа о возобновлении действия комплексного экологического разрешения готовит письмо хозяйствующему субъекту комплексного экологического разрешения (владельцу разрешения) с информацией о возобновлении действия комплексного экологического разрешения с приложением соответствующего приказа руководителя Уполномоченного органа, направляет письмо на визирование начальнику уполномоченного подразделения Уполномоченного органа и на подпись руководителю Уполномоченного органа или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом. Подписанное письмо с прилагающимся приказом направляется в канцелярию либо соответствующее структурное подразделение, сотрудник которого регистрирует письмо и вносит информацию в базу данных входящей (исходящей) документации. Письмо направляется в адрес хозяйствующего субъекта - владельца разрешения, почтовым отправлением. Электронные образы письма и приказа отправляются владельцу разрешения электронной почтой.

18.8. После подписания, регистрации и направления хозяйствующему субъекту письма о возобновлении действия комплексного экологического разрешения информация о приостановлении действия разрешения вносится ответственным исполнителем в соответствующий информационный ресурс (журнал, банк данных), а также размещается в электронно-цифровой форме в информационной системе общего пользования для доступа общественности.

18.9. Срок исполнения Уполномоченным органом административной процедуры по возобновлении действия комплексного экологического разрешения — 10 рабочих дней со дня получения информации, являющейся основанием для возобновления действия разрешения.

19. Административная процедура “Переоформление комплексного экологического разрешения”.

19.1. Административная процедура “Переоформление комплексного экологического разрешения” включает следующие административные действия:

- приём и регистрацию материалов Заявителя;

- назначение ответственного должностного лица;

- рассмотрение материалов Заявителя;

- переоформление комплексного экологического разрешения;

- информирование и выдачу переоформленного комплексного экологического разрешения либо письма об отказе;

- внесение информации о переоформлении разрешения в соответствующий информационный ресурс (журнал, банк данных, реестр) и в информационную систему общего пользования для доступа общественности.

19.2. Комплексное экологическое разрешение подлежат переоформлению при условии неизменности производственного процесса, расхода сырья и материалов, номенклатуры и объёмов выпускаемой продукции (услуг), характеристик выпусков выбросов и сбросов загрязняющих веществ, масс размещаемых отходов в случаях:

- реорганизации юридического лица в форме преобразования, изменения наименования, в том числе фирменного наименования, места его нахождения, государственного регистрационного номера записи о регистрации юридического лица и данных документа, подтверждающего факт внесения записи о юридическом лице в Единый государственный реестр юридических лиц (для юридического лица);

- изменения фамилии имени и (в случае, если имеется) отчества индивидуального предпринимателя, места его жительства, данных документа, удостоверяющего его личность, государственного регистрационного номера записи о государственной регистрации в качестве индивидуального предпринимателя и данных документа, подтверждающего факт внесения записи об индивидуальном предпринимателе в Единый государственный реестр индивидуальных предпринимателей (для индивидуального предпринимателя);

- порчи или утраты комплексного экологического разрешения.

Переоформление комплексного экологического разрешения допускается только в период действия разрешения. Срок действия переоформленного комплексного экологического разрешения не должен превышать срока действия переоформляемого разрешения.

В случае изменения наименования, реквизитов юридических лиц и индивидуальных предпринимателей подача заявления о переоформлении комплексного экологического разрешения должна осуществляться в срок, не превышающий 30 рабочих дней с момента реорганизации юридического лица или изменения реквизитов юридического лица или индивидуального предпринимателя.

19.3. Основанием для начала административного действия является поступление в Уполномоченный орган заявления о переоформлении комплексного экологического разрешения, содержащего сведения о наименовании юридического лица, юридический и почтовый адрес, телефон, ИНН, с указанием причины необходимости переоформления разрешения, и материалов, подтверждающих необходимость переоформления комплексного экологического разрешения (не представляется в случае переоформления разрешения в связи с утратой, порчей оригинала разрешения):

- оригинала выданного ранее Заявителю в установленном порядке комплексного экологического разрешения (не представляется в случае переоформления разрешения в связи с утратой, порчей оригинала разрешения);

- справки о неизменности производственного процесса, расхода сырья и материалов, номенклатуры и объёмов выпускаемой продукции (услуг), характеристик выпусков выбросов и сбросов загрязняющих веществ, масс размещаемых отходов, подписанной Заявителем.

19.4. Уполномоченный орган в установленном порядке и в сроки с использованием единой системы межведомственного электронного взаимодействия получает от уполномоченного органа исполнительной власти следующие документы о Заявителе:

а) выписка из Единого реестра юридических лиц – для юридического лица;

б) выписку из Единого государственного реестра индивидуальных предпринимателей – для индивидуального предпринимателя;

в) копию свидетельства о постановке на учет в налоговом органе.

19.5. Основаниями для отказа в переоформлении комплексного экологического разрешения являются:

- наличие в представленных материалах Заявителя недостоверной или искаженной информации;

- представление Заявителем неполного комплекта документов, определенного в соответствии с пунктом 19.3 Регламента.

О принятии решения об отказе в переоформлении комплексного экологического разрешения Заявитель информируется ответственным исполнителем по телефону и по почте письмом.

19.6. Сотрудник канцелярии либо соответствующего структурного подразделения Уполномоченного органа осуществляет регистрацию материалов Заявителя в течение 1 рабочего дня. Один экземпляр заявления и описи, подготовленной Заявителем, прилагающихся материалов с отметкой о дате приема направляется (вручается, возвращается) Заявителю.

Зарегистрированные материалы Заявителя передаются начальнику уполномоченного подразделения Уполномоченного органа для назначения ответственного исполнителя для их рассмотрения.

Начальник уполномоченного подразделения Уполномоченного органа в срок, не превышающий 1 рабочего дня, назначает ответственного исполнителя с учетом его должностных обязанностей, о чем ставит резолюцию на материалах Заявителя. Ответственное должностное лицо (ответственный исполнитель) в соответствии с резолюцией получает материалы для исполнения.

19.7. Ответственный исполнитель в течение 5 рабочих дней рассматривает материалы Заявителя на соответствие перечню документов, указанных в пункте 19.3 Регламента, и в случае выявления некомплектности материалов Заявителя готовит проект письма Заявителю о приостановлении рассмотрения материалов по причине их некомплектности, которое передает на подпись начальнику уполномоченного подразделения Уполномоченного органа или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом.

При установлении комплектности материалов Заявителя ответственный исполнитель рассматривает материалы Заявителя и готовит решение о переоформлении комплексного экологического разрешения (проект приказа руководителя Уполномоченного органа, проект комплексного экологического разрешения) либо об отказе в переоформлении комплексного экологического разрешения в соответствии с пунктом 19.5. Регламента. Подготовленный проект приказа руководителя Уполномоченного органа оформляется в соответствии с инструкцией по делопроизводству, визируется в установленном порядке и направляется на подпись руководителю Уполномоченного органа или лицу, его замещающему, имеющему право подписи в соответствии с должностным регламентом.

Основанием для переоформления комплексного экологического разрешения является подписанный в установленном порядке приказ руководителя Уполномоченного органа о переоформлении комплексного экологического разрешения.

19.8. Ответственный исполнитель Уполномоченного органа в срок не более 2 рабочих дней с даты выпуска приказа руководителя Уполномоченного органа о переоформлении комплексного экологического разрешения оформляет комплексного экологического разрешения осуществляет визирование разрешения в порядке, определенном должностным регламентом. Разрешение подписывается руководителем Уполномоченного органа либо лицом, его замещающим, и заверяется оттиском гербовой печати. После подписания переоформленного разрешения бланк ранее выданного разрешения (оригинала разрешения, представленного Заявителем) прилагается к материалам Заявителя (дело Заявителя), которые передаются на хранение в архив.

19.9. Подписанное переоформленное комплексного экологического разрешения направляется в канцелярию Уполномоченного органа, сотрудник которого вносит информацию о подписании разрешения в базу данных входящих документов и информирует по телефону Заявителя о времени выдачи комплексного экологического разрешения в день поступления подписанного разрешения. Оформленное разрешение выдается Заявителю либо его представителю по доверенности сотрудником канцелярии, осуществляющим регистрацию входящей (исходящей) документации либо отправляется по почте по заявлению Заявителя.

По заявлению Заявителя электронный образ разрешения направляется Заявителю посредством электронной почты.

Электронный образ комплексного экологического разрешения включается в информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

19.10. Ответственный исполнитель в течение 1 рабочего дня с даты подписания переоформленного комплексного экологического разрешения, вносит в базу данных информацию о выданном разрешении, а также в перечень выданных разрешений с пометкой о переоформлении. Должностное лицо Уполномоченного органа, ответственное за выполнение административного действия, прилагает копию выданного переоформленного разрешения (либо отказа в переоформлении) к материалам Заявителя (дело Заявителя), сформированное при исполнении государственной услуги по выдаче комплексного экологического разрешения, хранящееся в архиве Уполномоченного органа, которое возвращается в архив для хранения.

20. Административная процедура “Пересмотр комплексного экологического разрешения”.

20.1. Административная процедура “Пересмотр комплексного экологического разрешения” включает следующие административные действия:

- приём материалов (заявки) Заявителя;

- назначение ответственного должностного лица;

- рассмотрение материалов Заявителя, подготовку проекта разрешения и последующее издание приказа о выдаче пересмотренного комплексного экологического разрешения или об отказе в выдаче разрешения;

- оформление комплексного экологического разрешения;

- внесение информации о комплексном экологическом разрешении в соответствующий информационный ресурс (журнал, банк данных), включая информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

20.2. Основанием для начала административного действия является поступление в Уполномоченный орган Заявления хозяйствующего субъекта — владельца комплексного экологического разрешения о изменении технологических процессов, замене оборудования, сырья, повлекших увеличение установленных объёмов или масс выбросов, сбросов загрязняющих веществ, допустимых уровней физических воздействий, нормативов образования отходов производства и потребления и лимитов на их размещение и прилагающихся материалов в соответствии с пунктами 6.1 и 6.2 Регламента.

20.3. Пересмотр комплексного экологического разрешения осуществляется в порядке выдачи комплексного экологического разрешения, установленном настоящим Регламентом.

21. Административная процедура “Продление комплексного экологического разрешения”.

21.1. Административная процедура “Продление комплексного экологического разрешения” включает следующие административные действия:

- прием материалов (заявки) Заявителя;

- назначение ответственного должностного лица;

- рассмотрение материалов Заявителя, подготовку проекта разрешения и последующее издание приказа о выдаче п комплексного экологического разрешения или об отказе в выдаче разрешения;

- оформление комплексного экологического разрешения;

- внесение информации о комплексном экологическом разрешении в соответствующий информационный ресурс (журнал, банк данных), включая информационную систему общего пользования в электронно-цифровой форме для доступа общественности.

21.2. Основанием для начала административного действия является поступление в Уполномоченный орган Заявления хозяйствующего субъекта — владельца комплексного экологического разрешения с просьбой продлить комплексное экологическое разрешение в связи с окончанием срока действия и прилагающихся материалов в соответствии с пунктами 6.1 и 6.2 Регламента.

21.3. Продление комплексного экологического разрешения осуществляется в порядке пересмотра выдачи комплексного экологического разрешения, установленном настоящим Регламентом.

- Формы контроля за предоставлением государственной услуги

22. Текущий контроль за исполнением Регламента осуществляется должностными лицами, ответственными за организацию работы по исполнению государственной услуги, путём проведения проверок соблюдения и исполнения ответственными исполнителями положений Регламента, нормативных правовых актов Государства.

Периодичность осуществления текущего контроля устанавливается руководителем Уполномоченного органа.

Перечень должностных лиц, осуществляющих текущий контроль, устанавливается индивидуальными правовыми актами Уполномоченного органа.

Текущий контроль (далее — проверки) может быть плановым (осуществляться на основании полугодовых или годовых планов работы Уполномоченного органа и внеплановым (проводиться по конкретному обращению Заявителя, органов государственной власти, пр.).

При плановой проверке контролю подлежат:

- соблюдение сроков прохождения заявления о принятии предварительного решения с даты регистрации до поступления на исполнение ответственному исполнителю;

- соблюдение сроков исполнения заявления о принятии предварительного решения, в том числе при уведомлении заявителя о предоставлении дополнительной информации и при отклонении заявления;

- полнота и правильность оформления комплексного экологического разрешения;

- правильность и обоснованность принятого решения о выдаче комплексного экологического разрешения;

- обоснованность предложений ответственного исполнителя о принятии решения об отклонении заявления, прекращении действия, изменении или отзыве решения;

- соблюдение сроков при предоставлении мотивированного отказа в выдаче комплексного экологического разрешения.

Комплексное экологическое разрешение подлежит отзыву в случае нарушения установленных комплексным экологическим разрешением обязательных требований, повлекших причинение значительного вреда здоровью и имуществу граждан, окружающей среде, обнаружения искаженных данных, представленных в заявке на получение комплексного экологического разрешения или отчете, содержащем сведения об организации и о результатах осуществления производственного экологического контроля, а так же в случае снятия объекта хозяйственной и иной деятельности с государственного учёта объектов, оказывающих негативное воздействие на окружающую среду.

При проверке могут рассматриваться другие вопросы, связанные с исполнением государственной услуги (комплексные проверки) или исполнением отдельных административных процедур (тематические проверки).

Проверки полноты и качества исполнения государственной услуги осуществляются на основании индивидуальных правовых актов (приказов) Уполномоченного органа.

23. Внеплановые проверки могут проводиться по конкретному обращению заявителей или иных заинтересованных лиц.

23.1. Внеплановая проверка осуществляется посредством рассмотрения представленных заявителем документов, принятых должностными лицами Уполномоченного органа, на предмет соответствия предоставления государственной услуги порядку и срокам, установленным настоящим Регламентом.

23.2. Проверке подлежат результаты рассмотрения жалобы на действия (бездействие) и решения должностных лиц Уполномоченного органа при предоставлении государственной услуги, а также результаты принятых решений по не менее чем десяти заявлениям заявителей о выдаче комплексного экологического разрешения или мотивированный отказ в его выдаче; приостановлении и возобновлении действия комплексного экологического разрешения; переоформления и пересмотра комплексного экологического разрешения.

24. Результаты плановой и внеплановой проверок оформляются актом.

Акт составляется в двух экземплярах в произвольной форме, при этом в акте приводится информация о выявленных в ходе проверки нарушениях административных процедур и сроков, установленных настоящим Регламентом, с указанием должностных лиц, допустивших нарушения; о правомочности и обоснованности принятых решений; о соответствии должностных регламентов государственных гражданских служащих, участвующих в предоставлении государственной услуги по выдаче комплексных экологических разрешений, настоящему Регламенту в части наличия в них записи о соответствующих административных действиях; о выводах комиссии по результатам проверки полноты и качества предоставления государственной услуги.

25. Для проведения проверки полноты и качества исполнения государственной услуги может быть сформирована комиссия, в состав которой включаются государственные гражданские служащие Уполномоченного органа.

Деятельность комиссии осуществляется в соответствии с индивидуальными правовыми актами (приказами Уполномоченного органа).

Результаты деятельности комиссии оформляются в виде акта проверки, в котором отмечаются выявленные недостатки и предложения по их устранению.

Акт проверки подписывается председателем комиссии и руководителем проверяемого структурного подразделения Уполномоченного органа, ответственного за выдачу разрешительных документов.

26. По результатам проведенных проверок в случае выявления нарушений положений Регламента, иных нормативных правовых актов Государства осуществляется привлечение виновных лиц к ответственности в соответствии с законодательством Государства.

Ответственный исполнитель, уполномоченный на подготовку проекта комплексного экологического разрешения, несет персональную ответственность за соблюдение сроков рассмотрения документов, по результатам которых может быть принято решение о выдаче комплексного экологического разрешения.

Должностное лицо, уполномоченное принимать решение о выдаче комплексного экологического разрешения, несет персональную ответственность за правильность и обоснованность принятого решения.

Соблюдение установленной руководителем Уполномоченного органа периодичности осуществления текущего контроля, в том числе сроков плановых проверок, является обязательным.

По результатам осуществления контроля за исполнением государственной услуги по выдаче комплексных экологических разрешений принимаются меры по устранению выявленных недостатков.

27. Персональная ответственность государственных гражданских служащих Уполномоченного органа закрепляется в должностных регламентах в соответствии с требованиями законодательства Государства.

28. Уполномоченный орган осуществляет координацию и методическое обеспечение деятельности по вопросам выдачи комплексных экологических разрешений, а также контроля за исполнением государственной услуги.

29. Контроль исполнения административных процедур по предоставлению государственной услуги со стороны граждан, их объединений и организаций осуществляется с использованием соответствующей информации, размещенной на официальном Интернет-сайте Уполномоченного органа.

- Досудебный (внесудебный) порядок обжалования решений и действий (бездействия) Уполномоченного органа и его должностных лиц, предоставляющих государственную услугу

30. Заявитель имеет право на обжалование решений, действий (бездействия) Уполномоченного органа и его должностных лиц в досудебном (внесудебном) порядке:

1) жалобы на постановления по делам об административных правонарушениях рассматриваются в порядке, установленном законодательством Государства об административных правонарушениях;

2) жалобы на иные решения, действия (бездействие) должностных лиц, осуществляемые (принятые) в ходе предоставления государственной услуги, рассматриваются в порядке, установленном законодательством Государства.

31. Ответ на жалобу не дается по основаниям, указанным в законодательстве Государства:

1) отсутствия в жалобе фамилии (наименования) заявителя, направившего обращение, и почтового адреса, по которому должен быть направлен ответ;

2) содержания в жалобе нецензурных либо оскорбительных выражений, угроз жизни, здоровью и имуществу должностного лица, а также членов его семьи;

3) если текст жалобы, фамилия (наименование) и (или) почтовый адрес заявителя не поддаются прочтению;

4) если в письменном обращении заявителя содержится вопрос, на который заявителю многократно давались письменные ответы по существу в связи с ранее направляемыми обращениями, и при этом в обращении не приводятся новые доводы или обстоятельства.

32. Заявитель имеет право обратиться с жалобой лично или направить ее в письменном виде.

Заявитель имеет право на получение информации и документов, необходимых для обоснования и рассмотрения жалобы.

Должностные лица Уполномоченного органа:

- обязаны предоставить заявителю по его просьбе ознакомление с документами и материалами, необходимыми ему для обоснования и рассмотрения жалобы, т.е. обеспечить его информацией, непосредственно затрагивающей его права, если иное не предусмотрено законом;

- обязаны обеспечить объективное, всестороннее и своевременное рассмотрение жалобы, в случае необходимости — с участием заявителя, направившего жалобу, или его законного представителя;

- вправе запрашивать необходимые для рассмотрения жалобы документы и материалы в других государственных органах, органах местного самоуправления и у иных должностных лиц, за исключением судов, органов дознания и органов предварительного следствия;

- обязаны по результатам рассмотрения жалобы принимать меры, направленные на восстановление или защиту нарушенных прав, свобод и законных интересов заявителя, давать письменный ответ по существу поставленных в жалобе вопросов.

33. Руководители и должностные лица Уполномоченного органа, участвующие в предоставлении государственной услуги, проводят личный прием заявителя и рассматривают поступившие в устном порядке жалобы в течение 1 дня. Содержание устной жалобы заносится в карточку личного приема заявителя. О результатах рассмотрения устной жалобы должностное лицо, которое проводило прием заявителя, сообщает в устной форме лично или по телефону, в случае если изложенные в устном обращении факты и обстоятельства являются очевидными и не требуют дополнительной проверки. В остальных случаях дается письменный ответ по существу поставленных в жалобе вопросов.

34. Информация о месте приема, а также об установленных для приема днях и часах размещается на информационных стендах Уполномоченного органа, официальном сайте Уполномоченного органа.

35. При обращении заявителя в письменной форме с жалобой на действия должностных лиц в рамках осуществления конкретной административной процедуры (административного действия) жалоба рассматривается в срок, не превышающий десяти дней с момента получения обращения (жалобы).

36. Жалоба заявителя в письменной форме должна содержать следующую информацию:

- наименование государственного органа, в который направляется жалоба, либо фамилию, имя, отчество, должность соответствующего должностного лица;

- фамилию, имя, отчество (последнее - при наличии) гражданина, наименование юридического лица, которым подаётся жалоба, почтовый адрес, по которому должен быть направлен ответ;

- суть обжалуемого действия (бездействия) при исполнении конкретных административных процедур, установленных настоящим Регламентом;

- причины несогласия с обжалуемым действием (бездействием);

- обстоятельства, на основании которых Заявитель считает, что нарушены его права, свободы и законные интересы, созданы препятствия к их реализации либо незаконно возложена какая-либо обязанность в связи с исполнением должностными лицами Уполномоченного органа административных процедур (административных действий), установленных настоящим Регламентом;

- иные сведения, которые Заявитель считает необходимым сообщить.

В случае необходимости в подтверждение своих доводов к жалобе прилагаются документы и материалы либо их копии. В таком случае в жалобе приводится перечень прилагаемых к ней документов.

Жалоба подписывается подавшим ее лицом.

37. Схема направления заявителем жалоб, связанных с выполнением административных процедур (административных действий), установленных настоящим Регламентом:

- жалоба на действия (бездействие) должностных лиц Уполномоченного органа — руководителю Уполномоченного органа или его уполномоченному заместителю;

- жалоба на действия (бездействие) руководителя Уполномоченного органа — Министру уполномоченного министерства Государства или его уполномоченному заместителю.

38. Жалоба, поступившая в орган, предоставляющий государственную услугу, подлежит рассмотрению должностным лицом, наделенным полномочиями по рассмотрению жалоб, в течение 15 рабочих дней со дня ее регистрации, а в случае обжалования отказа органа, предоставляющего государственную услугу, должностного лица органа, предоставляющего государственную услугу, в приеме документов у заявителя либо в исправлении допущенных опечаток и ошибок или в случае обжалования нарушения установленного срока таких исправлений — в течение 5 рабочих дней со дня её регистрации.

39. По результатам рассмотрения жалобы должностное лицо, ответственный или уполномоченный работник принимает следующие решения:

1) об удовлетворении требований заявителя и о признании неправомерным действия (бездействия) должностного лица;

2) об отказе в удовлетворении жалобы.

Письменный ответ, содержащий результаты рассмотрения жалобы, направляется заявителю.

40. Заявитель имеет право обжаловать решения, действия (бездействие) должностных лиц Уполномоченного органа в соответствии с законодательством Государства в суде общей юрисдикции, арбитражном суде.

# Annex 4 Registration form for low polluting activities

| НАЗВАНИЕ РЕГИСТРИРУЮЩЕГО ОРГАНА Заявление на внесение в Государственный реестр объектов с незначительным потенциалом загрязнения окружающей среды Адрес, телефон, факс, электронная почта регистрирующего органа |  | Логотип регистрирующего органа |  |
| --- | --- | --- | --- |
| #### Форма предоставляется заполненной печатными буквами либо в электронном виде |  |  |  |
| Дата подачи заявления (день, месяц, год) Заполняется регистрирующим органом | Регистрационный номер Y Y Заполняется регистрирующим органом. При переписке ссылаться на данный номер |  |  |
| НАЗВАНИЕ ЗАЯВКИ |  |  |  |
| Персональные данные заявителя: Имя Фамилия Адрес: Адрес по месту регистрации Город: Индекс: Телефон: Факс: Мобильный: E-mail: ИНН: (ИНН указывается, если заявитель является частным предпринимателем). | Данные организации, для которой осуществляется регистрация: Название организации Адрес: Адрес по месту регистрации Город: Индекс: Телефон: Факс: Мобильный: E-mail: ИНН: ОКВЭД: |  |  |
| #### Краткое описание основной деятельности и технологий, на которые выдаётся разрешение. |

The GBRs definition listed in the Industrial Emissions Directive is as follows: “GBRs’ means emission limit values or other conditions, at least at sector level, that are adopted with the intention of being used directly to set permit conditions” (Article 3, paragraph 8)

The Application of General Binding Rules in the Implementation of the IPPC Directive, IMPEL Network, 2001

For example it is a regulation of small polluting sources in the Czech Republic where national ELVs are stipulated for ten pollutants.

In many EU countries that had single-medium permitting before the implementation of the IPPC Directive in 2007 as well as in part of the new Member States, this system has remained in place for installations that are not subject to integrated permitting.

This statement should be justified by report on noise measurements, the relevant regulation can speficify which activities are likely to cause the noice or alternatively which are exmpt from the noise measurement requirement.

This may not be relevant for the project countries, if so please replace by the relevant codes