EUWI – EECCA Component

Country Situation and Priorities – Tadjikistan

Prepared by the Technical Secretariat

October 2005

EUWI – EECCA Technical Secretariat

Country Situation and Priorities : Tadjikistan

# EUWI Objectives: Current Situation

Water Supply and Sanitation

## Objective 1: Improve the Institutional and Regulatory Framework

Action 1.1 Decentralise administrative responsibility from national level to municipal level.

Action 1.2 Provide institutional autonomy to the municipal water authorities

Action 1.3 Commercialise the autonomous utilities as corporations

Action 1.4 Introduce a formal regulatory structure

Kyrgyzstan is a mountainous country with a population of 4.9 million, about 70% of whom live in the rural areas, though there is an increasing trend for some of the rural population to move into the cities. The registered city populations do not therefore fully reflect the actual situation.

Provision of centralised water supply and sanitation services in Tadjikistan falls under 3 main categories:

- The 6 largest cities which are served by water and sewerage utilities (Vodokanal) which are directly responsible to the City Administrations (Mairie); [note that a draft report of the Urban Institute also lists Sarband and Varsob]

- The remainder of the urban areas (small towns and District Centres) which are served by water and sewerage utilities that operate under contract with the State Unitary Enterprise (SUE): The mandate of SUE is wider than just the provision of water supply and sewerage services. It also includes maintenance and servicing of public housing stock, maintenance and cleaning of municipal buildings and public places, district heating, and solid waste collection and disposal. There are 8 small towns and District Centres  that have no sewerage at all. The remainder generally have partial coverage of sewerage but there are even some 2-story buildings with no sewerage.

- The potable water supply services to 1.2 million people through 5 systems in 24 Districts at many former state and collective farms that are provided by the Rural Water Supply Enterprise (RWSE) of the Ministry of Water Resources. In these communities no communal sewerage is provided but sanitation utilises pit latrines. Originally 48 major schemes were planned but only 5 were started, and even those not completed before the break-up of the USSR, partly because not all the planned state and collective farms were constructed. Many schemes also provide water to the farm for cattle watering and other purposes. Some RWSE water supply schemes also supply water to some Vodokanals that are under the auspices of the SUE.

In addition, there are many small rural communities that are not provided with water supply or sanitation services from centralised services. Potable water in these communities is obtained from streams, springs, and irrigation canals. Sanitation in these communities utilises pit latrines.

Decentralisation and introduction of autonomy of operation of the water utilities have only been implemented partially. The 8 city Vodokanals are decentralised but are directly controlled by the city administrations that they serve. Major expenditures, changes in management staff and changes in tariff cannot be implemented without the approval of the city administration, which in turn is subject to controls from central government through the normal hierarchical system of local government. Proposals have already been made for reform of these utilities but government decisions are awaited. The State Unitary Enterprise (SUE) is the transformed Ministry of Housing and Communal Services (MHCS). Although the SUE now has a contract with the Vodokanals that lists services provided by SUE to the Vodokanals, other elements of the relationship, such as a fixed payment to the SUE and the responsibility of the Vodokanals to submit their accounts to SUE for approval indicate that the relationship is not much different from previously, when the Vodokanals were organs of the MHCS. In practise, the SUE operates little differently from when it was the Ministry, with transparent direct control over Vodokanals.

The SUE has prepared a proposal for its own reform and has presented it to government for consideration. The proposal is based on privatising housing and other municipal services.

The Rural Water Supply Enterprise is not decentralised at all and is a centrally controlled government organisation. The organisation is directly responsible for all aspects of the service in the schemes supplied, apart from those schemes where water is provided in bulk to the SUE vodokanals. Until 1997 the RWSE was directly responsible to Central Government, after which it was transferred to the responsibility of the Ministry of Water Resources and Land Reclamation (Melioratsia). Water delivery is 24 hr during Spring and Summer but falls to 4 – 5 hrs per day during the winter because of electricity shortages.

## Objective 2: Ensure Financial Viability of Utilities

Action 2.1 Adopt a financing plan to introduce full cost recovery

Action 2.2 Eliminate subsidies between industrial consumers and household consumers

Action 2.3 Adjust billing procedures so that water tariffs are based on actual water consumption

Action 2.4 Raise collection ratios to acceptable levels

Action 2.5 Use social tariffs for the poor

The tariff rates for provision of water supply and sewerage services are approved by the State Agency for Antimonopoly Policy and Business Support, in coordination with the Ministry of Finance and the Ministry of Economics. The occasional very poor quality of water in Dushanbe results in a poor attitude of consumers and resistance to paying increased fees.

The basic principles for financing the proposed transition of the SUE, as they relate to the provision of water supply and sewerage services are:

- Transition to self-financing;

- Government support will be needed

- The transition to new tariff rates should be gradual;

- It will be necessary to attract long-term credits to support the modernisation of the infrastructure;

- Reduction of expenditure by

  - Universal introduction of metering;

  - Tendering and contracting the provision of services on a competitive basis;

  - Regulation of tariffs on the basis of the reasonable costs for the works needed to supply and improve the service.

- Review the current cross-subsidies with a view to increasing the public’s share of financing the service.

- Providing social protection for the poor.

The SUE considers that reform will not be successful without simultaneous or even prior rehabilitation of the infrastructure to allow provision of a good service. SUE also calculates that the cost of full rehabilitation of Vodokanals reporting to SUE would be $ 380 million.

Although household metering is still at a relatively low level. the SUE Vodokanals have started a metering campaign, with the assistance of USAID and SUE is currently negotiating with a Russian firm for a licence to manufacture meters in-country. Reported consumption of 900 to 1000 l/c/d has fallen to 250 to 300 l/c/d once meters have been installed. However, low payment rates by government agencies (from which traditionally most revenue came) is causing a funding crisis.

The water supply schemes of the RWSE are seriously under-funded.  Fees are based on measurement of bulk supplies and divided pro-rata among the population of each scheme. Fee rates are 2 to 10 Dram per cu m (1 to 3 US cents). However, this provides only about 15% of the required income. A subsidy from government of around half the amount of collected fees is the only other source of funding, so that total credits are around 25% of estimated requirements. However, some maintenance and replacement has been done within this budget. Because of the inability to pay, some schemes or district centres ask for supplies to be reduced to only a few hours a day even when there is the capacity for more. This results in cross-subsidy from schemes that take 24 hr supplies because many of the operating costs are fixed and do not depend on the period of supply.

## Objective 3: Investment in Water Supply, Sanitation and Rehabilitation

Action 3.1 Identify the investment and rehabilitation needs through studies

Action 3.2 Prioritise building blocks

Action 3.3 Prepare projects

Action 3.4 Plan and obtain financing

Action 3.5 Implement

Dushanbe Vodokanal is typical of many of the 6 independent city water utilities, though it also has some specific problems. Most of the pipe network was constructed in 1931 of unprotected steel and is badly corroded so that pipeline bursts are one of the major problem is, though, the extremely turbid quality of raw water taken from the Varzob River after rain on the catchment. Internal pipework and fittings within housing blocks also cause pressure restrictions and high water losses.

There are 3 sources of raw water for Dushanbe: 1 wellfield, one river source, and one mixed source. The very high turbidity of the river and the mixed source after rain makes water undrinkable and hardly usable for other uses. The water treatment plant is no longer operating. The high turbidity is a serious health risk, as pathogens are carried in the turbid water and in the sediments left in the mains. Chlorination is difficult because of the difficulty of obtaining activated chlorine and the difficulty of transporting and handling it.

Some assistance has been provided by JICA ($ 2 million) to deal with the internal pipework and fittings of the oldest 15 housing blocks, including replacing toilet flushes. This project has dealt with about 8% of the city.

The World Bank has an investment project ($ 17 million) which is mainly replacing mains (37 km out of 720 km) and pumps. Most of the city’s pumps have now been replaced.

The wastewater treatment plant also does not operate and untreated 300 cu m/day of sewage is discharged directly into the Kafirnigan River.

The most important next investment projects would be:

- Procurement of a mini-plant for production of hyperchlorite (estimated cost $ 1.5 million);

- Construction of new wellfields and trunk mains to avoid the use of water from the Varzob River (estimated cost $ 45 million);

- Introduction of metering;

- Rehabilitation of the wastewater treatment plant (estimated cost ($ 45 million).

The first priority of the SUE Vodokanals is to rehabilitate mains and pump stations and then to engage in wide-spread metering. Rehabilitation of sewage treatment plants will be a third priority.

The installations of the RWSE are dominated by one large scheme in the Vaksh valley, which was planned to serve 860,000 people but now effectively only serves 500,000. The main intake is on a canal served by the Vaksh River and the scheme, which was initially constructed in 1977. The farms and district centres are served from this intake through a system of mains and pump stations, much of which is now in considerable decay.

Other major RWSE systems are those serving Hissar, Obikiek, Isfara, and Kuibishek and there are 70 individual village water supply systems.

Many of the schemes of the RWSE are in an advanced state of disrepair. In general, trunk mains and distribution networks have been constructed of unprotected and unlined steel. Depending on the aggressiveness of the soils, many of these pipes are badly corroded and high pressure mains suffer frequent bursts. The RWSE is repairing bursts in a fire-fighting manner but expects the incidence of bursts to increase. Pump stations are also in disrepair, usually with the spare capacity out of action and still utilising the original design of pumps, which have low efficiencies and result in high electricity costs. Some emergency rehabilitation work has already been carried out with assistance from UNOPS (using bilateral funds), UNICEF and USAID.

Humanitarian organisations (NGOs) have been working to support rural communities, including in provision of water supply. The solution mainly adopted by these organisations has been to install shallow wells with hand pumps either in hand-dug wells or in rehabilitated borehole. About 8,000 such wells have been installed but unfortunately the shallow groundwater is of poor quality in many places and seasonal water table fluctuations or changes following the cleaning of open drains have resulted in some wells drying. The humanitarian organisations are now moving to deep tubewell technology. However, the rate of installation is currently far too low to provide a viable alternative to the RWSE schemes for most of the population.

The costs of hand-pump wells have varied from $ 150 for the simplest wells serving 8 families to $ 1,800 for deep drilled wells, serving 250 people ($ 4 to $ 8 per capita). The cost of rehabilitating the Vaksh system is estimated informally to be about $ 1.5 to $ 2.0 million ($ 2 per capita).

## Objective 4: Ensure Access of the Poor to Water Services as a Human Right

Currently water and sanitation customers who are not able to pay their fees are not disconnected. Under proposals for reform of the SUE the need for protection of the poor is recognised although it is not spelt out exactly how that would be implemented.

## Objective 5: Safeguard Public Health

The Ministry of Health is responsible for monitoring the quality of drinking water and for disseminating public health messages, including messages regarding cleanliness and its connection with health.

There are regular summer epidemics of disease related to poor water quality, including diarrhoeal diseases. Typical problems are those faced by Dushanbe where part of the water supply is provided direct from the river, part from groundwater, and part mixed. After heavy rains the river that is the water supply source becomes extremely turbid. As the clarification and filtration elements of the water treatment works no longer operate, the water supplied to the customers is also turbid and the turbidity harbours infections. The water is always chlorination during such periods but the turbidity makes chlorination less effective. In addition, there are problems in obtaining chlorine.

There has been no project support to public health apart from a UNICEF project to introduce rapid methods of evaluating water suitability for drinking. Humanitarian organisations also disseminate health and cleanliness messages as part of their rural support programmes. A project for prevention of children’s diseases has been proposed by the Ministry to both Italy and Norway. The project would include staff training, education and laboratory equipment.

The government would welcome a project of the same type as the DfID public health and sanitation project currently running in Kyrgyzstan.

## Objective 6: Protect the Environment

Environmental protection is the responsibility of the Ministry of Ecology and Natural Resources. Aspects of water management associated with environmental protection are set out under Objectives 7 and 8.

Integrated Water Resources Management

## Objective 7: Establish and Implement National Policies for Integrated Water Resources Management.

Action 7.1  Establish national IWRM policy through:

        - Raising Political Awareness of Water Management Issues

        - Developing National Road Maps for Completing the Adoption of IWRM

Action 7.2  Support the implementation of national IWRM policy through:

        - Identification of national water management priorities;

        - Strengthening the legal and regulatory framework;

        - Institutional capacity building;

        - Strengthening water resources monitoring and assessment;

        - Improving access to information and public participation;

        - Providing the research base for IWRM; and

        - Investment in improvements to water use efficiency

In principle the arrangements for water management have changed very little from the USSR period, although the names of ministries might have changed and some new legislation has been introduced:

- Water abstraction allocations (“limits”) for irrigation and other major uses are approved by the Ministry of Water Resources and Land Reclamation (Melioratsi);

- River flows and river water quality are monitored by the Hydrometerological Service;

- Quality of effluent discharges is monitored by the State Committee for Environmental Protection and Forestry, which also issues licences for effluent discharges, usually with a medium term (about 3 years) or long term (about 25 years) period of validity;

- The Ministry of Health is responsible for monitoring waters that might impact health, especially drinking water, and recommends that measures are taken when water quality falls, usually during the summer;

- Groundwater abstractions require the permissions of the ministries of geology, Health, and environment.

In addition, the hydropower stations situated at reservoirs (Nurek and Kaiyrakum) are major influences on the regulation of rivers, though other off-stream power stations are of lower capacity and usually have no influence on river management as river flows and river water quality are always adequate.

The problems associated with integration of water management relate mainly to the exchange of information between the above agencies, although there is not always a need for close cooperation because of the large flows in the rivers in Tadjikistan. Most coordination is needed with downstream riparian countries. Issues that do arise are in regard to the need to simultaneously monitor flow and quality and to analyse the resulting effluent loads, though this exercise is currently not carried out routinely. Mineralisation is an issue on the Syrdarya river, though that also is a transboundary issue.

## Objective 8: Develop the Inter-state Cooperative Structures for River/lake Basin Management.

Action 8.1  Provide the foundation for interstate cooperation through:

        - Improving information management and sharing;

        - Establishing a joint public information and participation policy;

        - Joint analysis of water management issues to develop basin plans.

Action 8.2  Establish joint management bodies through:

        - Inter-state agreements; and

        - Joint water management bodies (e.g. Commission).

- Tadjikistan is a mountainous country and includes the upper catchments of the Amudarya River. In addition a short section of the middle reach of the Syrdarya River passes through northern Tadjikistan. Transboundary water management is thus mainly effected through the mechanism of the International Fund for the Aral Sea (IFAS), and the Interstate Commission for Water Coordination (ICWC). ICWC comprises meetings of the 5 water ministers of the Central Asian states and its subordinate bodies include the River Management Organisations (BVO), the Secretariat and the Scientific Information Service. River management plans are developed twice a year by these organisations and are monitored and managed by the BVO Syrdarya (Tashkent), BVO Amudarya (Urgench), and its sub-office (Vaksh).

- The main influence on the Amudarya River are the operation of Nurek Reservoir on the Vaksh River and another eservoir on the Kafirnigan River. The main influence on the Syrdarya River is the operation of Kaiyrakum Reservoir.

- Reservoirs are all operated by the electricity supply company under the direction of the Ministry of Energy.

- Apart from the Amudarya and Syrdarya, the only other major basin within Tadjikistan is the Zerafshan, which rises in Tadjikistan and flows into Uzbekistan. The river used to be a tributary of the Amudarya but the waters are now wholly diverted. There is currently little water diversion within Tadjikistan and there is no agreement on sharing of the river with Uzbekistan.

- Mineralisation issues on the Amudarya and Syrdarya can only be dealt with through improved land management.

# National Policies Regarding Further Changes of the Water Sector

## Water Supply and Sanitation

Support to the RWSE is identified under the government’s development policy for 2003 – 2015. However, no detailed plans have been developed yet.

The SUE has prepared a proposal for complete reform of the municipal services sector. This report is presently under consideration by government, so the resulting decision will establish the policy of the government towards reform of the water supply and sanitation services sector.

There is currently no policy to change the arrangements under which the Vodokanals serving the 8 largest cities operate.

Water is one of the highest priorities of Tadjikistan.

- The International Fresh Water Forum was held in Dushanbe, on 30 August to 1 September, 2003. At this Forum participants send a request to the General Secretary of the UN requesting recognition that the decade 2005 – 2015 should be assigned as the Action Decade "Water for Life".
b)An International Conference on regional cooperation in transboundary river basins was held in Dushanbe on May 30 to 01 June 2005..

## Integrated Water Resources Management

There are currently no policies to change the arrangements for water management.

The government is committed to continuing cooperation in management of the Amudarya through the international organisations of IFAS, ICWC and BVO Amudarya. Representatives of Afghanistan were invited to the 2005 Water Conference and Tadjikistan expects that negotiations will be entered into with Afghanistan as to their involvement in joint management of the Amudarya River.

# Short-Term Priorities

## Improvement of Urban Water Supply

In Dushanbe the problem of the turbid water after rains is most pressing. A mini chlorine plant is urgently needed and plans to obtain all potable water from groundwater should be implemented as soon as possible.

Restructuring of the provision of water supply and sewerage is urgently needed, to put the utilities on a commercial basis. Following a government decision in principle, assistance is required in formulating the detailed policies and implementing reforms, including institutional strengthening and revision of legislation.

## Reduction of Contamination by Municipal Wastewater

Wastewater treatment will remain a low priority for a few years, until the potable water supply has been improved.

## Improvement of Rural Water Supply and Sanitation

A major review of rural water supply is needed to identify the strategies for continued operation of the schemes managed by the Rural Water Supply Enterprise. The review should cover both institutional and physical aspects.

The institutional review should identify which parts of the schemes or which responsibilities could be transferred to management by local water users associations. Fee collection and maintenance of the distribution system might be suitable for transfer so that the RWSE becomes a bulk water supplier.

The physical review should include engineering and economic analysis to identify how best to rehabilitate or change each scheme. Options to be considered should include identification of new sources, break-up of large schemes into several smaller schemes, improved and more efficient pumping, replacement of trunk mains with smaller diameters when they are rehabilitated.

## National Coordination of River Management

There is a need to better improve coordination of water management, especially through development of information systems, formal instructions for information sharing and for dealing with emergencies.

## Improved Cooperation on Transboundary River Management

The sharing of information on the Amudarya River needs to be improved, as does the use of information for forecasting, updating forecasts and liasing with BVO Amudarya.

There is a need to bring Afghanistan into the management of the Amudarya River.

# Building Blocks

## Blocks Already Submitted

Development of water resources monitoring in discharge formation zone

Monitoring of water resources within discharge formation zones based on the data provided by hydrometeorological services (HMS) of Tajikistan, Kyrgyzstan and Uzbekistan and using space images.

The following objectives should be achieved:

Reinstatement of old hydrological stations and establishment of new ones; 
ordering of the monitoring, recording and observation network; provision of advanced communications means for the hydromonitoring service; training of specialists in monitoring; identification of watercourse sections prone to washing out with mud slides in order to protect drinking water intake stations and sanitary wastewater treatment facilities.

Economic aspects include development of the region through sound and efficient usage of water resources, as well as by introduction of chargeable water use for irrigation.

Dushanbe, Hodjent, Chkalovsk, Rogun, Kayrokum, and Nurek

It was reported that at the Obikek Valley scheme, 200 m of the 54 km main have recently been replaced.