# EUWI-EECCA Component

- National Policy Dialogue in Moldova related to Integrated Water Resources Management

- Policy package:

- Governmental Regulation on wastewater discharges from municipal sources

- Prepared and approved by the high-level Steering Group on the National Policy Dialogue in October 2007

- Revised in March 2008 and June 2008

- Adopted on 24 September 2008

- (Unofficial translation into English of the version of 24 September 2008 and of the official version – in Russian - as adopted by the Government on 10 October 2008)

- NPD/MD/SG/2008/2

- English

- Original: Russian

The Government of the Republic of Moldova

Resolution №________

from____________________2008

On approval of Regulation on terms for urban waste water discharge into natural reservoirs

For the purpose of harmonization of the national quality standards for waste water discharge into natural reservoirs with the requirements of EU Council Directive of 21 May 1991
concerning urban waste water treatment 91/271/СЕЕ, the Government DECIDES:

1. To approve Regulation on terms for urban waste water discharge into natural reservoirs (enclosed).

2. The central public administration bodies and other interested bodies should revise in 3 months the branch and departmental standards and to harmonize them with the requirements of the above-mentioned Regulation.

3. The Ministry of Environment and Natural Resources alongside with the Ministry of Construction and Territory Development and the Ministry of Local Public Administration should develop within a year the Action Plan on urban waste water collection, treatment and discharge into natural reservoirs.

Prime Minister							Zinaida GRECHANNAYA

Countersign:

Minister of Environment and Natural Resources			Violeta IVANOV

Minister of Local Public Administration				Valentin GUZNAC

Minister of Construction and Territory Development                      Vladimir BALDOVICH

Regulation on terms for urban waste water discharge into natural reservoirs

I. Field of application

1. The purpose of the present Regulation is protection of environment from pollution caused by the urban waste water discharge; establishment of the basic conditions on urban waste water collection, treatment and discharge into natural reservoirs; as well as determination of permissible pollution criteria for basic waste water quality indicators.

2. Terms of waste water discharge are presented in Table 1 of Regulation

3. The present Regulation is applied only for treated or untreated urban waste water and discharge from waste water treatment plants. Requirements of the present Regulation are applied in:

a) designing, harmonization; and in some cases, in issuing the water use permits, in case of sludge disposal from urban waste water treatment plants, and during expansion or development of waste water treatment facilities.

b) determination of preliminary treatment level for industrial waste water entering collecting systems and disposal of sludge from urban waste water treatment plants.

II. Concepts and definitions

4. For the purposes of the present Regulation the following concepts are used:

a) industrial waste water - any waste water which is discharged from premises used for carrying on any trade or industry, other than domestic waste water and run-off rain water;

b) domestic waste water - waste water from residential settlements and services which originates predominantly from the human metabolism and from household activities;

c) urban waste water - domestic waste water or the mixture of domestic waste water with industrial waste water and/or run-off rain water;

d) water use permit – technical legal act that stipulates conditions of putting into operation or maintenance of the new or existing facilities built on water or those for disposal of treated/untreated waste water;

e) population equivalent (1 p.e.) – the organic biodegradable load having a 5-day biochemical oxygen demand (BOD5) of 60 g of oxygen per day;

f) appropriate treatment – treatment of urban waste water by any process and/or disposal system which after discharge allows the receiving waters to meet the relevant quality objectives and the relevant provisions of this and other regulations and current decisions as well as water permits;

g) primary treatment - treatment of urban waste water by a physical and/or chemical process involving settlement of suspended solids, or other processes in which the BOD5 of the incoming waste water is reduced by at least 20 % before discharge and the total suspended solids of the incoming waste water are reduced by at least 50 %;

h) secondary treatment - treatment of urban waste water by a process generally involving biological treatment with a secondary settlement or other process in which the requirements established by present Regulation;

i) eutrophication - the enrichment of water by nutrients, especially compounds of nitrogen and/or phosphorus, causing an accelerated growth of algae and higher forms of plant life to produce an undesirable disturbance to the balance of organisms present in the water and to the quality of the water concerned;

j) sludge - residual sludge, whether treated or untreated, from urban waste water treatment plants;

k) check point – the point of water samples taking to carry out the laboratory analysis, this place could be:

a) before delivering urban and industrial waste water to the waste water treatment plants or

b) after waste water treatment and final discharge into surface water;

l) natural reservoirs – water source that accumulates waste water directly or after waste water treatment;

m) collecting system - a system of conduits which collects and conducts urban and/or industrial waste water;

n) modernized waste water treatment plants - waste water treatment plants that allow to reach the quality determined by the agreements and water permits being modernized and developed;

o) sensitive zones – surface water sites including those affected by eutrophication or having the additional quality requirements stipulated by the current legislation.

III. Urban waste water treatment and discharge

5. According to requirements of the present Regulation and current legislation, the urban and industrial waste water entering the urban collecting system should be relevantly treated.

6. The urban waste water treatment plants constructed in accordance with requirements of the present Regulation should be designed, assembled, operated and maintained in effective way of functioning for any climate conditions specific for its location. It is necessary to consider seasonal fluctuations of their loading already at the initial stage of the construction.

7. Discharges from urban waste water treatment plants should correspond to conditions from Table 1.

8. Requirements for discharges from urban waste water treatment plants to sensitive areas should correspond to those identified in Table 2.

Table 1. Requirements for discharges from urban waste water treatment plants

The values for concentration or for the percentage of reduction shall apply.

| Parameters | Concentration(2) | Minimum percentage of reduction (1) | Reference method of measurement |
| --- | --- | --- | --- |
| Biochemical oxygen demand (BOD5) | 25 mg/l О2 | 70-90 | Homogenized, unfiltered, undecanted sample. Determination of dissolved oxygen before and after five-day incubation at 20 °C ± 1 °C, in complete darkness. |
| Chemical oxygen demand (COD) | 125 mg/l О2 | 75 | Homogenized, unfiltered, undecanted sample Potassium dichromate |
| Total suspended solids | 35 mg/l | 90 | - Filtering of a representative sample through a 0,45 µm filter membrane. Drying at 105 °C and weighing, Centrifuging of a representative sample (for at least 5 min with mean acceleration of 2 800 to 3 200 g), drying at 105 °C and weighing |
| (1) Reduction in relation to the load of the influent. (2) The values of concentration are established by single tests |  |  |  |

Analyses concerning discharges from lagooning shall be carried out on filtered samples; however, the concentration of total suspended solids in unfiltered water samples shall not exceed 150 mg/l.

Table 2. Requirements for discharges from urban waste water treatment plants

to sensitive areas

The values for concentration or for the percentage of reduction shall apply.

| Quality parameters | Concentration(3), mg/l | Minimum percentage of reduction (1) | Reference method of measurement |
| --- | --- | --- | --- |
| Total phosphorus | 2 mg/l P (10,000 – 100,000 p. e.) 1 mg/l P (more than 100,000 p. e.) | 80 | Molecular absorption spectrophotometry |
| Total nitrogen (2) | 15 mg/l N (10,000 – 100,000 p. e.) 10 mg/l N (more than 100,000 p. e.) | 70-80 | Molecular absorption spectrophotometry |

(1) Reduction in relation to the load of the influent. 
(2) Total nitrogen means: the sum of total Kjeldahl-nitrogen (organic N + NH3), nitrate (NO3)-nitrogen and nitrite (NO2)-nitrogen. 
(3) The values of concentration are established by single tests, the average values are not allowed

9. According to the present Regulation, the water objects used as a source of drinking water supply having concentration of nitrogen above the established quality standards for surface water used as potable water, are considered as sensitive zones.

10. Sanitation-and-epidemiological requirements to treated waste water discharge into natural reservoirs are in the competence of the Ministry of Health.

11. The points of urban waste water discharge shall be chosen to minimize their adverse effects on receiving waters.

12. Treated waste water discharges should be re-used whenever appropriate, depending of their origin and field of use, with the authorization of eligible branch bodies. Re-used waste water should minimize the adverse effects on the environment.

13. Sludge arising from waste water treatment should be treated and stored properly or be re-used if necessary. Disposal routes should minimize the adverse effects on the environment and be specified in the projects of the new waste water treatment plants or in the water authorizations.

14. Sludge, depending of its origin and field of use, can be re-used only with the authorization of eligible bodies.

15. Industrial and domestic waste water discharges to the urban collecting systems should correspond to specifications and requirements of authorization on discharge collecting system serving the agglomeration.

IV. Limitations on urban waste water discharge

16. Waste water discharge into natural reservoirs does not contain:

а) polluting substances with increased toxic level, specified in p. 17, and substances forbidden according to the results of special research;

b) exceeding value of suspended solids that provokes increase in amount of sediments in small water streams

c) substances that increase turbidity, foaming or change the natural organoleptical characteristics of received waters.

17. Classes and groups of the substances selected on the basis of toxicity, stability and bioaccumulation include:

- organohalogen compounds;

- organic tin compounds and organophosphate compounds;

- cancerogenic substances;

- organic mercury compounds;

- organosilicate compounds;

- radioactive wastes concentrated in environment and water organisms

18. Discharge into natural reservoirs of waste water having substances of high risk from classes and groups mentioned in p. 17 is forbidden.

19. Discharges of waste water belonging to medical preventive and treatment institutions (infectious diseases hospitals, health resorts for tuberculosis patients, institutions for biological samples and vaccines preparations including warm houses), and also from livestock units and slaughtering plants could not be allocated in reservoirs without carrying out special preliminary disinfection.

20. Discharge of the treated waste water into drainage, irrigational canals or agricultural lands is carried out only by permission of nature protection and public health bodies.

V. Monitoring of waste water discharge from urban waste water treatment plants into natural reservoirs

21. Urban waste water discharges before its disposal into natural reservoirs should pass control monitoring procedures stipulated by the project documentation for waste water treatment plants and by the present Regulation.

22. Monitoring of collecting systems and/or urban waste water treatment plants as well as any discharge into natural reservoirs is a duty of all suppliers/operators of public services, bodies of the state control and supervisors in this area.

23. Waste water treatment plants should be designed or modified the way that allows to collect in special check points the representative samples of waste water discharges coming to treatment plants, outgoing after treatment or being taken from final waste water discharge into receiver.

24. Used methods of monitoring constitute the operating standard methods of the national level.

25. The samples are selected at check points within 24 hours or per certain intervals of time, depending on waste water treatment plant loading at the exit, and, if necessary, at the entrance, in order to supervise compliance of the instruction stipulated by present technical standards. For selection of samples are used the national and, if necessary, the international laboratory methods: ISO or European regulations (ER) to minimize the samples degradation from the moment of selection up to the analysis.

26. The treated waste water shall be assumed to conform the relevant parameters if, for each relevant parameter considered individually, samples of the water show that it complies with the relevant parametric value in the following way:

(a) for the parameters specified in Table 1, a maximum number of samples which are allowed to fail the requirements, expressed in concentrations and/or percentage reductions in Table 1, is specified in Table 3;

(b) for the parameters of Table 1 expressed in concentrations, the failing samples taken under normal operating conditions must not deviate from the parametric values by more than 100 %. For the parametric values in concentration relating to total suspended solids deviations of up to 150 % may be accepted;

(c) for those parameters specified in Table 2 the annual mean of the samples for each parameter shall conform to the relevant parametric values.

27. Extreme values for the water quality in question shall not be taken into consideration when they are the result of unusual situations like heavy rain.

Table 3 Permitted number of samples which can fail to comply with requirements

| Series of samples taken in any year | Maximum permitted number of samples which can fail to comply with requirements |
| --- | --- |
| 4 - 7 | 1 |
| 8 - 16 | 2 |
| 17 – 28 | 3 |
| 29 – 40 | 4 |
| 41 – 53 | 5 |
| 54 – 67 | 6 |
| 68 - 81 | 7 |
| 82 – 95 | 8 |
| 96 – 110 | 9 |
| 111 – 125 | 10 |
| 126 – 140 | 11 |
| 141 – 155 | 12 |
| 156 – 171 | 13 |
| 172 – 187 | 14 |
| 188 – 203 | 15 |
| 204 - 219 | 16 |
| 220 – 235 | 17 |
| 236 – 251 | 18 |
| 252 – 268 | 19 |
| 269 – 284 | 20 |
| 285 - 300 | 21 |
| 301 – 317 | 22 |
| 318 – 334 | 23 |
| 335 – 350 | 24 |
| 351 – 365 | 25 |

VI. Methods to establish maximum permissible pollutions loads for waste water discharge into natural reservoirs

28. Maximum permissible pollutions loads for waste water discharge into natural reservoirs are stipulated by the Table 1 of present Regulation and represents values of concentration in mg/l. The maximum permissible values of concentration are established for single tests only, the average values are not allowed, they are measured at the check point before disposal.

29. Maximum permissible values, mentioned in p. 28, are establishing according to requirements of present Regulation and should be introduced in:

1) the ecological expertise of projects for:

a) new objects;

b) operating objects that are modifying and improving technological processes of waste

water treatment;

c) operating objects that are planned to develop its capacity or waste water treatment facilities;

d) other operating objects which owing to investment activities change values of final parameters.

2). authorizations for water use, given out:

a) to new users in cases when in water use permits the conditions similar to conditions of the present Regulation;

b) to existing water-users only after performance and delivery in operation the corresponding capacities of waste water treatment.

30. On the basis of environmental projects findings, the permissible values of pollution load could be established below those mentioned in the Table 1, considering the existing permissible pollution load for reservoir above the point of discharge, and characteristic of a natural reservoir itself (after a procedure of establishing the category of its quality).

31. For substances that have no maximum permissible pollutions loads in operating standards or specifications, they are established on the basis of researches lead by specialized institutions, authorized by the Governmental decision. Research should contain methods to analyze the quality and quantity of corresponding substances as well as technologies of adequate treatment. Maximum permissible values are approved by the Government in a proper procedure.

32. In case waste water contains polluting substances more than it was established as maximum permissible values in present Regulation, additional treatment or acceptance of adequate technological measures to achieve permitted values is obligatory.

33. If waste water treatment plant is out of order due to exceptional natural accident, the environment and public health institutions can make deviation from the present Regulation for discharge values.

34. In some cases, after technological tests, initiation of biological methods of treatment, periodical inspections, or during development or modernization of the treatment plant capacities, it is assumed the excess of maximum permissible quality values, if it does affect health of population, water eco-systems or does not bring any material damage, under agreement with environment, public health and fish inspection bodies.

The water-user addresses 30 days prior planned date of inspection, repair, technological tests or start-up of the biological waste water treatment plant for coordination. The period when excess is authorized is established by proper permit, it should be no longer than 30 days, the possible admissible values for this period are also established by permit.

35. The existing users of waste water treatment facilities should reach the requirements on discharge standards for values exceeding the limit, specified in Tables 1 and 2 of present Regulations, during 3 years.

VII. Final provisions

36. Operators of public services or, in the certain cases, the owners of waste water treatment plants or systems of discharge into natural reservoirs are obliged to provide installation and proper functioning of waste water discharge measurement and control facilities; to sustain water checking operations in control points and, if possible, to install the automatic system to monitor the quality of waste water discharge considering the values specific for its activity. If yield of a receiver is 3 times bigger than volume of waste water discharge, in case this volume exceeds 500 l/day, it is necessary to install the disperse/diffusive system.

37. To protect water resources from pollution:

a) it is necessary to use wastewater and/or sludge containing biogenic substances as fertilizer or for irrigation on agricultural or wood lands with the permission the owners of the lands and authorization of the competent bodies from the Ministry of Agriculture and Food Processing and the Ministry of Health.

b) it is necessary to provide water resistance for all sludge storage facilities. Possible filtrates and storm flow discharges from storage facilities should be accumulated and treated in order to correspond the requirements of present Regulation.

38. The methods of analysis used to determine quality and quantity of polluting substances from Table 1 are stipulated by operating standards.

39. A sampling point of waste water discharge is the final point of discharge into receiver as stipulated by the requirements of present Regulation.

.

40. Frequency of checks, as well as minimum quantity of the tests selected during the certain period of time, is established by the permit of water use and depends on capacity of treatment facilities and a degree of influence of waste water discharge on a natural reservoir.

| HGO1141/2008 Внутренний номер: 329400 Varianta în limba de stat | Карточка документа |
| --- | --- |
| Республика Молдова |  |
| ПРАВИТЕЛЬСТВО |  |
| ПОСТАНОВЛЕНИЕ Nr. 1141 от 10.10.2008 |  |
| об утверждении Положения об условиях сброса городских сточных вод в естественные водоемы |  |
| Опубликован : 21.10.2008 в Monitorul Oficial Nr. 189 статья № : 1163 |  |
| В целях приведения национальных норм по качеству сточных вод при сбросе в естественные водоемы в соответствие с требованиями Директивы 91/271/СЕЕ относительно очистки городских сточных вод Правительство ПОСТАНОВЛЯЕТ: 1. Утвердить Положение об условиях сброса городских сточных вод в естественные водоемы (прилагается). 2. Органам центрального публичного управления и другим заинтересованным органам пересмотреть в 3-месячный срок отраслевые и ведомственные нормативы и привести их в соответствие с требованиями вышеуказанного положения. 3. Министерству экологии и природных ресурсов совместно с Министерством строительства и развития территории, Министерством местного публичного управления разработать в течение года план мероприятий по сбору, очистке и сбросу городских сточных вод в естественные водоемы. ПРЕМЬЕР-МИНИСТР Зинаида ГРЕЧАНЫЙ Контрассигнуют: первый зам. премьер-министра, министр экономики и торговли Игорь ДОДОН министр экологии и природных ресурсов Виолета ИВАНОВ министр местного публичного управления Валентин ГУЗНАК министр строительства и развития территории Владимир БАЛДОВИЧ № 1141. Кишинэу, 10 октября 2008 г. Утверждено Постановлением Правительства № 1141 от 10 октября 2008 г. ПОЛОЖЕНИЕ об условиях сброса городских сточных вод в естественные водоемы I. Область применения 1. Целью настоящего положения является охрана окружающей среды от загрязнения, вызываемого сбросом сточных вод, установление основных условий очистки и сброса сточных вод в естественные водоемы, а также предельно допустимых значений основных показателей качества этих вод. 2. Условия сброса сточных вод представлены в приложении №1 к настоящему положению. 3. Настоящее положение применяется только к городским сточным водам, которые были очищены, и к сточным водам, сбрасываемым с очистных сооружений. Требования настоящего положения применяются при: a) проектировании, согласовании и в некоторых случаях при выдаче разрешений на новые работы по использованию водных ресурсов, а также при сбросе сточных вод с существующих очистных сооружений, расширении или техническом перевооружении объектов, которые осуществляют сброс очищенных или неочищенных сточных вод; b) определении степени предварительной очистки промышленных сточных вод, которые отводятся в систему канализации и на городские очистные сооружения. II. Понятия и определения 4. Для целей настоящего положения используются следующие понятия: a) промышленные стоки – любые стоки с объектов, в которых осуществляется какая-либо коммерческая или промышленная деятельность, кроме стоков бытового происхождения и дождевых стоков; b) стоки бытового происхождения – стоки, поступающие из хозяйств и служб и состоящие, как правило, из продуктов жизнедеятельности и из бытовых отходов; c) городские стоки – стоки бытового происхождения или смесь бытовых стоков с промышленными и/или дождевыми стоками; d) водохозяйственное разрешение – технико-юридический акт, которым обусловливаются условия сдачи в эксплуатацию или эксплуатация новых или существующих объектов, построенных на воде или использующих воду для сброса очищенных или неочищенных сточных вод; e) эквивалент населения (ЭН) – органический, способный к биологическому разложению объем загрязнителей, имеющих 5-дневный показатель биохимического потребления кислорода (БПК5), равный 60 г кислорода в день; f) соответствующая очистка – очистка стоков с помощью любого процесса и/или системы удаления, в результате которой водотоки, принимающие сбросы, отвечают соответствующим требованиям настоящего положения и действующих заключений и водохозяйственных разрешений; g) первичная очистка – очистка сточных вод с помощью физического и/или химического процесса, который обеспечивает осаждение взвешенных твердых частиц, или других процессов, при которых показатель БПК5 для неочищенных стоков в процессе очистки снижается как минимум на 20%, а общее содержание взвешенных твердых частиц в неочищенных стоках – на 50%; h) вторичная очистка – очистка городских стоков с помощью биологического процесса, сопровождаемого вторичным осаждением, или другого процесса, при котором соблюдаются требования, предусмотренные настоящим положением; i) эвтрофикация – процесс накопления в водоеме питательных веществ, в особенности нитратов и фосфатов, что приводит к быстрому росту водорослей и водных растений и к нежелательным нарушениям баланса водных организмов, а также к изменениям качества воды; j) шлам – очищенный или неочищенный осадок, остающийся после очистки городских стоков очистными сооружениями; k) контрольный пункт – место, где отбираются пробы воды для выполнения лабораторного анализа; этим местом может быть перед подачей на очистные сооружения городских и промышленных сточных вод и после очистки стоков или прямого сброса место окончательного сброса сточных вод в водные объекты; l) естественный водоем – водный источник, который принимает сбрасываемые сточные воды напрямую или после очистки на очистных сооружениях; m) сети канализации – система трубопроводов и/или каналов, которые собирают и отводят городские и/или промышленные сточные воды; n) переоснащенные/модернизированные очистные сооружения - очистные сооружения, которые посредством их технического обновления, улучшения и дополнения, позволяют достичь качественных условий, устанавливаемых документами согласования и водохозяйственными разрешениями; o) чувствительные зоны – участки поверхностных вод и отдельные места поверхностных вод, подверженные эвтрофикации. III. Очистка и сброс городских сточных вод 5. Перед сбросом в природные накопители бытовые и промышленные сточные воды, отведенные в городские сети канализации, подлежат соответствующей очистке согласно требованиям настоящего положения и действующего законодательства. 6. Сооружения по очистке городских сточных вод, построенные согласно настоящему положению, должны проектироваться, строиться, эксплуатироваться и содержаться таким образом, чтобы они могли функционировать эффективно во всех климатических условиях, специфичных месторасположению. Необхо¬димо учитывать сезонные колебания их загрузки уже на начальном этапе строительства этих сооружений. 7. Сбросы с сооружений по очистке городских сточных вод должны соответствовать условиям приложения № 1 к настоящему положению. Анализ сбросов с отстойников должен проводиться на фильтрованных образцах. Вместе с тем концентрация взвешенных твердых веществ в нефильтрованных образцах не должна превышать 150 мг/л. 8. Сбросы с сооружений по очистке городских сточных вод в чувствительные зоны должны осуществляться с соблюдением условий, указанных в приложении № 2 к настоящему положению. 9. В целях настоящего положения водные объекты, используемые в качестве источника питьевого водоснабжения, в которых концентрации азота могут быть выше установленных нормативов качества поверхностных вод, предназначенных для забора питьевой воды, считаются чувствительными зонами. 10. Санитарно-эпидемиологические требования к качеству очищенных сточных вод, сбрасываемых в естественные водоемы, находятся в компетенции Министерства здравоохранения. 11. Пункты сброса городских стоков выбираются с учетом максимального снижения воздействия на водоемы. 12. Очищенные стоки должны по возможности повторно использоваться с разрешения соответствующих отраслевых органов в зависимости от происхождения и области использования. Повторное использование этих вод должно осуществляться при условии минимального негативного воздействия на окружающую среду. 13. Шлам, остающийся после процесса очистки стоков, следует обрабатывать и складировать соответствующим образом или использовать повторно, если это необходимо. Способы его размещения и использования должны свести к минимуму отрицательное воздействие на окружающую среду и должны быть определены в проектах новых очистных сооружений или в водохозяйственных разрешениях. 14. Использование шлама возможно только при наличии заключения соответствующих органов в зависимости от происхождения и области использования. 15. Промышленные и бытовые сточные воды, сбрасываемые в городские канализационные сети, должны соответствовать техническим условиям и требованиям разрешений на сброс сточных вод в систему канализации населенных пунктов. IV. Ограничения по сбросу городских сточных вод 16. Сточные воды, сбрасываемые в естественные водоемы, не должны содержать: а) загрязняющие вещества с повышенной степенью токсичности, указанные в пункте 17, а также вещества, запрет на которые был установлен специальным исследованием; b) взвешенные вещества с превышением значений, которые могут привести к отложениям в малых водотоках; c) вещества, которые могут привести к повышению мутности, образованию пены или изменению органолептических свойств водоприемников по сравнению с их естественным состоянием. 17. К классам и группам веществ, отобранных на основе токсичности, стойкости и биоаккумуляции, относятся: a) органогалогенные соединения; b) органические соединения олова и органофосфоритные соединения; c) вещества с канцерогенными свойствами; d) органические соединения ртути; e) органосиликатные соединения; f) радиоактивные отходы, которые концентрируются в окружающей среде и водных организмах. 18. Запрещается сброс в естественные водоемы вместе со сточными водами веществ, которые принадлежат к классам и группам, указанным в пункте 17, и которые имеют высокую степень опасности. 19. Сточные воды, сбрасываемые медицинскими лечебными и профилактическими учреждениями (инфекционные больницы, санатории для больных туберкулезом, учреждения по подготовке биологических препаратов - теплицы и вакцины), животноводческими комплексами и скотобойнями не могут быть отведены в водоемы без проведения специальной предварительной дезинфекции. 20. Отведение очищенных сточных вод в осушительные, ирригационные каналы или на сельскохозяйственные земли осуществляется только с разрешения природоохранных органов и органов здравоохранения. V. Мониторинг сброса сточных вод с городских очистных сооружений в естественные водоемы 21. Городские сточные воды перед сбросом в естественные водоемы должны пройти мониторинг в соответствии с процедурами контроля, установленными в проектной документации очистных сооружений и в настоящем положении. 22. Мониторинг сетей канализации и/или городских очистных сооружений и любых других сбросов в естественные водоемы является обязанностью всех поставщиков/операторов публичных услуг, органов государственного контроля и надзора в этой области. 23. Очистные сооружения должны проектироваться или модифицироваться таким образом, чтобы из установленных контрольных пунктов можно было отобрать репрезентативные пробы из стоков на входе очистных сооружений и из очищенного стока или из окончательного стока, перед сбросом в приемник. 24. Используемые методы мониторинга являются действующими стандартными методами национального уровня. 25. В контрольных точках пробы отбираются в течение 24 часов или в определенные интервалы времени в зависимости от загрузки очистных сооружений на выходе, при необходимости, и на входе очистных сооружений - для наблюдения соответствия предписаниям, установленным настоящим положением. При отборе проб применяются национальные, и по необходимости, международные лабораторные методы: соответственно методы ISO или европейские нормы (ЕН) с тем, чтобы свести к минимуму деградацию образцов за период времени с отбора до анализа. 26. Очищенные стоки считаются отвечающими предельно допустимым значениям, если для каждого отдельно взятого параметра образцы стоков покажут, что они соответствуют конкретному параметру следующим образом: а) для параметров (показателей) из приложения № 1 – максимальное количество образцов, которые могут не соответствовать нормам концентрации и/или снижению процентного содержания, указанного в приложении № 3; b) для параметров, указанных в приложении № 1, выраженных значениями концентрации, - максимальное количество образцов, отобранных при нормальных условиях эксплуатации, не должно отклоняться от указанных значений более чем на 100%. Для значений концентрации, относящихся к общему содержанию взвешенных твердых частиц, могут приниматься отклонения до 150%; c) для параметров из приложения № 2 – среднегодовые показатели образцов по каждому параметру не должны превышать предельных значений, установленных для конкретного параметра. 27. Экстремальное значение параметров, относящихся к качеству воды, не принимаются во внимание, если они получены в результате аномальных ситуаций, например, обильного выпадения осадков. VI. Способы установления предельно допустимых значений загрязнителей в сточных водах, сбрасываемых в естественные водоемы 28. Максимально допустимая нагрузка в отношении загрязнителей в сточных водах, сбрасываемых в естественные водоемы, предусмотрена в приложении № 1 к настоящему положению и представляет концентрации, выраженные в мг/л. Значение этих предельных концентраций установлено для текущих единовременных проб; не допускаются средние концентрации, и они замеряются в контрольной точке до сброса. 29. Допустимые значения, указанные в пункте 28, устанавливаются в соответствии с требованиями настоящего положения и вносятся в: 1) экологические заключения проектов, которые выдаются для: a) новых объектов; b) действующих объектов, которые модифицируют и улучшают технологические процессы производства или очистки сточных вод; c) действующих объектов, на которых предусматривается расширение производственных мощностей или мощностей по очистке сточных вод; d) других действующих объектов, которые вследствие инвестиционных действий меняют значения конечных параметров; 2) разрешения на водопользование, выданные: a) новым пользователям в случаях, когда в водохозяйственном разрешении были предусмотрены условия, аналогичные условиям настоящего положения; b) существующим водопользователям только после выполнения и сдачи в эксплуатацию соответствующих мощностей по очистке сточных вод. 30. На основании экологических заключений проектов и водохозяйственных разрешений можно установить предельно допустимые значения ниже, чем они предусмотрены в приложении № 1 к настоящему положению, исходя из существующей нагрузки загрязнителей на водоеме выше точки сброса сточных вод, и принимая во внимание характеристику естественного водоема (после процедуры установления категории его качества). 31. Для веществ, для которых в действующих стандартах или нормативах не предусмотрены максимально допустимые значения, они устанавливаются на основании исследований, проведенных специализированными институтами, уполномоченными поста¬новлением Правительства. Исследование должно содержать также методы анализа качества и количества соответствующих веществ, а также технологии адекватной очистки. Предельно допустимые значения утверждаются Правительством в установленном порядке. 32. В случае, если сточные воды содержат загрязняющие вещества сверх предельно допустимых значений, установленных настоящим положением, обязательна дополнительная очистка или принятие адекватных технологических мер для достижения разрешенных значений. 33. В случае сброса сточных вод с очистных сооружений, которые в результате природной катастрофы полностью или частично вышли из строя, природоохранные органы и органы здравоохранения могут сделать отступление от настоящего положения. 34. В отдельных случаях – после технологических испытаний, при запуске биологических ступеней очистных сооружений, периодических обследований или во время выполнения работ по техническому перевооружению или расширению мощностей очистных сооружений с согласия природоохранных органов и органов здравоохранения, рыбной инспекции допускается превышение предельных значений показателей качества, если это не представляет опасности для здоровья населения, водных экосистем или не ведет к материальному ущербу. Водопользователь обращается за согласованием за 30 дней до запланированной даты начала обследования, ремонтных работ, технологических испытаний или запуска станции биологической очистки. Соответствующим разрешением устанавливается период, в течение которого разрешается превышение, но не более чем на 30 дней, а также устанавливаются максимально допустимые значения для показателей в этот период. 35. Существующие пользователи, эксплуатирующие очистные мощности, должны в течение 3 лет достичь требований настоящего положения относительно норматива по сбросу по тем значениям, по которым существует превышение предельных значений, указанных в приложениях № 1 и № 2 к настоящему положению. VII. Заключительные положения 36. Операторы публичных услуг или в определенных случаях владельцы очистных сооружений или систем отвода сточных вод в естественные водоемы обязаны обеспечить монтаж и соответствующее функционирование средств измерения расхода сбрасываемых сточных вод с регистрацией и контролем расходов; должны способствовать отбору проб воды для анализа в установленных местах и, по возможности, установить автоматическую систему контроля качества сбрасываемых сточных вод с измерением специфических для проводимой деятельности параметров. При сбросе сточных вод объемом более 500 л/сутки в приемник, дебит которого превышает не менее чем в 3 раза дебит сточных вод, в точке сброса предусматривается дисперсная/диффузионная система. 37. В целях охраны водных ресурсов от загрязнений: a) следует использовать сточные воды и/или шлам, которые содержат биогенные вещества для удобрения или орошения сельскохозяйственных или лесных земель, с согласия владельцев соответствующих земель и по согласованию с компетентными органами Министерства сельского хозяйства и пищевой промышленности и Министерства здравоохранения; b) необходимо обеспечить водонепроницаемость всех полигонов для хранения шлама. Возможные фильтраты, а также ливневые стоки, стекающие с этих полигонов, необходимо накапливать и очищать таким образом, чтобы они соответствовали требованиям настоящего положения. 38. Методы анализа, используемые для определения качества и количества загрязняющих веществ, включенных в приложение № 1, предусмотрены действующими стандартами. 39. Точкой отбора проб сточных вод, с точки зрения контроля соответствия требованиям настоящего положения, является точка окончательного сброса сточных вод в приемник. 40. Частота проверок и соответственно минимальное количество проб, отобранных в определенный период времени, устанавливается в разрешении на водопользование в зависимости от мощности очистных сооружений и степени воздействия сбросов на естественный водоем. anexa 1-3 |  |